# 3GPP TSG-RAN WG4 Meeting #116	R4-2510082

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.14.3.2

Source:	China Telecom

Title:	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

In RAN4 #115 meeting, FR2-1 SSB based L3 measurement delay reduction for connected mode was discussed and the WF was approved [1].

In this paper, we present our views on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1]:

Issue 1-1-3: Expand solution 1 to FR1 case [Wayforward]: Further discuss in next meeting: If extend for FR1: Aspect 1: co-located or non-collocated - Option 1: only limit to serving cell measurement (E///) - Option 2: cover both serving cell and neighboring cell measurements - In clause 7.6.4 of TS 38.133, it clearly sys only co-located deployment is applied for intra-band contiguous CA. - Companies are encouraged to check the spec in this meeting. Aspect 2: Network configuration on enable/disable the feature - Option 1: the configuration is the same for different FRs - Option 2: the configuration can be different for different FRs (Nokia, ZTE, China Telecom, Apple, HW, E///) - Under the condition that the UE capability is per FR [or per band]. - Option 2 will be adopted if the solution 1 will be extended to FR1.

In our views, there also exists the demand on L3 measurement delay reduction for FR1 band and solution 1 is also beneficial for FR1. It’s proposed to expand solution 1 to FR1 intra-band contiguous CA co-located scenario. Besides, considering the difference between FR1 and FR2, network configuration on enable/disable the feature can be different for different FRs.

Proposal 1: Expand solution 1 to FR1 intra-band contiguous CA co-located scenario.

Proposal 2: Network configuration on enable/disable the feature can be different for different FRs.

Issues related UE capability for CSSF enhancement

Status in the WF [1]:

Issue 1-3-1: UE capability for CSSF enhancement option 1 (measure one carrier per band) Agreement in AdHoc: - For CSSF enhancement solution 1 in Rel-19, a new optional per-UE capability, apply for FR2-1 only. Note: the UE capability can be revisited if this feature is expanded to FR1 case.

If CSSF enhancement solution 1 is expanded to FR1 case, the UE capability can be differentiated between different FRs, in our views, UE capability can be per FR or per band.

Proposal 3: If CSSF enhancement solution 1 is expanded to FR1 case, UE capability can be per FR or per band.

- Conclusion

The contribution presents our views on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC, with the following proposals:

Proposal 1: Expand solution 1 to FR1 intra-band contiguous CA co-located scenario.

Proposal 2: Network configuration on enable/disable the feature can be different for different FRs.

Proposal 3: If CSSF enhancement solution 1 is expanded to FR1 case, UE capability can be per FR or per band.

- References

- R4-2508264, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #115, 19 – 23 May, 2025.

- R4-2505561, Topic summary for [115][212] NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #115, 19 – 23 May, 2025.