# 3GPP TSG-RAN WG4 Meeting #116	R4-2510091

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.23.3.2

Source:	China Telecom

Title:	Discussion on adaptation of common signal channel transmission for enhancements of NES for NR

Document for:	Discussion

- Introduction

In RAN4 #115 meeting, adaptation of common signal/channel transmission for enhancements of network energy savings for NR were discussed and the WF was approved [1].

In this paper, we present our views on adaptation of common signal/channel transmission for enhancements of network energy savings for NR captured in the WF [1] and summary [2].

- Discussion

SSB adaptation – General

Status in the summary [2]:

Issue 1-2-4: L1 requirements – Measurement restriction impact - Proposals - Option 1: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB. (Huawei) - Option 2: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply. (CTC, Vivo) - Recommended WF: - Discuss above option.

In our understanding, it can be guaranteed that no collision exists between CSI-RS and SSB before SSB adaptation, however, after SSB adaptation, the adapted SSB may collide with CSI-RS, therefore, whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS needs to be discussed. In our views, in order to reduce the impact of legacy CSI-RS measurement, existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS, i.e., to measure one of adapted SSB and CSI-RS.

Proposal 1: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply.

SSB adaptation – L3 Requirements impacts

Status in the WF [1] and summary [2]:

Issue 1-3-4: Colliding with MG - Proposals - Option 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell. (Nokia) - Recommended WF: - Agree on option 1.

If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, it’s proposed to skip measurements in the overlapping SMTC occasion of the serving SCell.

Proposal 2: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

- Conclusion

The contribution presents our views on adaptation of common signal/channel transmission for enhancements of network energy savings for NR, with the following proposals:

Proposal 1: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply.

Proposal 2: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

- References

- R4-2508441, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei, RAN WG4 Meeting #115, 19 –23 May, 2025.

- R4-2505571, Topic summary for [115][222] Netw_Energy_NR_enh_Part2, Huawei, RAN WG4 Meeting #115, 19 –23 May, 2025.