# 3GPP TSG-RAN WG4 Meeting #116	R4-2510093

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.24.5.2

Source:	China Telecom

Title:	Discussion on RRM core requirements for LP-WUS/WUR

Document for:	Discussion

- Introduction

In RAN4 #115 meeting, RRM core requirements for LP-WUS/WUR was discussed and the WF were approved [1].

In this paper, we present our views on RRM core requirements for LP-WUS/WUR captured in the WF [1] and summary [2].

- Discussion

General aspects

Status in the summary [2]:

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case) - Proposals - P1: No LR measurement and evaluation requirements apply at the legacy state, i.e., for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3). (vivo xiaomi Apple CT Huawei ZTE) - P1-1: The requirements in clause apply when the LP-WUR is in ON state. Before entering LP-WUS, RRM offloading and RRM relaxation mode and after exiting LP-WUS, RRM offloading and RRM relaxation mode, it is up to LP-WUS UE implementation when and how to turn the LP-WUR to ON state for serving cell measurement. (Nokia) Background: - Summary of entry/exit conditions based on existing RAN2’s agreements Case; Entry conditions; Exit conditions. LP-WUS monitoring; MR threshold and optional LR threshold; LR threshold. RRM measurement fully offloading (Case 1); MR threshold and optional LR threshold; LR threshold. RRM measurement relaxation (Case 3); MR threshold and optional LR threshold. Agreement at RAN4 114bis: - At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation. Recommendations: Agree P1 based on majority view. Discuss whether P1 can be correctly captured in the specification or whether to use P1-1.

For LP-WUR status at legacy case, it was agreed that at legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation. In our views, at legacy state, MR is working without offloading or relaxation, the legacy MR evaluation requirements applied to MR is efficient and there is no need to apply LR evaluation requirements, thus we prefer that no LP-WUR measurement and evaluation requirements apply when LR is “on” at the legacy state.

Proposal 1: No LP-WUR measurement and evaluation requirements apply when LR is “on” at the legacy state.

Status in the summary [2]:

Issue 1-1-11: RRM requirements for FR2 - Proposals - P1: Prioritize defining RRM requirements for FR1 in this WI (Apple Nokia) - P2: There is not any concrete proposal on FR2 requirements so far. Suggest RAN4 to confirm whether FR2 RRM requirement needs be defined in this release, or confirm the status of FR2 requirement will not impact the completion of WI (vivo, Nokia) Recommendations: Check whether P1 or P2 is agreeable.

In our views, suggest to prioritize defining RRM requirements for FR1 in this WI. If there are proposals on FR2 in this meeting, confirm the status of FR2 requirement will not impact the completion of WI.

Proposal 2: Prioritize defining RRM requirements for FR1 in this WI.

Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Status in the summary [2]:

Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity - Proposals - P1: No need to define upper bound for SSB-based LP-WUR measurement periodicity. (oppo xiaomi Nokia CT Huawei Apple) - P2: Prefer the periodicity for measurement and evaluation delay requirements for LP-WUR based on SSB in IDLE/Inactive mode is 1.28s if the LO periodicity is 2.56s. (vivo ZTE) Background: RAN4 113 agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity. Recommendations:

For upper bound on SSB-based LP-WUR measurement periodicity, since DRX cycle and LO periodicity are the same based on RAN1 agreement, and periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity, in our views, it’s preferred not to define upper bound for SSB-based LP-WUR measurement periodicity.

Proposal 3: No need to define upper bound for SSB-based LP-WUR measurement periodicity.

- Conclusion

The contribution presents our views on Rel-19 RRM core requirements for LP-WUS/WUR, with the following proposals:

Proposal 1: No LP-WUR measurement and evaluation requirements apply when LR is “on” at the legacy state.

Proposal 2: Prioritize defining RRM requirements for FR1 in this WI.

Proposal 3: No need to define upper bound for SSB-based LP-WUR measurement periodicity.

- References

- R4-2508280, WF on RRM requirements for NR_LPWUS, vivo, RAN WG4 Meeting #115, 19 – 23 May, 2025.

- R4-2505572, Topic summary for [115][223] NR_LPWUS, vivo, RAN WG4 Meeting #115, 19 – 23 May, 2025.