# 3GPP TSG-RAN WG4 Meeting #116	R4-2510094

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.24.6

Source:	China Telecom

Title:	Discussion on RRM performance requirements for LP-WUS/WUR

Document for:	Discussion

- Introduction

In RAN4 #115 meeting, RRM performance requirements for LP-WUS/WUR were discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements for LP-WUS/WUR captured in the WF [1] and summary [2].

- Discussion

LP-WUR test case design

Status in the summary [2]:

Issue 3-2-1: Test case categories/scenarios - Proposals - P1: Consider test case for the following categories: Category 1: FR1 LP-WUR monitoring related test case; Category 2: FR1 RRM offloading related test case; Category 3: FR1 RRM relaxation related test case (vivo) - P2: Introduce a test case for to test LR Evaluation (delay) requirements. (Nokia) - P3: Design test cases for LP-WUR measurement and evaluation at RRC_IDLE/INACTIVE state; design test case for relaxed MR measurement for serving cell/neighbour cell measurements (CT) - P4: Define test cases for evaluation of exit conditions for the following cases. (Huawei) - Case 1: UE exist Case 1 to legacy - Case 2: UE exist Case 3 to legacy - Case 3: UE exist Case 1 to Case 3 - P5: RAN4 to define the test cases to verify both MR RRM relaxation and MR RRM offloading scenarios. RAN4 to introduce the RRM relaxation test cases similar as Rel-16 cell reselection (Ericsson) - P6: RAN4 to discuss whether to introduce an applicability to the LP-WUR UE which supports both LP-SS and SSB. (Ericsson) Recommendations:

In the core part, issues including LP-WUR monitoring, RRM offloading and RRM relaxation are mainly discussed in RAN4 RRM meetings, related test cases need to be designed.

Proposal 1: It’s proposed to design test cases for LP-WUR monitoring, RRM offloading and RRM relaxation.

Status in the summary [2]:

Issue 3-2-2: Test case for OOK based LR and OFDB based LR - Proposals - P1: Introduce separate sets of test cases for UE supporting PSS/SSS based LP-WUR and UE supporting LP-SS based LP-WUR. (Nokia Huawei) - P2: RAN4 to introduce a single test for both LP-SS and SSB based LP-WUR UE (Ericsson) Recommendations:

In the core part, LP-WUR measurement requirements are defined based on PSS/SSS and LP-SS, in order to verify LP-WUR measurement requirements based on the two signals, it’s proposed to introduce separate sets of test cases for UE supporting PSS/SSS based LP-WUR and UE supporting LP-SS based LP-WUR.

Proposal 2: Introduce separate sets of test cases for UE supporting PSS/SSS based LP-WUR and UE supporting LP-SS based LP-WUR.

Status in the summary [2]:

Issue 3-2-8: On FR1/FR2 for test case - Proposals - P1: RAN4 to only introduce FR1 based LP-WUS test case. (Ericsson) Recommendations:

Since the core requirements are mainly discussed in FR1, it’s proposed to design test cases in FR1.

Proposal 3: It’s proposed to design test cases in FR1.

- Conclusion

The contribution presents our views on RRM performance requirements for LP-WUS/WUR, with the following proposals:

Proposal 1: It’s proposed to design test cases for LP-WUR monitoring, RRM offloading and RRM relaxation.

Proposal 2: Introduce separate sets of test cases for UE supporting PSS/SSS based LP-WUR and UE supporting LP-SS based LP-WUR.

Proposal 3: It’s proposed to design test cases in FR1.

- References

- R4-2508280, WF on RRM requirements for NR_LPWUS, vivo, RAN WG4 Meeting #115, 19 – 23 May, 2025.

- R4-2505572, Topic summary for [115][223] NR_LPWUS, vivo, RAN WG4 Meeting #115, 19 – 23 May, 2025.