3GPP TSG-RAN WG4 Meeting # 116                                        R4-2510100

Bengaluru, India, Aug. 25 – 29, 2025

Agenda Item: 7.25.3.2

Source: MediaTek Inc.

Title: Discussion on CSI-RS L1 measurement for LTM

Document for: Discussion

# Introduction

RAN#107 agreed on the revised WID “NR mobility enhancements Phase 4” [1]. According to WID, RAN4 is supposed to define requirements for the following measurement enhancements:

- Event triggered L1 report,

- CSI-RS L1 measurements,

- CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch.

Last meeting, there were some discussions [2]. In the next, we would like to provide our views on the open issues of CSI-RS L1 measurements.

# Discussion

## 2.1 Measurement procedure of CSI-RS based L1 measurement

In the past meetings, RAN4 had extensive discussions on the measurement procedure of CSI-RS based L1 measurement. In RAN4#114bis, the following agreement was reached.

#### Issue 4-2: Measurement procedure #### Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 Agreement: - Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting. - Further discuss whether step 2 can be skipped for serving cell or not. #### Issue 4-2-2: How to interpretate the UE capability (for FR2-1) Agreement: - For a UE not supporting the capability, Step 2 is necessary. - The CSI-RS resources in Step 3 should be at least Type-D QCL’ed with the SSB in Step 2. - UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period assuming the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’ - For a UE supporting the capability, Step 2 can be skipped, and if the Step 2 is skipped: - The CSI-RS resources in Step 3 should be Type-D QCL’ed with the SSB in Step 1. - The CSI-RS resources should be periodic or semi-persistent, and the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’ - UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period, and further discuss the detail of beam sweeping factor.

In RAN4#115, the issue “whether step 2 can be skipped for serving cell” was further discussed and now there are several options on the table.

#### Issue 3-2: whether step 2 is skipped for serving cell? Candidate options: - Option A: RAN4 requirements regarding ‘skipping step 2’ do not apply for the case CSI-RS based L1 measurement for LTM candidate cell is configured in legacy CSI-RS based L1 measurement in serving cell in FR2. (QC, HW) - Option A1: Requirement regarding ‘skipping step 2’ is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to ‘off’, is one of the FR2 serving cells. (QC, OPPO) - Option A2: The CSI-RS based L1-RSRP measurement requirement is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to 'off', is one of the FR2 serving cells (QC). - Option B: keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM. (MTK, Apple, E///)

The common part of the all the options is not to define requirements for the new procedure (i.e., skip step 2) for serving cell. The difference is whether to allow such configuration. Our first priority is not to allow “skipping step 2” for serving cell(s). From implementation point of view, we also think it is strange to configure different CSI-RSs for L1 measurement of the same cell at both serving cell’s configuration and LTM candidate’s configuration, not only for the new procedure.  In our understanding, Option A2 is not only for the new procedure. If so, Option A2 is also acceptable to us. We are not sure about the intention of focusing on FR2 only. At this time, we still prefer Option B.

Proposal 1: For CSI-RS based L1-RSRP measurement in FR2-1, step 2 can not be skipped when the candidate cell is a serving cell, e.g., PSCell or SCell.

## 2.2 RRM requirements for intra-f CSI-RS based L1-RSRP measurement when RTD<CP

Last meeting, RAN4 discussed how to define measurement requirements for intra-f CSI-RS based L1-RSRP measurement. The main controversial part is how to handle the collision of CSI-RS resources between different cells. According to the discussion, there are two options.

#### Issue 3-3: Sharing factor for resource collision between serving and neighbour cell Candidate options: - For FR2 CSI-RS L1 measurement requirement, - if the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain,. - Option 1: sharing factor shall be introduced. (HW, QC, E///) - Option 2: introduce measurement restriction as legacy. (MTK, vivo)

For serving cell, the same CSI-RS resource may be configured for L1-RSRP measurement and also other purposes, e.g., RLM, BFD and CBD. One question is whether to have unified solution. After checking SSB based L1-RSRP measurement requirements, we found RAN4 adopted different solutions as shown below. It seems we don’t have unified solution in the legacy.

|  | SSB for L1-RSRP measurement on serving cell | SSB for RLM | SSB for BFD | SSB for CBD |
| --- | --- | --- | --- | --- |
| SSB for L1-RSRP measurement on neighbour cell | sharing factor | Measurement restriction No requirement if collided |  |  |

We don’t think it is an issue. If collision happens, following the requirements defined for SSB based L1-RSRP measurement, UE is supposed to measure SSB of serving cell and neighbour cell(s) TDM-ly. If the SSB collided with neighbour cell L1-RSRP measurement is also used for RLM/BFD/CBD, according to the requirements of RLM/BFD/CBD, it is up to UE implementation on how to measure. Then one of the implementations is following the requirements defined for SSB based L1-RSRP measurement. So if UE follows the rule defined for collision between SSB for L1-RSRP measurement on serving cell and neighbour cell(s), UE meets the requirements defined for both SSB based L1-RSRP measurement and RLM/BFD/CBD. Therefore, even different solutions seem adopted for L1-RSRP measurement and RLM/BFD/CBD, it is also fine.

Back to CSI-RS L1 measurement, if a CSI-RS is only configured for RLM/BFD/CBD, we think it is possible for NW to avoid the collision with CSI-RS L1 measurement of neighbour cell(s). So we propose to define measurement restriction requirements when CSI-RS is configured for RLM/BFD/CBD collided with CSI-RS L1 measurement of neighbour cell(s).

Proposal 2: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define measurement restriction when it is overlapped with CSI-RS configured for RLM/BFD/CBD.

Based on last meetings discussion, it seems not early for NW to make sure CSI-RS for L1-RSRP measurement on different cells non-overlapping in time domain. Given this fact, it is acceptable to introduce a sharing factor if the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain in FR2.

Proposal 3: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, introduce a sharing factor when it is overlapped or adjacent with CSI-RS configured for L1-RSRP measurement on other cells.

## 2.3 Early CSI acquisition

Last meeting, RAN4 discussed whether to define requirements for the baseline (CSI acquisition after LTM CSC MAC-CE reception) and/or early CSI acquisition before LTM CSC MAC CE, and there are several options:

#### Issue 3-9: early CSI acquisition Candidate options: - Option 1: Define timeline for baseline early CSI acquisition. (vivo, Nokia, E///) - Option 2: Define scheduling/measurement restriction requirements for optional early CSI acquisition. (QC) - Option 3: not define RAN4 requirements for early CSI acquisition. (MTK, HW, Apple)

For the baseline (CSI acquisition after LTM CSC MAC-CE reception), UE will acquire CSI after UE is ready to receive data on target cell. For cell switch, we think the first priority is to finish the procedure as fast as possible to cause less interruption. After UE is ready to receive data on target cell, if there is CSI-RS that can be used to acquire CSI information before the first PUSCH occasion to convey the early CSI report (e.g., first CG or DG PUSCH after CSC for RACH-less LTM), then UE can acquire CSI information and report CSI information. If to add CSI acquisition delay in cell switch delay and NW starts scheduling after CSI report, the whole cell switch delay and interruption will be longer, which is not preferred. Therefore we prefer not to define new cell switch delay requirements for the baseline.

For information RAN1#121 Agreement For PUSCH to convey the early CSI report, - For RACH-less LTM, the first CG or DG PUSCH after CSC is used - For RACH-based LTM with CFRA, PUSCH scheduled by RAR or Msg.A - For RACH-based LTM with CBRA, the first CG or DG PUSCH after HARQ-ACK transmission for Msg.4 or Msg.B

Proposal 4: For the baseline (CSI acquisition after LTM CSC MAC-CE reception), additional CSI acquisition delay should not be added in cell switch delay requirements to avoid delaying the whole cell switch procedure. Not to define new cell switch delay requirements for the baseline.

For the optional feature (CSI acquisition before LTM CSC MAC-CE reception), it is far more complex than CSI-RS L1-RSRP measurement. Similar issues with CSI-RS based L1-RSRP measurement from the candidate cell should be discussed, such as the pre-requisite condition, maximum RTD value, gap-less vs. gap-based, etc. But not all the agreements for CSI-RS L1-RSRP measurement are applicable here. For example, UE can perform L1-RSRP measurement on truncated CSI-RS resources (48 RBs), but this may be not workable for CSI acquisition. At least from our point, we prefer not to define any RRM requirements for the optional feature.

Proposal 5: For the optional feature (CSI acquisition before LTM CSC MAC-CE reception), not to define any RRM requirements in R19.

# Summary

In this paper, we provide some views on CSI-RS measurement on neighbor cell. We have the following proposals:

Proposal 1: For CSI-RS based L1-RSRP measurement in FR2-1, step 2 can not be skipped when the candidate cell is a serving cell, e.g., PSCell or SCell.

Proposal 2: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define measurement restriction when it is overlapped with CSI-RS configured for RLM/BFD/CBD.

Proposal 3: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, introduce a sharing factor when it is overlapped or adjacent with CSI-RS configured for L1-RSRP measurement on other cells.

Proposal 4: For the baseline (CSI acquisition after LTM CSC MAC-CE reception), additional CSI acquisition delay should not be added in cell switch delay requirements to avoid delaying the whole cell switch procedure. Not to define new cell switch delay requirements for the baseline.

Proposal 5: For the optional feature (CSI acquisition before LTM CSC MAC-CE reception), not to define any RRM requirements in R19.

# 4 Reference

RP-250339, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.

R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple, RAN4#115.