3GPP TSG-RAN WG4 Meeting # 116                                        R4-2510103

Bengaluru, India, Aug. 25 – 29, 2025

Agenda Item: 7.25.4

Source: MediaTek Inc.

Title: Discussion on RRM performance requirements for R19 mobility

Document for: Discussion

# Introduction

In this contribution, we would like to discuss the RRM performance requirements to be defined for R19 mobility.

# Discussion

## 2.1 Accuracy requirements of CSI-RS based L1 measurement

In RAN4#112bis, the following agreement was reached for CSI-RS L1-RSRP measurement on neighbor cell (R4-2416862). Last meeting, RAN4 agreed not to define RAN4 requirements for L1 measurement with gap or RTD>CP case for CSI-RS based L1 measurement on neighbor cell.

#### Issue 2-11: CSI-RS resource density and BW - Agreements: - For CSI-RS based L1-RSRP measurement on neighbor cell, define the measurement period and accuracy requirements based on Density=3 with (≥) 48 PRBs.

Based on the above agreement, legacy CSI-RS L1-RSRP measurement requirements for serving cell are also applicable to neighbor cell.

Proposal 1: For CSI-RS L1-RSRP measurement on neighbor cell, legacy CSI-RS L1-RSRP accuracy requirements for serving cell are also applicable.

## 2.2 Test cases for event-triggered L1 reporting

For L3 measurement, there are periodic reporting, event-triggered reporting and event-triggered periodic reporting. RAN4 only defined test cases for event-triggered reporting. We think the reason behind is that UE measurement behaviour is almost the same for periodic reporting, event-triggered reporting and event-triggered periodic reporting.

Observation 1: For L3 measurement, RAN4 only defined test cases for event-triggered reporting.

For SSB L1 measurement, RAN4 had defined test cases for intra-f, inter-f without gap and inter-f with MG in R18 for both FR1 and FR2. The only difference between periodic reporting and event-triggered reporting is that UE need to compare the measurement results and determine whether to report. We don’t think it is necessary to test all the cases for event-triggered reporting. For CSI-RS L1 measurement, we believe we will define the test cases for all the scenarios. When it comes to how to compare the measurement results and determine whether to report, there is no difference from SSB L1 measurement.

Proposal 2: For event-triggered L1 reporting, define the following two test cases:

- Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

- Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

2.3 Test cases for CSI-RS L1 measurement

For CSI-RS L1 measurement, we propose to define 5 test cases:

- CSI-RS based L1 RSRP measurement for neighbour cell in FR1

- CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement

- CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement

- CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

- CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

Proposal 3: For event-triggered L1 reporting, define the following 5 test cases:

- CSI-RS based L1 RSRP measurement for neighbour cell in FR1

- CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement

- CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement

- CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

- CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

2.4 Test cases for conditional LTM

According to WID RP-250339, the scope is limited to non-DC. So we don’t need to define test cases for conditional PSCell switch.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

Given that UE needs to pass legacy LTM test cases, there is no need to verify all aspects of cell switch procedure. We only need to focus on the delta part, i.e., cell switch is triggered by UE. Given the fact that the delay and interruption of conditional LTM is exactly the same as CHO if T/F tracking cannot be skipped. We propose the following test list for CLTM and subsequent CLTM.

- CLTM from FR1 to FR1 with early TCI state activation

- CLTM from FR2 to FR2 with early TCI state activation

- subsequent CLTM from FR1 to FR1

- subsequent CLTM from FR2 to FR2

Proposal 4: introduce the following test cases for CLTM and subsequent CLTM.

- CLTM from FR1 to FR1 with early TCI state activation

- CLTM from FR2 to FR2 with early TCI state activation

- subsequent CLTM from FR1 to FR1

- subsequent CLTM from FR2 to FR2

As subsequent CLTM naturally include a complete CLTM procedure. Thus for UE which can pass subsequent CLTM, test case for CLTM can be skipped.

Proposal 5: For UE which pass subsequent CLTM, test case for CLTM can be skipped.

# Summary

In this paper, we provide some view on performance requirements of R19 mobility. We have the following proposals:

Proposal 1: For CSI-RS L1-RSRP measurement on neighbor cell, legacy CSI-RS L1-RSRP accuracy requirements for serving cell are also applicable.

Observation 1: For L3 measurement, RAN4 only defined test cases for event-triggered reporting.

Proposal 2: For event-triggered L1 reporting, define the following two test cases:

- Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

- Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

Proposal 3: For event-triggered L1 reporting, define the following 5 test cases:

- CSI-RS based L1 RSRP measurement for neighbour cell in FR1

- CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement

- CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement

- CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

- CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

Proposal 4: introduce the following test cases for CLTM and subsequent CLTM.

- CLTM from FR1 to FR1 with early TCI state activation

- CLTM from FR2 to FR2 with early TCI state activation

- subsequent CLTM from FR1 to FR1

- subsequent CLTM from FR2 to FR2

Proposal 5: For UE which pass subsequent CLTM, test case for CLTM can be skipped.

# 4 Reference

RP-250339, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.