3GPP TSG-RAN WG4 Meeting # 116	                                                         R4-2510104

Bengaluru, India, Aug. 25 – 29, 2025

Agenda item:	7.17.2

Source:	MediaTek Inc.

Title:	Discussion on General Aspects for AI/ML air

Document for:	Discussion

# Introduction

In this contribution, we provide our views on some general aspects for AI/ML air.

# Discussion on LCM

RAN1 had some discussion on the signalling procedure on applicable functionality reporting for beam management UE-sided model based on the RAN2 input in LS R2-2407848. RAN2 discussed the procedure further based on RAN1 reply LS. The progress is copied here for information.

Fig. 1

Agreement in RAN1#119 - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

Agreement in RAN2#129 - Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1). - The full inference configuration is sent in CSI-ReportConfig - Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability - FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig) - Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause - If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). - The provided periodic CSI configuration should be consistent with reported UE capabilities - FFS option B - Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

Agreement in RAN2#129bis Agreements on option B 1 RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

Here we provide our understanding to current RAN1 and RAN2 agreements.

- Option A includes CSI-ReportConfig in Step 3, which provides a full inference configuration. Therefore, after UE’s applicability confirmation in Step 4 via RRCReconfigurationComplete, the activation process can proceed at least for periodic CSI reporting.

- Option B does not provide full inference configuration in Step 3. Therefore, even after UE indicates its applicability confirmation in Step 4, Step 5 is still needed to provide remaining information (e.g., CSI-ReportConfig). Note that after Step 5, UE still needs to send RRCReconfigurationComplete (denoted as Step 6) back to network.

Last meeting, RAN4 discussed whether and how to define activation delay requirements and reached the following agreements:

Activation delay: - Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction - It is FFS for CSI prediction - For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism. - For semi-persistent CSI reporting - Activation delay starts at the reception of the MAC-CE/DCI - For aperiodic CSI reporting - Activation delay starts at the reception of the DCI

For Option A, as there are different understandings on RAN2 agreement for periodic CSI reporting, RAN4 sent LS for further clarification. RAN4 shall wait for RAN2 reply. But for option B, RAN4 can carry on the discussion.

Proposal 1: For periodic CSI reporting, RAN4 can start the discussion on how to define model activation delay requirements for Option B at first.

For Option B, in step 3, NW configures one set or multiple sets of inference related parameters for applicability report. Before step 5, UE does not know which periodic CSI report will be configured. It is not reasonable to ask UE to start to active the corresponding AI functionality even before step 5. In addition, UE is supposed to reply RRCReconfigurationComplete within a short time duration. It is hardly possible to guarantee that UE can activate the functionality before replying RRCReconfigurationComplete. So for Option B, UE’s activation procedure starts from RRC complete message after Step 5.

Proposal 2: For periodic CSI reporting, UE’s activation procedure starts from RRC complete message after Step 5 for Option B.

To facilitate the requirement discussion, it is important to first align the terminology. Here, we follow the similar naming approach as SCell activation in Section 8.3 of TS38.133. The overall activation delay can be divided into 2 terms: Tactivation_time+ TCSI_Reporting.

- Tactivation_time is the duration UE needs to activate the AI functionality. Note that this duration may not be testable, because it is a pure UE internal operation.

- TCSI_Reporting is the duration between the time that UE has activated the functionality and the time UE transmits the first CSI report in UL. It consists detail components such as the duration waiting for RS, duration for measurement processing, duration for inference and duration waiting for UL.

Fig. 2 illustrates the idea by taking Option B and periodic CSI reporting as an example. In this paper, we will focus on Tactivation_time. The details of TCSI_Reporting are kind of more straightforward.

Fig. 2 Time line of activation delay, assuming Option B and periodic CSI reporting

Proposal 3: For Option B, the overall activation delay of periodic report can be divided into 2 terms: Tactivation_time+ TCSI_Reporting. Tactivation_time is the duration UE needs to activate the AI functionality. TCSI_Reporting is the duration between the time that UE activated the functionality and the time UE transmit the first CSI report in UL.

Now, we focus on Tactivation_time. To define the delay for it, we need to understand what will happen inside UE. The fact is that even if the model is stored already on the UE device, the model may be stored in different potential locations to balance the cost and performance, e.g., outer memory (state B) or inner memory (state C) as shown in Fig.3. In general, the inner memory is more expensive than the outer memory while the access speed is faster.

Fig.3

When UE receives Step 5 RRCReconfiguration, the AI model can be at any state among B, C, and D. In other words, the time required to activate the model is not a fix value. It depends on the initial location of the model. Furthermore, for any given state transition (e.g., BC, CD), the delay still varies with the model size and type.

Observation 1: The Tactivation_time is not a constant value. It depends on the initial location where the AI model is stored in UE upon receiving Step 5 RRC message as well as the model size and type.

However, the delay of Tactivation_time is needed by network. Taking AI/ML BM case 1 as an example, NW may stop transmitting the to-be-predicted beams to reduce overhead. It is important for NW to know when the functionality at UE side has been activated so that NW knows when to stop transmitting the to-be-predicted beams. It is essential to introduce a mechanism to align Tactivation_time between NW and UE. Since Tactivation_time is not a constant value and depends on the conditions such as the size and the internal location of the model, it cannot be pre-defined in RAN4 requirements nor through static UE capability report. The most practical approach is to ask UE to indicate Tactivation_time to network through RRCReconfigurationComplete message.

Observation 2: It is important for NW to know when the functionality at UE side has been activated.

Proposal 4: Send LS to RAN2 to allow UE to report the time needed for functionality activation to network through RRCReconfigurationComplete message.

Semi-persistent/Aperiodic report

For semi-persistent/Aperiodic report, the reporting will be activated by MAC CE/DCI. As the procedure of semi-persistent report would be similar as aperiodic report, we will use semi-persistent report as an example in the next.

One main difference to periodic report is that UE does not need to start to perform inference after reporting applicability. UE will wait for the MAC CE triggering. Hence, it is not necessary to completely activate the functionality right after reporting applicability, like periodic reporting case. On the other hand, upon receiving MAC CE (or DCI), we expect the remaining delay to activate the functionality should not be too long. If upon receiving the MAC CE/DCI command, the model is in state B (outer memory), the time needed to be activated (from state B to E) may be too long, making semi-persistent and aperiodic reporting difficult to use.

To make the mechanism of semi-persistent/aperiodic report work well, and avoid UE to report the model state when receiving MAC CE/DCI command we think it is better to get the functionality partially prepared after reporting applicability, e.g., stay at state C. It means the triggering command, MAC CE or DCI, should be sent to UE after a certain time upon reporting applicability. In this sense, the overall delay can be partitioned into following components: Tactivation_time1+ Tactivation_time2+ TCSI_Reporting.

- Tactivation_time1 is the duration UE needs to partially prepare the AI functionality before UE is able to receive the MAC CE triggering.

- Tactivation_time2 is the remaining duration UE needs to fully activate the AI functionality, starting from the time UE receives the MAC CE.

- TCSI_Reporting is the duration between the time that UE fully activates the functionality and the time UE transmits the first CSI report in UL. It consists detail components such as the duration waiting for RS, duration for measurement processing, duration for inference and duration waiting for UL.

Fig. 4

Usually, RAN1 is supposed to define the timeline of DCI triggered semi-persistent or aperiodic report. It is better to have aligned design for semi-persistent and aperiodic report.

Proposal 5: For semi-persistent and aperiodic report, the applicable functionality shall be partially prepared before the triggering command (MAC CE/DCI).

Proposal 6: For semi-persistent report and aperiodic report, the overall delay can be partitioned into following components: Tactivation_time1+ Tactivation_time2+ TCSI_Reporting. Tactivation_time1 is the duration UE needs to partially prepare the AI functionality before UE is able to receive the MAC CE triggering. Tactivation_time2 is the duration UE needs to fully activate the AI functionality, starting from the time UE receives the MAC CE. TCSI_Reporting is the duration between the time that UE fully activates the functionality and the time UE transmits the first CSI report in UL.

In RAN1#121, RAN1 reached the following agreement for DCI triggered semi-persistent or aperiodic report. The timeline of DCI triggered CSI report is shown in Fig. 5. In our understanding, Z3’+d’ is the inference delay required. (Z3+d) – (Z3’+d’) is similar as Tactivation_time2.

Agreement For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1 and BM-Case 2, when applicable, extend legacy Z3/Z3’ to Z3+d / Z3’+d’, where d and d’ are reported by UE per SCS for BM-Case 1 and BM-Case 2 respectively - Detailed values of d and d’ can be further discussed in UE feature.

Fig. 5

To align with RAN1’s agreement, for MAC CE triggered semi-persistent report, we propose Tactivation_time2 = THARQ +3ms + X, where 3ms is the time for MAC CE decoding, and X is up to UE capability. If proposal 6 can be agreed, we propose X as 2ms for AI/ML BM.

Proposal 7: For MAC CE triggered semi-persistent report, Tactivation_time2 = THARQ +3ms + X, where 3ms is the time for MAC CE decoding, and X is up to UE capability. If proposal 6 can be agreed, candidate value of X can be 2ms for AI/ML BM.

In the past meetings, RAN4 also discussed whether to define functionality deactivation requirements. In our view, if deactivating a functionality would cause interruption on data or other functionalities, then functionality deactivation requirements shall be defined. Otherwise, functionality deactivation requirements are mainly defined from NW’s perspective, e.g., NW gets to know when to release the reporting resources configured for a particular functionality. We understand that it highly depends on implementation whether there will be interruption on data or other functionalities. But if there is interruption on data or other functionalities, especially with long interruption, the use case may not work well. So we prefer not to define functionality deactivation interruption requirements. As for functionality deactivation delay requirements, it can be up to NW vendors.

Proposal 8: No interruptions are expected during functionality deactivation procedure. Open to define functionality deactivation delay requirements or not.

# Conclusion

In this contribution, we provide our views for AI/ML general aspects. Observations and proposals are summarized as follows.

Proposal 1: For periodic CSI reporting, RAN4 can start the discussion on how to define model activation delay requirements for Option B at first.

Proposal 2: For periodic CSI reporting, UE’s activation procedure starts from RRC complete message after Step 5 for Option B.

Proposal 3: For Option B, the overall activation delay of periodic report can be divided into 2 terms: Tactivation_time+ TCSI_Reporting. Tactivation_time is the duration UE needs to activate the AI functionality. TCSI_Reporting is the duration between the time that UE activated the functionality and the time UE transmit the first CSI report in UL.

Observation 1: The Tactivation_time is not a constant value. It depends on the initial location where the AI model is stored in UE upon receiving Step 5 RRC message as well as the model size and type.

Observation 2: It is important for NW to know when the functionality at UE side has been activated.

Proposal 4: Send LS to RAN2 to allow UE to report the time needed for functionality activation to network through RRCReconfigurationComplete message.

Proposal 5: For semi-persistent and aperiodic report, the applicable functionality shall be partially prepared before the triggering command (MAC CE/DCI).

Proposal 6: For semi-persistent report and aperiodic report, the overall delay can be partitioned into following components: Tactivation_time1+ Tactivation_time2+ TCSI_Reporting. Tactivation_time1 is the duration UE needs to partially prepare the AI functionality before UE is able to receive the MAC CE triggering. Tactivation_time2 is the duration UE needs to fully activate the AI functionality, starting from the time UE receives the MAC CE. TCSI_Reporting is the duration between the time that UE fully activates the functionality and the time UE transmits the first CSI report in UL.

Proposal 7: For MAC CE triggered semi-persistent report, Tactivation_time2 = THARQ +3ms + X, where 3ms is the time for MAC CE decoding, and X is up to UE capability. If proposal 6 can be agreed, candidate value of X can be 2ms for AI/ML BM.

Proposal 8: No interruptions are expected during functionality deactivation procedure. Open to define functionality deactivation delay requirements or not.

# Reference

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface