3GPP TSG-RAN WG4 Meeting # 116	                                                      R4-2510106

Bengaluru, India, Aug. 25 – 29, 2025

Agenda item:	7.19.2

Source:	MediaTek Inc.

Title:	Discussion on general aspects for AI mobility

Document for:	Discussion

- Introduction

RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

In this contribution, we would like to discuss some general issues.

- Discussion

2.1 Ground truth

In the past meetings, RAN4 reached the following progress on how to determine the ground truth. In the next, we would like to analyze more.

RAN4#114 Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered. - Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2. Issue 2-1-2: Ground Truth Definition for RSRP accuracy - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list RAN4#114bis Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1 Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

The fact is that L3 measurement works well in the real field. In the real field, suppose there are several UEs at the same location. No matter how the UEs perform L3 measurement, no matter how they do L1 filtering and L3 filtering, the reported measurement results at the same reporting occasion should be similar. There will be some difference, but the divergence should not be too large.

Then back to the test, under the same channel realization, the reported RSRP value at the same reporting occasion of different UEs shall be similar too. There will be some difference, but the divergence should not be too large.

We totally understand other companies’ concern on using the transmission power at TE to derive the ground truth. After checking the simulation results provided by [2] and [3], we also question the feasibility. As commented by other company, L3 filtering is kind of up to UE implementation. We checked the impact on the ground truth under different measurement intervals and shifted start positions in Fig.1. The figure shows CDF of the difference between filtered L3-RSRP with different measurement intervals & shifted start positions and filtered L3-RSRP with an interval of 40ms & 0ms shifted start position. In the simulation, we adopted the same dataset used by RAN2 for scenario 2 (FR1 to FR1 intra-frequency temporal domain case B). Sliding window is adopted. It can be observed that the RSRP difference in the range of [5%,95%] is less than ±2dB.   We think it is acceptable and it also justifies why the whole L3 measurement mechanism works well in the real field allowing proprietary filtering.

Fig. 1

We understand that different L1 filtering and L3 filtering may be adopted by UE vendors. It may be necessary to further check whether it is workable to use the transmission power at TE to derive the ground truth. In our understanding, the simulations shall at least include the following:

- Similar simulations as shown in Xiaomi and QC’s contributions last meeting

  - The genie aided measurement results considering different L1 filtering and L3 filtering.

- The margin to add when using different L1 filtering and L3 filtering rather than a defined L1 filtering and L3 filtering.

  - It also requires simulating the performance of the model which is supposed to be done in WI phase.

Proposal 1: If to derive the ground truth through transmitted power at TE, additional margin should be considered due to different L1 filtering and L3 filtering may be adopted by UE vendors. The value of the margin can be obtained through simulations.

Another alternative is to use the reported results as ground truth. If so, the test will contain two steps. In one step, TE will not transmit SSB on some occasions/frequency layers and UE will predict the measurement results at those occasions/frequency layers. In another step, TE will transmit SSB on all the occasions/frequency layers. Even it is possible to make the channel realization in the two steps the same, UE measurement behaviour will not be exactly the same. There was proposal that the ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. We think it makes sense. In our view, it is the instantaneous fluctuation at different occasions due to fast fading which results to varied value of ground truth corresponding to different L1 filtering and L3 filtering.  Even to use the reported results as ground truth, the uncertainty introduced by “Yms” shall be considered too. So no matter which alternative finally adopted, the impact of the instantaneous fluctuation at different occasions due to fast fading shall be considered.

Proposal 2: If to use reported RSRP as ground truth, the ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. The value of Y is relevant to UE implementation.

At this stage, due to limit simulations or analysis on the value of the margin to be added for Alt.1 and the value of Y for Alt.2, we suggest not to do down-selection at SI stage.

Proposal 3: During the SI, RAN4 not to do down-selection on the two Alternatives of ground truth definition.

2.2 How to handle cells not detectable?

It is observed that there were some proposals to study the impacts on cell identification for L3 cell-level measurement prediction by AI/ML or whether the predicted cell is known or unknown. We agree that this is an important issue for further study. Due to cell identification related discussion is usually conducted in RAN4, RAN2 may not realize the issue yet and it is more appropriate to discuss the issue in RAN4.

For scenario 2 and 4, since they are intra-cell prediction, UE is supposed to detect and measure the cell before prediction. So cell identification is not an issue in the test or in the field. But in the simulations, the data with low SNR (i.e., the cell is not detectable) shall be excluded in the dataset at first.

Proposal 4: When do the simulations, the data containing cells with too low SNR (e.g., <-6dB) should be excluded from the dataset.

For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), since measurement on target cell is not always conducted, RAN4 should discuss the impact on cell identification.

In RAN2 simulations, only co-located cases are considered. Even the cells of two frequency layers are co-located. There are at least two different scenarios. One scenario is that the coverage of the two cells are similar. Another is the coverage of the two cells are different. At least in our R2 simulations, the BS configurations (e.g., antenna pattern, downtilt angle) of the two frequency layers are also the same (the coverage of the two cells are similar). In this scenario, as the coverage of the two cells are nearly the same, UE can know which cell is detectable based on the detection result of the measured frequency layer. But if the coverage of two co-located cells are different, it is hard to know which cell is detectable on the to-be-predicted frequency layer without measurement. In RAN4, we may need to discuss the assumptions on scenario 3 at first.

Proposal 5: For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), discuss whether co-located cells on different frequencies are with varied BS configurations (e.g., antenna pattern, downtilt angle) or not.

Though cell detection, UE can get Cell ID and rough timing. Regarding Cell ID, if Cell#X’ have similar coverage as Cell#X, it is possible to inform UE cell ID of Cell#X’ and UE can know which cell’s RSRP that UE is predicting. But if Cell#X’ have different coverage from Cell#X, we are not sure whether it is workable to inform UE cell ID of the neighbor cell(s) by serving cell. If cell ID of the to-be-predict cell is unknown, we don’t know how NW understands UE’s prediction results.

Proposal 6: For scenario 3 (inter-frequency measurement prediction), discuss how UE gets cell ID of the to-be-predict cell(s) for reporting the prediction results to network.

2.3 Impact of Measurement error

2.3.1 Measurement error model

As guided by the progress last meeting, we would like to discuss how to add measurement error for AI mobility in FR1.

Issue 2-1-7: Impacts of measurement error on prediction accuracy WF: Companies are encouraged to bring proposals on how to capture factors to be considered for error models to consider the impact of measurement errors on prediction accuracy.

Although it is L3 measurement considered in AI mobility, UE is supposed to measure each sample and derive L3 measurement result after L1 filtering and L3 filtering. For each measurement occasion, BB error and RF error will be introduced independently. After L1 filtering, BB error will be further flattened. But from the point of adding error, BB error and RF error shall be added on each measurement occasion independently. In other words, when derive BB error distribution, one shot measurement is assumed.

The way to add measurement error is shown in Fig. 2:

- In SLS simulation, SINR of each Tx beam is calculated dependently.

  - In the calculation of SINR, noise is generated as Gaussian distribution and UE receiver noise figure is set to 9dB which is widely used in FR1. Useful signal and interferences from other cells are calculated considering Tx power, path-loss, channel gain and beamforming gains.

- After calculating SINR of each beam, we look up the table to get the corresponding variance.

  - If the SINR is not an integer or out of the range [-24:3:9] dB, we use interpolation method to obtain the mean and variance.

- A random BB error is generated following Gaussian distribution with the mean and variance corresponding to the SINR value.

- RF error is modelled as Gaussian distribution under ±3 dB RF error (μ=0,  σ2=3). A random RF error is generated for all the beams of the same frequency layer per SMTC.

Fig. 2

Proposal 7: For AI mobility, in FR1, measurement error should be added as:

- BB error:

  - Added independently per Tx beam per cell per measurement occasion according to the corresponding SINR.

  - The distribution of BB error under certain SINR is obtained through LLS assuming one shot measurement.

- RF error:

  - Modelled as Gaussian distribution under ±3 dB RF error (μ=0,  σ2=3).

  - A random RF error is generated for all the beams of the same frequency layer per SMTC.

2.3.2 Simulation results

We checked the impact of measurement error with adding measurement error as above. In the simulation, the settings are as below:

- Scenario: temporal prediction

- OW: 4 x 40ms, PW: 40ms, 80ms, 120ms, 160ms, 200ms, 240ms, 280ms, 320ms

- L1 filtering: Sliding window

- L3 filtering: k=4

- Inputs to AI model: RSRPs of two cells (the first two strongest) at 4 occasions in OW (2x4)

- Output from AI model: RSRPs of two cells at 8 predicted occasions (2x8)

- Measurement error is added at both training and inference phase. The measured RSRP are used as ground truth, i.e., there is measurement error in ground truth.

- Inference performed when enter condition of A3 event (threshold 2dB, hysteresis=0) is met.

Table. 1 shows the performance of absolute RSRP difference and relative RSRP difference at 90% tile point for difference cases (with error or without error, RSRP difference at all the PWs are counted together). It can be observed that the impact of measurement error on predicted absolute RSRP accuracy and relative RSRP accuracy is marginal. It may because the error is averaged after L1 and L3 filtering.

Table 1

| RSRP difference at 90% tile point | Absolute RSRP difference [dB] | Relative RSRP difference [dB] |
| --- | --- | --- |
| no RSRP error | 1.83 | 2.39 |
| BB error only | 1.96 | 2.57 |
| RF error only | 2.05 | 2.73 |
| BB + RF error | 2.03 | 2.68 |

Observation 1:  The impact of measurement error is marginal.

2.4 Generalization

RAN2 have done some evaluation on generalization performance, i.e.,

- across different UE speeds

- across different cell configurations

  - Umi: ISD = 200m, BS antenna height = 10m, BS Tx power = 40dBm

  - Uma: ISD = 500m, BS antenna height = 25m, BS Tx power = 44dBm

- across different frequency prediction directions

  - 2GHz  4GHz

  - 4GHz  2GHz

However, we don’t think it is sufficient as the generalization performance over some measurement/evaluation related parameters has not been evaluated yet. E.g., different L3 filtering configurations, event thresholds, SMTC period, MGRP. We prefer the model can be agnostic to these configurations. Therefore, we suggest RAN4 continuing working on the generalization issue w.r.t. different parameters in the work item phase.

Observation 2: For generalization performance evaluation, RAN2 didn’t touch measurement/evaluation related parameters, e.g., different L3 filtering configurations, event thresholds, SMTC period, MGRP.

Proposal 8: RAN4 shall keep evaluating the generalization performance over measurement/evaluation related parameters, e.g., different L3 filtering configurations, event thresholds, SMTC period, MGRP and etc.

We have simulated the model performance under different L3 filtering configurations, i.e., k= 0, 2,4,6,8. In the simulation, the settings are as below:

- Scenario: temporal prediction

- OW: 4 x 40ms, PW: 40ms, 80ms, 120ms, 160ms

- L1 filtering: Sliding window

- Input to AI model: RSRP of two cells (the first two strongest) at 4 occasions in OW (2x4)

- Output from AI model: RSRP of two cells at 4 predicted occasions (2x4)

- Measurement error is added at both training and inference phase. The measured RSRP are used as ground truth, i.e., there is measurement error in ground truth.

- Inference performed when enter condition of A3 event (threshold 2dB, hysteresis=0) is met.

Table. 2 shows the performance of absolute RSRP difference and relative RSRP difference at 90% tile point with different k (RSRP difference at all predicted instances the PWs are counted together). It can be observed that the predicted RSRP is more accuracy with a larger k (historical samples hold a greater weight). When L3 filtering is not used, i.e., k=0, the accuracy performance is the worst. As the performance difference between k=0 and other values is larger, it seems not good to define a unified requirements for all L3 filtering configurations. RAN4 needs to discuss how to define the accuracy requirements for different L3 filtering configures.

Table 2

| k | Absolute RSRP difference [dB] | Relative RSRP difference [dB] |
| --- | --- | --- |
| 0 | 3.42 | 4.25 |
| 2 | 1.12 | 1.34 |
| 4 | 0.76 | 0.93 |
| 6 | 0.49 | 0.62 |
| 8 | 0.35 | 0.45 |

Observation 3: The performance with different L3 filtering configurations diverge.

Proposal 9: Further discuss how to define RSRP accuracy requirements for different L3 filtering configurations.

2.5 Prioritize measurement event prediction for temporal domain prediction

According to the SID, there are two cases, RRM measurement prediction and measurement event prediction. If indirect prediction is used for measurement event prediction, RRM measurement prediction is the basis. Due to this, both RAN2 and RAN4 focused on RRM measurement prediction at the beginning. But in the field, event triggered L3 measurement report is widely used, and periodic L3 measurement report is seldomly used. For legacy L3 measurement, RAN4 only defined test cases for event triggered L3 measurement report. We know this is the last meeting for the SI and it is meaningless to discuss which case to be prioritized. But as we are paving the way for R20 WI, we think it is important to discuss which case should be the focus.

Observation 4: For legacy L3 measurement, RAN4 only defined test cases for event triggered L3 measurement report.

For temporal domain prediction, there are two cases, Case A and Case B. As in temporal domain prediction Case A, NW will not skip any SSB transmission. For the sake of power saving, it is not necessary for UE to predict the future all the time. When to do inference should be totally up to UE. We think it is more reasonable that UE do the prediction only when an event may occur, i.e., only do the prediction for measurement events.

From TR38.744 In intra-frequency temporal domain case A, continuous measurement results in PW are predicted by continuous historical measurement result(s) in OW. Then OW and PW slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding. One example is illustrated in Figure 5.2.1.1-1: Figure 5.2.1.1-1: Example of intra-frequency temporal domain case A

For temporal domain prediction Case B, according to TR38.744, there are mainly two skipping patterns as shown below. Example 1 would have big impact on legacy UEs (not supporting Case B) as SSB is not transmitted periodically. Example 2 may have impact on legacy UEs too, if SSB is not transmitted periodically. We suggest prioritizing the cases without legacy UE impact, e.g., Case A.

From TR38.744 Figure 5.2.1.1-2: Skipping pattern example 1 of intra-frequency temporal domain case B Figure 5.2.1.1-3: Skipping pattern example 2 of intra-frequency temporal domain case B

Proposal 10: Prioritize the cases without legacy UE impact, e.g., Case A.

Even for case B, we think it is not necessary for UE to predict the skipped occasions especially when RSRP of serving cell is high enough and neighboring cell is low. That is why it is workable to extend the measurement delay for UEs not at cell edge in R16 power saving. When to do inference should be up to UE. What matters is that UE can report in time when an event is met. So for temporal domain prediction case B, we think RAN4 should focus on measurement event prediction too.

Proposal 11: For temporal domain prediction, measurement event prediction should be the focus and RAN4 only need to define test cases for measurement event prediction.

- Conclusion

In this contribution, we provide our views for RRM measurement prediction. Proposals and observations are summarized as follows.

Proposal 1: If to derive the ground truth through transmitted power at TE, additional margin should be considered due to different L1 filtering and L3 filtering may be adopted by UE vendors. The value of the margin can be obtained through simulations.

Proposal 2: If to use reported RSRP as ground truth, the ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. The value of Y is relevant to UE implementation.

Proposal 3: During the SI, RAN4 not to do down-selection on the two Alternatives of ground truth definition.

Proposal 4: When do the simulations, the data containing cells with too low SNR (e.g., <-6dB) should be excluded from the dataset.

Proposal 5: For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), discuss whether co-located cells on different frequencies are with varied BS configurations (e.g., antenna pattern, downtilt angle) or not.

Proposal 6: For scenario 3 (inter-frequency measurement prediction), discuss how UE gets cell ID of the to-be-predict cell(s) for reporting the prediction results to network.

Proposal 7: For AI mobility, in FR1, measurement error should be added as:

- BB error:

  - Added independently per Tx beam per cell per measurement occasion according to the corresponding SINR.

  - The distribution of BB error under certain SINR is obtained through LLS assuming one shot measurement.

- RF error:

  - Modelled as Gaussian distribution under ±3 dB RF error (μ=0,  σ2=3).

  - A random RF error is generated for all the beams of the same frequency layer per SMTC.

Observation 1:  The impact of measurement error is marginal.

Observation 2: For generalization performance evaluation, RAN2 didn’t touch measurement/evaluation related parameters, e.g., different L3 filtering configurations, event thresholds, SMTC period, MGRP.

Proposal 8: RAN4 shall keep evaluating the generalization performance over measurement/evaluation related parameters, e.g., different L3 filtering configurations, event thresholds, SMTC period, MGRP and etc.

Observation 3: The performance with different L3 filtering configurations diverge.

Proposal 9: Further discuss how to define RSRP accuracy requirements for different L3 filtering configurations.

Observation 4: For legacy L3 measurement, RAN4 only defined test cases for event triggered L3 measurement report.

Proposal 10: Prioritize the cases without legacy UE impact, e.g., Case A.

Proposal 11: For temporal domain prediction, measurement event prediction should be the focus and RAN4 only need to define test cases for measurement event prediction.

- Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur), RAN#105, Melbourne, Australia, September 9-12, 2024.

- R4-2503308, Discussion on impacts on RAN4 requirement for RRM measurement prediction in AI mobility, Xiaomi, RAN4#114bis.

- R4-2504524, Impact of AI based mobility on RRM measurement prediction, Qualcomm, RAN4#114bis.