3GPP TSG-RAN WG4 Meeting # 116	                                                      R4-2510111

Bengaluru, India, Aug. 25 – 29, 2025

Agenda item:	4.4.2

Source:	MediaTek Inc.

Title:	Discussion on the usage of TCI-ActivatedConfig for SCell activation

Document for:	Discussion

# Introduction

RAN4 discussed the requirements related to SCell activation using tci-ActivatedConfig for a couple of meetings. In RAN4#113, the CR R4-2420200 was agreed. But we recently found that after the agreed CR, the understanding is not aligned with RAN2 spec. In this discussion paper, we would like to further discuss the issue.

# Discussion

In Rel-17, tci-ActivatedConfig is introduced by RAN2 for direct scell activation. In the previous discussions, it seems RAN4 misunderstood that tci-ActivatedConfig can also configured at scell addition without direct activation. Based on this understanding, the CR R4-2420200 was agreed in RAN4#113 to clarify that the indicted TCI can be used at the first scell activation after scell addition. However, we recently found that it is clear specified in RAN2 spec that when this TCI configuration “tci-ActivatedConfig” is used for SCell, it is only used in case of “sCellState is configured” (as highlighted part below), which is the direct SCell activation.

ServingCellConfig ::= SEQUENCE { <Skip> tci-ActivatedConfig-r17 TCI-ActivatedConfig-r17 OPTIONAL, -- Cond TCI_ActivatedConfig <Skip> } tci-ActivatedConfig If configured for an SCell, or if configured for the PSCell when the SCG is being activated upon the reception of the containing message, the UE shall consider the TCI states provided in this field as the activated TCI states for PDCCH/PDSCH reception on this serving cell. If configured for the PSCell when the SCG is indicated as deactivated in the containing message: - the UE shall consider the TCI states provided in this field as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent - if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the TCI states provided in this field for PDCCH as RS for RLM, respectively for BFD. When this field is absent for the PSCell and the SCG is being deactivated: - the UE shall consider the previously activated TCI states as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent - if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the previously activated TCI states for PDCCH as RS for RLM, respectively for BFD.. Conditional Presence; Explanation. TCI_ActivatedConfig; This field is optional Need N for SCells if sCellState is configured, otherwise it is absent. This field is optional Need S for the PSCell when the SCG is indicated as deactivated or is being activated, otherwise it is absent. This field is absent for the PCell.. SCellConfig ::= SEQUENCE { sCellIndex SCellIndex, sCellConfigCommon ServingCellConfigCommon OPTIONAL, -- Cond SCellAdd sCellConfigDedicated ServingCellConfig OPTIONAL, -- Cond SCellAddMod ..., [[ smtc SSB-MTC OPTIONAL -- Need S ]], [[ sCellState-r16 ENUMERATED {activated} OPTIONAL, -- Cond SCellAddSync secondaryDRX-GroupConfig-r16 ENUMERATED {true} OPTIONAL -- Need S ]], [[ preConfGapStatus-r17 BIT STRING (SIZE (maxNrofGapId-r17)) OPTIONAL, -- Cond PreConfigMG goodServingCellEvaluationBFD-r17 GoodServingCellEvaluation-r17 OPTIONAL, -- Need R sCellSIB20-r17 SetupRelease { SCellSIB20-r17 } OPTIONAL -- Need M ]], [[ plmn-IdentityInfoList-r17 SetupRelease {PLMN-IdentityInfoList} OPTIONAL, -- Cond SCellSIB20-Opt npn-IdentityInfoList-r17 SetupRelease {NPN-IdentityInfoList-r16} OPTIONAL -- Cond SCellSIB20-Opt ]] } ##### 5.3.5.5.9 SCell Addition/Modification The UE shall: 1> for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition): 2> add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated; 2> if the sCellState is included: 3> configure lower layers to consider the SCell to be in activated state; 2> else: 3> configure lower layers to consider the SCell to be in deactivated state;

In RAN2#129, they also agreed the CR R2-2501371, in which a UE capability for SCell activation with TCI activation configuration was introduced. As highlighted below, the prerequisite to support the new capability is to support direct scell activation.

directSCellActivationWithTCI-r17 Indicates whether the UE supports direct NR SCell activation with activated TCI states configuration (i.e. tci-ActivatedConfig). A UE supporting this feature shall also indicate support of at least one of directMCG-SCellActivation-r16, directMCG-SCellActivation-r17, directMCG-SCellActivationResume-r16, directMCG-SCellActivationResume-r17, directSCG-SCellActivation-r16, directSCG-SCellActivation-r17, directSCG-SCellActivationResume-r16, and directSCG-SCellActivationResume-r17.

The agreed changes in CR R4-2420200 are for the case that tci-ActivatedConfig is configured at scell addition without direct activation, which conflicts with RAN2 spec. So we propose to revert back to the original.

One of the changes in CR R4-2420200 - Provided this is the initial activation after SCell addition, and - UE receives the SCell activation command and TCI-ActivatedConfig is configured for the SCell.

Observation 1: When TCI configuration “tci-ActivatedConfig” is used for SCell, it is only used for direct SCell activation. The agreed changes in CR R4-2420200 are for the case that tci-ActivatedConfig is configured at scell addition without direct activation, which conflicts with RAN2 spec.

Proposal 1: Remove scell activation requirements related to the case that tci-ActivatedConfig is configured at scell addition without direct activation.

# Conclusion

In this contribution, we provide our views for RRM measurement prediction. Proposals and observations are summarized as follows.

Observation 1: When TCI configuration “tci-ActivatedConfig” is used for SCell, it is only used for direct SCell activation. The agreed changes in CR R4-2420200 are for the case that tci-ActivatedConfig is configured at scell addition without direct activation, which conflicts with RAN2 spec.

Proposal 1: Remove scell activation requirements related to the case that tci-ActivatedConfig is configured at scell addition without direct activation.