3GPP TSG-RAN WG4 Meeting#116		R4-2510119

Bengaluru, India, Aug 25th -29th, 2025

Agenda item:	7.22.3.2

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on RF requirements for ambient IoT device 1

Document for:	Discussion

Introduction

In RAN4#115 meeting, a WF [1] was agreed. This contribution continues to discuss remaining issues as follows.

- Topic 3-1: Transmit output power Issue 3-1-1: Transmit output power Agreement in RAN4#115: - Performance metric: EIRP - FFS on how to determine device’s peak antenna gain direction - FFS on whether to measure multiple directions - Take [10] dB backscatter loss for OOK as starting point, based on the assumption that the incoming CW signal and the backscatter signal are aligned with the peak gain direction of the device antenna. - FFS for BPSK - The power of the backscattered signal shall include only the 1st lower sideband and the 1st upper sideband, excluding the carrier itself. - Topic 3-2: Modulation quality Issue 3-2-1: SFO requirement Agreement in RAN4#115: - No need to directly test SFO if SFO can be indirectly verified via other test. - FFS whether to explicitly define SFO requirements or not. Issue 3-2-2: Modulation quality requirements Agreement in RAN4#115: - The modulation quality won’t be introduced, and it can be verified by the backscattering loss for OOK or BPSK. - Topic 3-3: Emission requirements Issue 3-3-1: Emission requirements Agreement in RAN4#115: - Use EIRP as performance metric - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing - The applicable frequency range for SEM: from the CBW edge to [OOB boundary] - Define flat requirements for SEM as starting point, [9dBc~14dBc] based on achievable harmonic performance - Define requirements applicable to all CBWs, FFS on how to reduce the test burden. - Use the [maximum] CW input power level, FFS on the value. Issue 3-3-1: spurious emission requirements Agreement in RAN4#115: - Use EIRP as performance metric - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing - OOB boundary - Option 1: D2R channel bandwidth plus minimum D2R channel bandwidth - Option 2: max of 500kHz and 10 times NB where NB equals to D2R CBW - Other options are not precluded - Define requirements applicable to all CBWs, FFS on how to reduce the test burden. - Use the [maximum] CW input power level, FFS on the value. - Topic 3-4: Reference sensitivity Agreement in RAN4#115: - Using [-34dBm] as starting point to define REFSENS requirements, based on the peak gain direction of device antenna. - Consider to define orientation requirements with following options: - Option 1: min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Xth percentage - Option 2: spatial averaging EIS - Option 3: EIS in the peak direction - Testing metric: 10% miss detection rate, FFS on whether to consider false alarm - FFS for the FRC, consider following as starting point: SCS; 15; 15; 15; 15. PRB; 1; 2; 3; 4. TBS; 20; 20; 20; 20. CRC; 6; 6; 6; 6. M; 6; 12; 24; 24. - Topic 3-5: Others Issue 3-5-1: Maximum input power Agreement in RAN4#115: - Define maximum input level at peak antenna direction, using [-4dBm] as starting point. Issue 3-5-2: Occupied bandwidth Agreement in RAN4#115: - Do not define occupied bandwidth requirements.

Discussion

TX requirements

- Transmitter output power

We propose letting the device declare the peak antenna gain direction in order to save testing time and cost. In our understanding, R2D and D2R will share the same antenna, therefore, we can choose Tx or Rx requirements to measure multiple directions performance. For Rx requirements, it is easier to measure multiple directions performance compared to Tx requirements.

Proposal 1: Device declares the peak antenna gain direction.

Based on some papers, backscatter loss is related to incident power. The backscatter loss is 25dB when the input CW power range is from -15dBm to -5dBm; the backscatter loss is 10dB when the input CW power range is from -35dBm to -15dBm. In our understanding, we don’t need to describe the total characteristic curve of the backscatter, we can define two points with input CW power. One is the high backscatter loss, and the other one is the low backscatter loss. In order to avoid to limit the implementation of device, we can take the middle input CW power as the two points separately. Therefore, we can define backscatter loss as follow for OOK modulation. The backscatter loss is 10dB@input CW power= -25dBm and 25dB@input CW power= -10 dBm. In addition, regarding BPSK for backscatter loss, it is different from OOK. First of all, there is no energy when OOK chip is “0” which means the off state. However, BPSK has energy whatever BPSK is “+1” or “-1”, therefore, OOK backscatter loss is larger more than 3dB compared to BPSK. In addition, from the receiver’s perspective, it is needed to remove DC component to avoid the saturation of receiver amplifier and maintain the judge threshold. In summary, OOK backscatter loss is larger more than 6dB compared to BPSK backscatter loss in theory. In order to give some margin for BPSK from an implementation perspective, we can define BPSK backscatter loss is 6dB@input CW power= -25dBm and 21dB@input CW power= -10dB.

Proposal 2: Define the backscatter loss as follows.

- For OOK, 10 dB @input CW power= -25dBm and 25dB@input CW power= -10 dBm.

- For BPSK, 6 dB @ input CW power= -25dBm and 21dB@input CW power= -10 dBm.

- SFO requirement

10% SFO is assumed as the minimum performance for devices. Whether to define SFO requirements or not is needed to be discussed. When defining the channel bandwidth, we have calculated the impact of SFO into it. The SEM requirement indirectly reflects the impact of SFO. Most importantly, the frame structure of D2R includes midamble. The purpose of inserting midamble is to estimate the SFO more accurately from the reader’s perspective to decrease the impact of SFO and improve the demodulation of D2R. To sum up, we can guarantee the SFO requirements in many aspects. Therefore, from our perspective, there is no need to define SFO requirements.

Proposal 3: No need to define SFO requirement explicitly.

- SEM requirements

We can be based on achievable harmonic performance to define flat requirements. The 3rd harmonics and 5rd harmonics are out of CBW edge, which are included in SEM requirements. The power of the 3rd harmonics occupied of the signal power is 1/9. 10 log (1/9) is -9.5dB. However, the signal of D2R is 2SB.  We need to calculate the 1SB of 3rd harmonics to define the SEM requirements. The power of the 3rd harmonics of 1SB occupied of the signal power is 9.5+3=12.5dBc. In our understanding, there is no filter to suppress the 3rd harmonics, therefore, we can define 12.5dBc as SEM requirements. In addition, implementation needs some margin. We can define 12dBc as SEM requirements.

Proposal 4: Define flat 12dBc/1SB as SEM requirements.

- Spurious emission requirements

Regarding the boundary of OOB, according to the ITU SM 329, we need to differentiate narrow band or broadband. For broadband, in the case of some digital, broadband or pulse-modulated systems, ±250% may apply to channel separation. For narrow-band, 10NB (necessity bandwidth) may be applied.

As all we know, spurious emission needs to be ≤ -36dBm. We assume the maximum backscatter power is -10dBm, then we can derive the power of 21st harmonic can meet the requirement. The span between fc and 21st harmonic is (1/2*Tchip)*21. If we assume the Tchip is 0.69 us, then the span between fc and 21st harmonic is 15.12MHz, then we can calculate the 10 times NB (4/Tb), then the 10 times NB(R=1) is 28.8M. If we assume the Tchip is 133.3us, then the span between fc and 21st harmonic is 78.7 kHz, then the 10 times NB(R=1) is 165 kHz. If we assume the Tchip is 2.08 us, then the span between fc and 21st harmonic is 3.75 MHz, then the 10 times NB(R=1) is 10.55MHz; then the 10 times NB(R=64) is 5.45MHz, if we define the NB is transmission bandwidth, then the 10 times NB is 150kHz. The spurious emission cannot meet the requirements. If we define CBW as the NB, then A-IoT device1 can meet the ITU requirements.

Proposal 5: Define the maximum 500 kHz or 10 times NB as the boundary A-IoT out of band and general spurious emission domain and the NB (necessity bandwidth) is the CBW of D2R. We can assume the maximum backscatter power is -10dBm (the distance between CW and device is 1m).

RX requirements

- Reference sensitivity

We can derive reference sensitivity according to the link budget table 1 from TR 38.769. According to the last meeting, the coverage target is 15m.  The pathloss isPL=33.63+21.9log10d3D+20log10fc.

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| (0) System configuration |  |  |  |
| [0A] | Scenarios | D1T1-A1/A2/B/C D2T2-A1/A2/B/C | D1T1-A1/A2/B/C D2T2-A1/A2/B/C |
| [0A1] | CW case | N/A | 1-1/1-2/1-4/2-2/2-3/2-4 |
| [0B] | Device 1/2a/2b | Device 1/2a/2b | Device 1/2a/2b |
| [0C] | Center frequency (MHz) | 900MHz (M), 2GHz (O) | 900MHz (M), 2GHz (O) |
| [0D] | Topology/Pathloss model | For D2T2: BS pathloss model is reused for intermediate UE with antenna height = 1.5 m [0D]-Alt1: InF-DL NLOS [0D]-Alt2: InH-Office LOS For D1T1: InF-DH NLOS | For D2T2: BS pathloss model is reused for intermediate UE with antenna height = 1.5 m [0D]-Alt1: InF-DL NLOS [0D]-Alt2: InH-Office LOS For D1T1: InF-DH NLOS |
| (1) Transmitter |  |  |  |
| [1D] | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | For BS: 2(M) or 4(O) antenna elements for 0.9 GHz For Intermediate UE: 1(M) or 2(O) | 1 |
| [1E] | Total Tx Power (dBm) | For BS in DL spectrum for indoor - [1E]-R2D-Alt1: 33dBm(M), - [1E]-R2D-Alt2: 38dBm(O), - [1E]-R2D-Alt3: 24dBm(M) - Companies to report if PSD constraints are imposed (company to report the condition for applying PSD constraints in Row [5A]) For UL spectrum for indoor, - [1E]-R2D-Alt4:23dBm (M) - [1E]-R2D-Alt5:26dBm(O) | For device 1/2a: (see note 1) - [1E]-D2R-Alt1: For scenarios ‘B’, the device Tx Power is calculated by CW received power which can be derived by at least CW2D distance (m) value and other related factors. - [1E]-D2R-Alt2: For scenarios ‘A1’ and ‘A2’, the device Tx Power is calculated by assuming CW2D pathloss = D2R pathloss. For device 2b: For scenarios ‘C’ - [1E]-D2R-Alt3: -20 dBm(M) - [1E]-D2R-Alt4: -10 dBm(O) |
| [1E1] | CW Tx power (dBm) | N/A | For scenario ‘A1’, ‘A2’ and ‘B’ - Report a value from the candidate values [1E]-R2D-Alt1 / [1E]-R2D-Alt2 / [1E]-R2D-Alt3 from [1E]-R2D if CW in DL spectrum - Report a value from the candidate values [1E]-R2D-Alt4 / [1E]-R2D-Alt5 from [1E]-R2D if CW in UL spectrum. Note: only applicable for device 1/2a |
| [1E2] | CW Tx antenna gain (dBi) | N/A | Company to report, the value equals: - UE Tx ant gain, or - BS Tx ant gain Note: Only applicable for device 1/2a |
| [1E3] | CW2D distance (m) | N/A | For scenarios ‘B’ D1T1-B: - 5m, - 10m, - 20m - CW2D distance is derived assuming CW node is located with the same position as ‘R1’ in ‘A1’ scenario. (See note 1) D2T2-B: - 5m, - 10m, For scenarios ‘A1’ and ‘A2’: Calculated (see note 1), (i.e., CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss) Note 1: Only applicable for device 1/2a. Note 2: Companies to report which value(s) are evaluated. |
| [1E4] | CW2D pathloss (dB) | N/A | Calculated (see note1) Note 1: Only applicable for device 1/2a Note 2: For CW2D pathloss model, use the same pathloss model as used for R2D/D2R. |
| [1E5] | CW received power (dBm) | N/A | Calculated (see note1) Note: Only applicable for device 1/2a |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | 180kHz(M), 360kHz(O), 1.08MHz(O) | Refer to LLS table [2a1] |
| [1G] | Tx antenna gain (dBi) | For BS for indoor, 6 dBi(M), 2dBi(M) For intermediate UE, 0 dBi | For A-IoT device, 0dBi |
| [1H] | Ambient IoT backscatter loss (dB) due to Modulation factor | N/A | OOK: 6 dB PSK: 0 dB FSK: Y dB It is applicable for device 1 and 2a. Companies to report and justify their assumptions for Y. Companies to report in row 3D if they assume any additional related loss. |
| [1J] | Ambient IoT on-object antenna penalty | N/A | 0.9dB or 4.7dB |
| [1K] | Ambient IoT backscatter amplifier gain (dB) | N/A | 10 dB (M) 15 dB (O) Note: Only for device 2a |
| [1N] | Cable, connector, combiner, body losses, etc. (dB) | For BS, X dB, X <=3 to be reported by companies with justification provided in row 5A For intermediate UE, 1 dB | N/A |
| [1M] | EIRP (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| (2) Receiver |  |  |  |
| [2A] | Number of receive antenna elements / TxRU / chains modelled in LLS | Same as [1D]-D2R | Same as [1D]-R2D |
| [2B] | Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1b] ED bandwidth | Refer to LLS table [2a3] |
| [2C] | Receiver antenna gain (dBi) | same as [1G]-D2R | Same as [1G]-R2D |
| [2X] | Cable, connector, combiner, body losses, etc. (dB) | N/A | Same as [1N]-R2D |
| [2D] | Receiver Noise Figure (dB) | For RF-ED receiver 20dB, Device 2 For IF/ZIF receiver 15dB, Device 2 Other values can be optionally used and reported by companies | For BS as reader: 5dB For intermediate UE as reader: 7dB |
| [2E] | Thermal Noise power spectrum density (dBm/Hz) | -174 | -174 |
| [2F] | Noise Power (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| [2G] | Required SNR/CNR | Reported by companies for Budget-Alt2 | Reported by companies for Budget-Alt2 |
| [2H] | Ambient IoT on-object antenna penalty | 0.9 dB or 4.7 dB | Not applicable |
| [2J] | Budget-Alt1/ Budget-Alt2 | Budget-Alt1/ Budget-Alt2 (see note1) | Budget-Alt2 |
| [2K] | CW cancellation (dB) | N/A | Companies to report for scenario A2/A1/B for BS and intermediate UE. Notes: - Only applicable for device 1/2a - The value provided is for the unmodulated single-tone CW. The value for other cases (e.g. multi-tone CW) can be reported by companies. |
| [2K1] | Remaining CW interference (dBm) | N/A | Calculated (see Note 1) Note: only applicable for device 1/2a |
| [2K2] | Receiver sensitivity loss(dB) | N/A | Calculated (see Note 1) Note: only applicable for device 1/2a |
| [2L] | Receiver Sensitivity (dBm) | For Budget-Alt1 For device 1 (RF-ED), for example: {-30 dBm, -36 dBm, -40 dBm, etc} For device 2 (RF-ED), for example: {-40 dBm, -45 dBm, etc} For Budget-Alt2 Calculated (see note1) | Calculated (see Note 1) Note 2L: the receiver sensitivity includes the receiver sensitivity loss [2K2], i.e. after CW cancellation at least if ‘A2’ scenario is used |
| (3) System margins |  |  |  |
| [3A] | Shadow fading margin (dB) | For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS | For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS |
| [3B] | polarization mismatching loss (dB) | 3 dB | 3 dB |
| [3C] | BS selection/macro-diversity gain (dB) | 0 dB Note: only applicable for D1T1 | 0 dB Note: only applicable for D1T1 |
| [3D] | Other gains (dB) (if any please specify) | Reported by companies with justification | Reported by companies with justification |
| (4) MPL / distance |  |  |  |
| [4A] | MPL (dB) | Calculated (see Note 1) | Calculated (see Note 1) |
| [4B] | Distance (m) | Calculated (see Note 1) | Calculated (see Note 1) |
| （5）Other |  |  |  |
| [5A] | Other notes | Companies to report | Companies to report |

For R2D, [4A] = [1M] + [2C] - [2H] - [2L] - [3A] - [3B] + [3C] + [3D], we can get the [2L] = [1M] + [2C] - [2H] - [3A] - [3B] + [3C] + [3D] - pathloss, reference sensitivity = -25.37dBm. In addition, we need to maintain some margin, such as 3dB. The reference sensitivity can be -29dBm.

Proposal 6:  The reference sensitivity can be -29dBm.

Based on the TS 38.104, the false alarm probability is the conditional total probability of erroneous detection of the preamble when input is only noise. Based on the CFRA, we need to check the device ID. If we just have noise, we cannot check the device ID successfully, thus the false alarm is not reasonable. The missed detection rate is similar to the false alarm rate. We can define success rate(≥ 90%)  as the metric of reference sensitivity can be success rate(≥ 90%).

Proposal 7: The metric of reference sensitivity can be success rate (≥ 90%).

- Maximum input level

The purpose of the definition is to prevent the whole link from being saturated, thus causing the deterioration of SNR and performance. According to the investigation for envelop detector, the dynamic range of envelop detector is about 30-45dB. In order to refer to the worst case, we can assume the dynamic range of envelop detector is 30dB, and the reference sensitivity of device 1 is -34dBm. Therefore, the maximum input level can be -4dBm.

Proposal 8: Using 30dB of dynamic range of device receiver to define maximum input level. The maximum input level can be -4dBm.

Conclusions

In this paper, we provide our views on RF requirements for ambient IoT device, we have made the following proposals.

Proposal 1: Device declares the peak antenna gain direction.

Proposal 2: Define the backscatter loss as follows.

- For OOK, 10 dB @input CW power= -25dBm and 25dB@input CW power= -10 dBm.

- For BPSK, 6 dB @ input CW power= -25dBm and 21dB@input CW power= -10 dBm.

Proposal 3: No need to define SFO requirement explicitly.

Proposal 4: Define flat 12dBc/1SB as SEM requirements.

Proposal 5: Define the maximum 500 kHz or 10 times NB as the boundary A-IoT out of band and general spurious emission domain and the NB (necessity bandwidth) is the CBW. We can assume the maximum backscatter power is -10dBm (the distance between CW and device is 1m).

Proposal 6:  The reference sensitivity can be -29dBm.

Proposal 7: The metric of reference sensitivity can be success rate (≥ 90%).

Proposal 8: Using dynamic range of device receiver to define maximum input level. The maximum input level can be -4dBm.

Reference

[1] R4-2508116, WF on requirements for system parameters and A-IoT device, CMCC, TSG RAN4 meeting #115