3GPP TSG-RAN WG4 Meeting #116	R4-2510120

Bengaluru, India, 25th – 29th August 2025

Source:	Qualcomm Incorporated

Title:	RF specifications for PC2 and PC1.5 for handheld UE, FWA and FWVM devices

Agenda item:	6.5.2

Document for:	Approval

# 1.	Introduction

In RAN#108 a new WID [1] to define RF specifications for handheld UE, FWA and fixed-wireless/vehicle-mounted (FWVM) devices was created. In this paper we present our initial views on this topic.

# 2. 	Discussion

This WID [1] stipulates the creation of RF specifications for handheld UE, FWA and FWVM (fixed-wireless/vehicle mounted) applications in bands n100 and n101 as shown below.

The core objectives of the WI are to develop RF requirements that are applicable to different UE power classes (PC), for different UE devices types (handheld, FWA (Fixed Wireless Access), FWVM (fixed-wireless/vehicle-mounted)) for a single TDD or a single FDD band as described below.

- Objective 1.  For TDD band: PC1.5 and PC2 for handheld UE and FWA, and PC2, PC1.5 and PC1 for FWVM

- Objective 2.  For FDD band: PC2 for handheld UE and FWA, and PC2 and PC1 for FWVM

Note: The bands with the default power class shall be introduced before the high power UE is requested for this band in this present WI.

Redcap UEs are out-of-scope of this WI (there is a separate REL-19 WI NR_PC2_RedCap_UE addressing this case).

ECC decision (20)02[2] addresses the designation of bands n100 and n101 to be used in the future railway mobile communication system (FRMCS). The ECC decision (20)02 details RF specifications for cab-radio (FWVM) devices and railway mobile radio (RMR) terminals other than Cab-radios. Cab-radios are non-portable devices that enable communication between a train and ground stations.

According to the ECC decision (20)02 for devices with output power larger than 23dBm and less than or equal to 31dBm (i.e. PC2 and PC1.5) operating in bands n100 and n101 only cab-radio is allowed. So, though the WID stipulates the creation of specifications for different device types such as handheld UE, FWA and FWVM as their output power is either PC2 or PC1.5 they all must all conform to the cab-radio RF specification.

Observation 1: All PC2 and PC1.5 devices operating in bands n100 and n101 must conform to the cab-radio specification detailed in ECC decision (20)02.

Based on the ECC decision (20)02 the FRMCS minimum ACLR requirement for PC2 and PC1.5 is 37dB. This means that the FRMCS ACLR requirement needs to be 6dB higher than that for the equivalent NR power classes. In NR the ACLR specification for PC1 is 37dBm. We see 2 possible reference designs for creating FRMCS PC2 and PC1.5 Tx RF specifications. The first is to use a NR PC2 PA and to enhance its performance to meet an ACLR of 37dB and the second is to use a PC1 PA. We think that the second option would be easier than the first as we it would be challenging to modify an existing PC2 PA to achieve 6dB larger ACLR. However, RAN4 should further discuss both options before making a final decision. If the second option is adopted, then the MSD values in the existing standard [3] for PC2 and PC1.5 can be re-used for FRMCS. However, if it is deemed that more stringent MPR values are required then a MPR measurement campaign would be needed to generate new MPR numbers for PC2 and PC1.5 for FRMCS.

Observation 2: FRMCS has a 6dB larger ACLR requirement compared to NR for PC2 and PC1.5

Observation 3: NR PC1 has the same ACLR specification as PC2 and PC1.5 for FRMCS

Observation 4: It would be challenging to modify an existing NR PC2 PA to have 6dB larger ACLR

Proposal 1: RAN4 should further discuss whether a reference design with an enhanced PC2 PA or one using a PC1 PA should be used to generate the Tx RF specifications for the PC2 and PC1.5 in bands n100 and n101.

The ACLR specification of 37dB for PC2 and PC1.5 for FRMCS should be captured as a new subsection in the ACLR section 6.5.2.4 of TS38.101-1 [3] so that it is clearly distinguishable from the existing NR handheld UE and FWA ACLR specifications.

Proposal 2: Capture the FRMCS ACLR specification for PC2 and PC1.5 in a new subsection in the ACLR section 6.5.2.4 of TS38.101-1 [3]

# Conclusion

In this paper we present our preliminary thoughts on defining Tx RF specifications for handheld UE and fixed-wireless/vehicle-mounted (FWVM) devices operating in bands n100 and n101 and make the following observations and proposals.

Observation 1: All PC2 and PC1.5 devices operating in bands n100 and n101 must conform to the cab-radio specification detailed in ECC decision (20)02.

Observation 2: FRMCS has a 6dB larger ACLR requirement compared to NR for PC2 and PC1.5

Observation 3: NR PC1 has the same ACLR specification as PC2 and PC1.5 for FRMCS

Observation 4: It would be challenging to modify an existing NR PC2 PA to have 6dB larger ACLR

Proposal 1: RAN4 should further discuss whether a reference design with an enhanced PC2 PA or one using a PC1 PA should be used to generate the Tx RF specifications for the PC2 and PC1.5  in bands n100 and n101.

Proposal 2: Capture the FRMCS ACLR specification for PC2 and PC1.5 in a new subsection in the ACLR section 6.5.2.4 of TS38.101-1 [3]

# References

[1] RP-251402, Revised WID: Rel-19 High power UE (power class 1.5 and 2) for NR FR1 TDD/FDD single band for handheld/FWA UEs, and high-power UE operation (power class2, 1.5 and 1) for FWVM (fixed-wireless/vehicle-mounted) use cases in a single NR band, June 9th – 13th , 2025

[2] Harmonised use of the paired frequency bands 874.4- 880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR), ECC Decision (20)02, 28th June 2024

[3] TS38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V19.1.0