3GPP TSG-RAN WG4 Meeting #116	R4- 2510121

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.28.3

Source:	MediaTek Inc.

Title:	Discussion on RF requirement impact for IoT NTN phase 3

Document for:	Approval

- Introduction

In RAN#104 plenary meeting, the WID of “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” was approved in [1]. One of the main objectives is to enable multiplexing of multiple UEs via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH and to update RF requirements accordingly if needed.

In this contribution, our views on the RF requirements impact are provided for discussion.

- Discussion

2.1		RF requirement impact for IoT-NTN phase 3

Some of the main Rel-19 WID [1] objectives for IoT NTN are shown below for reference.

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account

2.2		Previous Meeting Agreements

In RAN4#115 meeting, the approved WF [2] provided the guidance for following works. The partial content is listed below for reference.

RF core requirements Issue 2-1: Whether UE is required to maintain phase continuity and power consistency for the duration of OCC group. - Agreement: - FFS on UE RF requirements and whether related UE capabilities would be needed. - Take the NR-NTN DMRS bundling requirements as a starting point to evaluate the phase continuity requirements for IoT-NTN with slot level OCC.

In RAN1#120bis meeting, the approved LS [3] provided the information for the following works. The content is listed below for reference.

Title: LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3 1. Overall description RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. - FFS: under which conditions the above applies, e.g., under the same conditions that phase continuity applies for DMRS bundling. RAN1 agreed the following: 2. Actions: RAN1 respectfully ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed.

2.3		Phase continuity and power consistency for OCC

It is expected that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

Observation 1: It is expected that the NTN UE is required to maintain phase continuity and power consistency for the duration of one OCC group.

Proposal 1: If NTN UE supports the OCC feature, NTN UE is required to maintain phase continuity and power consistency for the duration of one OCC group.

Proposal 2: Regarding “Take the NR-NTN DMRS bundling requirements as a starting point to evaluate the phase continuity requirements for IoT-NTN with slot level OCC”, consider whether the following phase continuity requirements can be used for IoT-NTN UE with supporting OCC feature.

====================Start of the change 1==========================

6.4B.2	Transmit modulation quality for Category NB1 and NB2

Transmit modulation quality requirements for Category NB1 and NB2 UEs for BPSK and QPSK modulation as specified in clause 6.5.2F of 36.101 [7] are applicable.

6.4B.2.1	Phase continuity requirements for OCC feature

Table 6.4B.2.1-1: Maximum allowable phase difference for OCC feature

| UL channel | Modulation order | Phase difference between any slot p-1 and slot p |
| --- | --- | --- |
| NPUSCH | BPSK, QPSK | 25 degrees |

The IoT-NTN OCC feature and above requirements can be applicable when all the following conditions are met within the OCC operation time window:

-	RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated.

-	Modulation order does not change.

-	No network commanded TA takes effect.

-	There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE.

-	UE is not scheduled with uplink transmission of other physical channel/signal in-between the NPUSCH transmissions.

-	Doppler and delay conditions are set to constant.

====================End of the change 1==========================

3 Conclusion

In this paper, our views on RF requirements impact for NPUSCH with OCC feature are provided as follows:

Observation 1: It is expected that the NTN UE is required to maintain phase continuity and power consistency for the duration of one OCC group.

Proposal 1: If NTN UE supports the OCC feature, NTN UE is required to maintain phase continuity and power consistency for the duration of one OCC group.

Proposal 2: Regarding “Take the NR-NTN DMRS bundling requirements as a starting point to evaluate the phase continuity requirements for IoT-NTN with slot level OCC”, consider whether the following phase continuity requirements can be used for IoT-NTN UE with supporting OCC feature.

====================Start of the change 1==========================

6.4B.2	Transmit modulation quality for Category NB1 and NB2

Transmit modulation quality requirements for Category NB1 and NB2 UEs for BPSK and QPSK modulation as specified in clause 6.5.2F of 36.101 [7] are applicable.

6.4B.2.1	Phase continuity requirements for OCC feature

Table 6.4B.2.1-1: Maximum allowable phase difference for OCC feature

| UL channel | Modulation order | Phase difference between any slot p-1 and slot p |
| --- | --- | --- |
| NPUSCH | BPSK, QPSK | 25 degrees |

The IoT-NTN OCC feature and above requirements can be applicable when all the following conditions are met within the OCC operation time window:

-	RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated.

-	Modulation order does not change.

-	No network commanded TA takes effect.

-	There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE.

-	UE is not scheduled with uplink transmission of other physical channel/signal in-between the NPUSCH transmissions.

-	Doppler and delay conditions are set to constant.

====================End of the change 1==========================

4 Reference

- RP-241624, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc.

- R4-2508767, “Way Forward for [115][315] IoT_NTN_Ph3”, MediaTek Inc.

- R1-2503071, “LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3”, MediaTek Inc.