3GPP TSG-WG4 Meeting # 116	R4- 2510138

# Bangaluru, India  August 25th – August 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5819 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | maintenance CR for SSB less inter-band collocated CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For a UE supporting scellWithoutSSB-InterBandCA-r18, RAN4 agreed that UE derives coarse timing from the reference cell and fine timing from the SSB less SCell using periodic TRS or A-TRS. However, the same is not reflected for the SCell UL timing requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the UL timing requirements for SSB-less SCells to support fine timing derivation based on TRS/A-TRS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SCell UL timing requirements are not correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

## 7.1	UE transmit timing

### 7.1.1	Introduction

The UE shall have capability to follow the frame timing change of the reference cell in RRC_CONNECTED state or when transmitting PUSCH on CG resources for SDT in RRC_INACTIVE. The uplink frame transmission takes place NTA+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in pTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. For serving cell(s) in sTAG, UE shall use any of the activated SCells as the reference cell for deriving the UE transmit timing for the cells in the sTAG. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

In the requirements of clause 7.1.2, the term reference cell on a carrier frequency subject to CCA is not available at the UE refers to when at least one SSB is configured by gNB, but the first two successive candidate SSB positions for the same SSB index within the discovery burst transmission window are not available during at least one discovery burst transmission window, at the UE due to DL CCA failures at gNB during the last 1280 ms; otherwise the reference cell on the carrier frequency subject to CCA is considered as available at the UE.

For UE supporting rach-EarlyTA-Measurement-r18 for LTM, and if the candidate cell is a neighbour cell or a secondary serving cell without uplink carrier,

-	UE shall have capability to follow the frame timing of the reference cell. The PRACH transmission for early TA acquisition takes place NTA+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. UE shall use this candidate cell as the reference cell for deriving transmit timing. UE initial transmit timing accuracy is defined in the following requirements.

For multi-DCI based multi-TRP operation with two TAs, for each TAG, the uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the downlink reference point.

For multi-DCI based multi-TRP operation with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState TS 38.331 [2] having the same TAG as the uplink signal.

For multi-DCI based multi-TRP operation with two TAs if the UE is not provided SSB-MTC-AdditionalPCI, the reference point for PRACH transmission triggered by PDCCH order, is the first detected path (in time) of one of the downlink reference signal(s) in the DL or joint TCI state of the reference cell associated with the same coresetPoolIndex as PDCCH carrying PDCCH order if PRACH association indicator is 0, and that of the other coresetPoolIndex if PRACH association indicator is 1.

For multi-DCI based multi-TRP operation with two TAs if the UE is provided SSB-MTC-AdditionalPCI, the reference point for PRACH transmission triggered by PDCCH order,

-	if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

-	if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG.

Where NTA and NTA offset are commanded by the network independently for each TAG TS 38.331 [2]. NTA offset for first TAG and second TAG are as defined in n-TimingAdvanceOffset and n-TimingAdvanceOffset2 respectively, if configured, otherwise it is NTA offset defined in table 7.1.2-2.

For a UE supporting scellWithoutSSB-InterBandCA-r18, the reference point for PUCCH/PUSCH/SRS/PRACH of SSB less SCell, is the first detected path (in time) of one of the corresponding TRS/A-TRS from the SSB less SCell.

In the requirements of clause 7.1.2, the SSB applies for both CD-SSB and NCD-SSB if the UE supports ncd-SSB-BWP-Wor-r18.

### 7.1.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in table 7.1.2-1. This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission, or it is the first transmission sent on the PSCell for activating the deactivated SCG without RACH.

-	when it is the transmission for PUSCH on CG resources for SDT in RRC_INACTIVE.

-	when it is the first transmission on target cell after UE receives LTM cell switch command.

When the UL SCS is 120 kHz or smaller, the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. When the UL SCS is 480 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 80 ms. When the UL SCS is 960 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 40 ms.

The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc. The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna. NTA for PRACH is defined as 0.

NTA+NTA offset×Tc (in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA for other channels is not changed until next timing advance is received. The value of NTA offset depends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). NTA offset is defined in table 7.1.2-2.

For multi-DCI based multi-TRP operation with two TAs for first transmission in a DRX cycle for PUCCH, PUSCH, SRS and PRACH transmission triggered by PDCCH order, UE initial transmission timing error requirements specified in this clause is applicable for each TAG and shall be met for each TAG separately. The reference point for each TAG for the PUCCH/PUSCH/SRS and PRACH transmission triggered by PDCCH order is defined in clause 7.1.1.

For a UE supporting scellWithoutSSB-InterBandCA-r18, the reference point for the UE initial transmit timing control requirement of the SSB less SCell shall be the downlink timing of the SSB less SCell cell minus NTA+NTA offset×Tc. The downlink timing of the SSB less SCell is defined as the time when the first path (in time) of the TRS/A-TRS is received from the SSB less SCell at the UE antenna. NTA for PRACH is defined as 0.

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2-1 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| 2-2 | 120 | 120 | 3.5*64*Tc |
|  |  | 480 | 1.58*64*Tc |
|  | 480 | 120 | 2.86*64*Tc |
|  |  | 480 | 1.35*64*Tc |
|  |  | 960 | 0.90*64*Tc |
|  | 960 | 120 | 2.80*64*Tc |
|  |  | 480 | 1.13*64*Tc |
|  |  | 960 | 0.86*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |

Table 7.1.2-2: The Value of NTA offset

| Frequency range and band of cell used for uplink transmission | NTA offset (Unit: TC) |
| --- | --- |
| FR1 FDD or TDD band with neither E-UTRA–NR nor NB-IoT–NR coexistence case | 25600 (Note 1) |
| FR1 FDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case | 0 (Note 1) |
| FR1 TDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case | 39936 (Note 1) |
| FR2 | 13792 |
| Note 1: The UE identifies NTA offset based on the information n-TimingAdvanceOffset as specified in TS 38.331 [2]. If UE is not provided with the information n-TimingAdvanceOffset, the default value of NTA offset is set as 25600 for FR1 band. In case of multiple UL carriers in the same TAG, UE expects that the same value of n-TimingAdvanceOffset is provided for all the UL carriers according to clause 4.2 in TS 38.213 [3] and the value 39936 of NTA offset can also be provided for an FDD serving cell. Note 2: Void |  |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for PUCCH, PUSCH and SRS transmission, the UE shall be capable of changing the transmission timing according to the received downlink frame of the reference cell except when the timing advance in clause 7.3 is applied.

Table 7.1.2-3: void

If the UE uses a reference cell on a carrier frequency subject to CCA for deriving the UE transmit timing, then the UE shall meet all the transmit timing requirements defined in clause 7.1.2 provided that the reference cell is available at the UE. If the reference cell is not available at the UE on a carrier frequency subject to CCA, then the UE is allowed to transmit in the uplink provided that the UE meets all the transmit timing requirements defined in clause 7.1.2; otherwise the UE shall not transmit any uplink signal.

If a reference cell on a carrier frequency belonging to the pTAG, which is subject to CCA, is not available at the UE then the UE is allowed to use any of available activated SCell(s) at the UE in pTAG as a new reference cell. If the SCell used as reference cell is deactivated, or becomes not available, the UE is allowed to use another active serving cell in pTAG as new reference cell.

If a reference cell on a carrier frequency belonging to the sTAG, which is subject to CCA is not available at the UE thenthe UE is allowed to use any of available activated SCell(s) at the UE in sTAG as a new reference cell.

<End of change1>