# 3GPP TSG-RAN WG4 Meeting #116                              R4-2510143

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.20.4.2

Source: 	CMCC

Title: 			 Discussion on other RRM requirements for NR MIMO Phase 5

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR MIMO Phase 5 was approved [1]. In RAN#105 meeting, the revised WID are approved [2], the objectives are duplicated as following.

RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In last meeting, there was discussion on NR MIMO Phase 5, and a WF was agreed [3]. This contribution provides discussion on RRM requirements on enhancements for asymmetric DL sTRP/UL mTRP scenarios.

- Discussion

## 2.1 Enhancements for asymmetric DL sTRP/UL mTRP scenarios

Sub-topic 2-1 Scenario#1 Issue 2-1: For Scenario#1, whether to update RACH requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: FFS Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: - Option 1: Gradual timing adjustment applies for each TAG and shall be met for each TAG. - Option 2: The gradual timing adjustment requirements are only applicable to TRP with both UL and DL. Issue 2-3: For Scenario#1, how to define MRTD and MTTD requirements? Way forward: FFS

Scenario #1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s). According to RAN1 agreements, for scenario #1, one downlink reference timing is supported and applied to both TAGs. The RAN4 agreements on RRM requirements for 2TA enhancement for scenario#1 are summarized as following.

RAN4 #114 (R4-2502596) Agreement: - For scenario 1, for both TAGs, the uplink transmission(s) timing for PUCCH/PUSCH/SRS take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state. - The above agreement can be revisited if not aligned with RAN1 further agreement. RAN4 #114bis (R4-2504903) Agreement: - RAN4 to define timing requirements for FR1 as well as FR2.

It can be seen that the agreements on transmission timing are only for PUCCH/PUSCH/SRS. The requirements for PRACH are not defined. In our view, same agreements can be used for PRACH. In detail, for scenario 1, for both TAGs, the uplink transmission(s) timing for PRACH take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state.

Proposal 1: for scenario 1, it is proposed that, for both TAGs, the uplink transmission(s) timing for PRACH take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state.

As for gradual timing requirements, sine scenario #1 is one DL reference signal with two UL TAGs, our view is that same gradual timing adjustment applies for each TAG and shall be met for each TAG.

Proposal 2: for scenario 1, it is proposed that same gradual timing adjustment applies for each TAG and shall be met for each TAG.

- Conclusion

This contribution provides discussion on RRM requirements on asymmetric DL sTRP/UL mTRP scenarios. The proposals are:

Proposal 1: for scenario 1, it is proposed that, for both TAGs, the uplink transmission(s) timing for PRACH take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state.

Proposal 2: for scenario 1, it is proposed that same gradual timing adjustment applies for each TAG and shall be met for each TAG.

- Reference

- RP-234007, New WID: NR MIMO Phase 5

- RP-242394, WID revision: NR MIMO Phase 5

- R4-2508272, WF on RRM requirements for NR_MIMO_Ph5_Part2, MediaTek

- R4-2504903, WF on RRM requirements for NR_MIMO_Ph5 (Part2), MediaTek

- R4-2502596, WF on RRM requirements for NR_MIMO_Ph5 (Part2), MediaTek

- R4-2502667, LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP

- R1-2503091, Draft Reply to RAN4 LS on DL timing reference to support two TA enhancement for asymmetric DL sTRP/ UL mTRP

- R4-2420108, WF on RRM requirements for NR_MIMO_Ph5 (Part2), MediaTek

- R1-2410870, Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement

- R4-2416921, WF on RRM requirements for NR_MIMO_Ph5 (Part2), MediaTek

- R4-2414035, WF on RRM requirements for NR_MIMO_Ph5, Samsung