# 3GPP TSG-RAN WG4 Meeting #116                              R4-2510145

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.25.3.1

Source: 	CMCC

Title: 			 Discussion on event triggered L1 measurement reporting

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR mobility enhancements Phase 4 was approved [1]. In RAN #107 meeting, the revised WID was approved [2]. The detailed objectives are duplicated as following.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Specify RRM requirements related to the above objectives as necessary [RAN4]

In last meeting, there was discussion on measurements related enhancements for LTM, and a WF was agreed [3]. This contribution provides discussion on event triggered L1 measurement reporting.

- Discussion

- Issue 2-1: applicability of L1-RSRP reporting delay Candidate options: - Option 1: RAN4 considers following conditions in the test for L1-RSRP reporting delay requirements: (vivo, Xiaomi) - No indicate TCI change during the evaluation period. - No configuration change w.r.t measurement RS during the evaluation period - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period - Option 2: Seems no need to capture assumptions listed under option-1 into core requirements. (Apple) - Option 2a: The proposed L1-RSRP reporting assumptions are not captured in core requirements. During performance phase, RAN4 can consider to ensure serving beam kept unchanged (e.g., no indicate TCI change, and no configuration change) for L1 event trigger reporting test cases. (HW)

Firstly, from the perspective of core requirements, we do not think the conditions listed in option 1, e.g. no TCI state change, no configuration change, etc, need to be captured in the applicability of core requirements, since it results in limitation on network implementation. As for whether these conditions are needed in the test, we are open for discussion. According t RAN2 agreements that TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT. And TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam. Following RAN2 agreements, for example, if there is TCI state change during the evaluation period, UE need to perform mesurement on the new current beam, and TTT timer is not restarted if the entering condition is still met with the new current beam. We do not see the technical issue to have such limitation in the test.

Proposal 1: for applicability of L1-RSRP reporting delay, it is proposed not to capture the assumptions (e.g. no indicate TCI change, no configuration change, etc) in the core requirements. As for whether to capture these assumptions in the tests, it can be further discuss.

- Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Define a separate capability for supported number of events triggered L1 measurement reporting. (CTC, Nokia, Xiaomi, CATT, vivo, MTK, ZTE, CMCC, E///) - Details: - Option 1a:(vivo) - RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability. - Reporting criteria for LTM event triggered reporting is 4 per frequency layer. - The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured. - Option 1b: E/// - RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. - RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS - Option 2: update the existing requirements to cover L1 event triggered reporting criteria per measurement category. (OPPO) - Option 3: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple)

For legacy event triggered L3 measurement reporting, the capabilities for support of event triggering reporting criteria are defined (section 9.1.4, TS38.133). Similarly, for Rel-19 event triggered L1 measurement reporting, it is necessary to define the capability. However, according to the discussion in last meeting, some companies mentioned that the number of events to be supported by UE shall be discussed in RAN1 first. We do not have strong view. But at least according to our knowledge, RAN1 have not discussed this. LS to RAN1 can be considered if necessary.

If it is agreed to define capability for supported number of events triggered L1 measurement reporting in RAN4, the related issue is how to define the capabilities for support of event triggering L1 measurement reporting, whether to update L3 capability or to define a seperate capability for L1 measurement reporting. In our view, a seperate capability is prefered. It is not a good way to replace or update legacy capability for event triggered L3 measurement reporting. For Rel-18 LTM measurement, the requirements are applied with the condition that the cell is known, which means that L3 measurement need to be configured. And also, network may adjust LTM configuration based on L3 measurement report.

Proposal 2: If it is agreed to define capability for supported number of events triggered L1 measurement reporting in RAN4, it is proposed to define a separate capability for supported number of events triggered L1 measurement reporting, to avoid the impact on L3 measurement reporting.

- Conclusion

This contribution provides discussion on event triggered L1 measurement reporting. The proposals are:

Proposal 1: for applicability of L1-RSRP reporting delay, it is proposed not to capture the assumptions (e.g. no indicate TCI change, no configuration change, etc) in the core requirements. As for whether to capture these assumptions in the tests, it can be further discuss.

Proposal 2: If it is agreed to define capability for supported number of events triggered L1 measurement reporting in RAN4, it is proposed to define a separate capability for supported number of events triggered L1 measurement reporting, to avoid the impact on L3 measurement reporting.

- Reference

- RP-234036, New WID: NR mobility enhancements Phase 4

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4

- R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2504965, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2414060, WF on R19 Further NR mobility enhancement, Apple

- RP-233970, Revised WID on Further NR Mobility Enhancements

- R2-2404102, Report of 3GPP TSG RAN WG2 meeting #125bis

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4