# 3GPP TSG-RAN WG4 Meeting #116                               R4-2510147

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.25.3.3

Source: 	CMCC

Title: 			 Discussion on conditional intra-CU LTM

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR mobility enhancements Phase 4 was approved [1]. In RAN #107 meeting, the revised WID was approved [2]. The detailed objectives are duplicated as following.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Specify RRM requirements related to the above objectives as necessary [RAN4]

In last meeting, there was discussion on measurements related enhancements for LTM, and a WF was agreed [3]. This contribution provides discussion on conditional intra-CU LTM.

- Discussion

Issue 1-2-1: TRRC < Agreement > - TRRC is the RRC procedure delay. Issue 1-2-2: Tmeasure < Agreement > In C-LTM delay, Tmeasure is measurement time. It is defined from the end of TEvent_DU until UE starts TCLTM-RRC-processing to a target cell. For SSB based L1-RSRP measurement: - Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered, and follow the definition of measurement delay in the event-triggered L1 reporting delay discussed under the corresponding agenda item. For SSB based L3-RSRP measurement: - Reuse Tmeasure in CHO. For CSI-RS based L1-RSRP measurement: - For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period. - Further discuss the conditions for TCSI-RS_SFN_intra = 0. - Further discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS. For CSI-RS based L3-RSRP measurement: - RAN4 does not define CLTM requirements for the case where CSI-RS based L3-RSRP measurements are used in the execution condition evaluation. Issue 1-2-3: TLTM-RRC-processing /TCLTM_execution < Agreement > - TCLTM-RRC-processing is used in C-LTM delay, discuss the description of TCLTM-RRC-processing in draftCR directly. Issue 1-2-4: General on Tfirst-RS and TRS-proc < Agreement > - [Tfirst-RS and TRS-proc] are included in Tinterrupt. Issue 1-2-5: The values and the conditions for Tfirst-RS = 0 and TRS-proc = 0 < Agreement > ad hoc - Tfirst-RS = 0 and TRS-proc = 0 if the target TCI state is on the serving cell or LTM candidate cell active TCI state list or is associated with a TCI state on the active TCI state list. - Note: the limitation of 160/480 ms from Rel-18 still applies. Issue 1-2-6: TLTM-processing < Agreement > - TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. - Reuse Rel-18 LTM values for TLTM-processing in the CLTM cell switch delay. Issue 1-2-7: TLTM-IU < Agreement > - TLTM-IU is the interruption uncertainty during LTM cell switch. - For RACH-based LTM cell switch, TLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. - For RACH-less LTM cell switch, TLTM-IU_is the uncertainty on transmitting the first uplink transmission on the target cell.

In last meeting, there is good progress on conditional intra-CU LTM. Most of the open issues are resolved, one remaining issue is whether[Tfirst-RS and TRS-proc] are included in Tinterrupt.

Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. TRS-proc is the time for SSB processing. In Rel-18 LTM, both are included in Tinterrupt. As for Rel-19 CLTM, some companies propose not to be included. On one hand, we understand the motivation to minimize the interruption since network does not know when the condition is met for C-LTM. On the other hand, it is up to UE implementation whether these procedure or part of procedures (e.g. waiting time of Tfirst-RS is not included in T_interrupt and RS tracking time of Tfirst-RS is included in T_interrupt) will cause interruption.

According to Rel-18 LTM, the value of Tfirst-RS and TRS-proc can be zero under certain conditions, which are duplicated as following. In our view, we can follow similar way as that for Rel-18 LTM, Tfirst-RS andTRS-proc are included in Tinterrupt, and the value is zero under certain conditions.

TS 38.133, Interruption time for LTM (6.3.1.3) Tfirst-RS = 0 and TRS-proc= 0 under the following conditions: - The target TCI state indicated in the LTM cell switch command is in the serving cell active TCI state list, or - The UE is configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and - The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8.25.3 and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than TCI state activation delay stated in section 8.25.3 + 160 ms, or - the measurement period of the SSB associated to target TCI state is not larger than 160 ms after the LTM candidate cell TCI state activation MAC-CE is received, or - The target cell is an FR1 cell, and the UE is not configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and - The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8. 25.3, and not more than TCI state activation delay stated in section 8. 25.3 + [480 ms], or [NOTE: longer L3 measurement delay may be expected for 480 ms after TCI state activation delay stated in section 8.25.3.] - The time between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than [160 ms]. Otherwise, Tfirst-RS is the time to the first SSB transmission on the target cell after Tcmd + TLTM-RRC-processing + TLTM-processing. TRS-proc = 2 ms.

In last meeting, it was agreed that Tfirst-RS =0 and TRS-proc = 0 if the target TCI state is on the serving cell or LTM candidate cell active TCI state list or is associated with a TCI state on the active TCI state list. Based on above consideration, it is proposed that Tfirst-RS and TRS-proc are included in Tinterrupt.

Proposal 1: it is proposed that both Tfirst-RS and TRS-proc are included in Tinterrupt.

- Conclusion

This contribution provides discussion on conditional LTM. The proposals are:

Proposal 1: it is proposed that both Tfirst-RS and TRS-proc are included in Tinterrupt.

- Reference

- RP-234036, New WID: NR mobility enhancements Phase 4

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4

- R4-2508283, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

- R4-2504910, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

- R4-2502602, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

- R4-2420102, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

- R4-2416863, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

- R4-2414060, WF on R19 Further NR mobility enhancement, Apple

- RP-233970, Revised WID on Further NR Mobility Enhancements

- R2-2404102, Report of 3GPP TSG RAN WG2 meeting #125bis

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4