# 3GPP TSG-RAN WG4 Meeting #116                              R4-2510149

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.26.3

Source: 	CMCC

Title: 			 Discussion on RRM core requirements XR (eXtended Reality)

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on XR (eXtended Reality) for NR Phase 3 was approved [1]. According to the WID, one of the objectives having RAN4 impact is to enable transmission/reception in gaps/restrictions that are caused by RRM measurements, the details are duplicated as following.

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In last meeting, there was discussion on XR (eXtended Reality) for NR Phase 3, and a WF was agreed [2]. This contribution provides discussion on this topic.

- Discussion

- Issue 1-1: UE power class <Way forward> - Option 1: The requirements for inter-frequency measurement gap cancellation apply for UE power class 3.

For gap cancellation, we do not see the necessity to limit to UE power class 3. Firstly, even consider other power class except for power class 3, there is no extra RAN4 workload and RRM impact are limited. From perspective of RRM requirements, UE power class has limited impact, only the M value in the delay requierments are related with power class. According to current discussion on measurement delay extention due to gap skipping, M values has no imapct. Secondly, limiting to power class 3 means that network can only configure gap skipping indication to UE power class 3 , which has limitation on network implementation. Based on above consideration, it is not proposed to limit to power class 3.

Proposal 1: for gap cancellation, we do not see the necessity to limit to UE power class 3.

- Issue 2-1: Type 2 gaps <Agreement> - MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules. - The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules. - DCI skipping cannot be applied if any gap is configured with gapAssociationPRS - FFS the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms

It was agreed that MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules, and the timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules. The remaining issue is the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms.

As baseline, X is 5ms, and optional is 3 ms with UE capability. For the case that non colliding concurrent gaps are separated by less than X ms, we believe X is 5ms, since the concurrent gaps are considered as colliding if the distance between the two gap occasions is equal to or smaller than 4ms. For the scenario that concurrent gaps are separated by less than X ms and X is 3ms, it is the colliding case, and MG cancelling DCI applies to the MG surviving the collision handling rules.

For the scenario that concurrent gaps are separated by less than X ms and X is 5ms, there are two cases：

- Case 1 is MG cancelling indication is received before the concurrent gaps.

- If the distance between the indication and the first gap occasion is larger than X ms, the first gap occasion is cancelled, as shown in Figure 1.

- If the distance between the indication and the first gap occasion is smaller than X ms, and the distance between the indication and the second gap occasion is larger than X ms, the second gap occasion is cancelled, as shown in Figure 2.

- If the distance between the indication and the second gap occasion is smaller than X ms, the first gap occasion of next concurrent gap is cancelled, as shown in Figure 3.

- Case 2 is MG cancelling indication is received beween twp gap occasions of the concurrent gaps

- since the distance between two gap occasions are smaller than X ms, the the first gap occasion of next concurrent gap is cancelled, as shown in Figure 4.

In summary, MG cancelling DCI cancels the MG occasion which is the first occasion that satisfy the X ms from the cancelling indication reception.

Figure 1

Figure 2

Figure 3

Figure 4

Proposal 1: for the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms:

- If concurrent gaps are separated by less than X ms and X is 3ms, it is the colliding case, and MG cancelling DCI applies to the MG surviving the collision handling rules.

- If concurrent gaps are separated by less than X ms and X is 5ms, MG cancelling DCI cancels the MG occasion which is the first occasion that satisfy the X ms from the cancelling indication reception.

Issue 3-2: Formulas for measurement delay extension <Way forward>: - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Option 1: Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - Option 2: Max( 200 ms, ( 8 + Nskip ) × Max(MGRP , SMTC period) ) × CSSFinter - Option 3: - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - where Tskip = Nskip ´ Max(MGRP , SMTC period) - Other options are not precluded - where Nskip is the number of skipped MG occasions in the measurement period. - FFS: CSSFinter

It was agreed to follow NR-U-like approach for designing the formulas for measurement delay extension. Following is the duplication of legacy measurement period for NR intra-frequency measurements with CCA. It can be seen that Lmeas,gaps is used to reflect the number of SMTC occasions not available at the UE due to LBT. For XR, measurement gap occasions indicated to be skipped are not available for measurement. It can be seen that the situation are similar between NR-U and XR. The way to define delay requirements for CCA can be reused to calculate measurement delay extension due to MG skipping for XR.

Table 9.2A.6.2-1: Measurement period for intra-frequency measurements with gaps (FR1)

| Condition | TSSB_measurement_period_intra_CCA |
| --- | --- |
| No DRX | max(200 ms, (5+Lmeas,gaps) x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x (5+Lmeas,gaps)) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | (5+Lmeas,gaps) x (MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When DRX is not configured, Lmeas,gaps is the number of SMTC occasions not available at the UE during TSSB_time_index_intra_CCA for measurement where Lmeas,gaps < Lmeas,gaps,max. When DRX is configured, Lmeas,gaps is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TSSB_time_index_intra_CCA for measurement where Lmeas,gaps < Lmeas,gaps,max. When configured with DRX, the UE is not required to determine the availability of SMTC occasions more frequent than once per DRX cycle. When configured with measurement gaps, the UE is not required to determine the availability of SMTC occasions more frequent than once during MGRP. FFS: The UE is not required to determine the availability of SMTC occasions more frequent than what is required by CSSFintra. NOTE 2: Lmeas,gaps,max = 7 for Max(DRX cycle, SMTC period, MGRP)≤40ms where DRX cycle is 0 for non-DRX, Lmeas,gaps,max = 5 for 40ms<Max(DRX cycle, SMTC period, MGRP)≤320ms, Lmeas,gaps,max = 3 for DRX cycle>320ms. NOTE 3: Upon exceeding Lmeas,gaps,max over the TSSB_measurement_period_intra_CCA period of time, the UE has to restart the measurement procedure. |  |

Proposal 2: taking FR1 inter-frequency measurements with gap as an example, for the case of no DRX, and not considering Type 2 gaps, the measurement delay extension due to measurement skipping is proposed as following:

- Max(200 ms, ( 8 + Nskip ) Max(MGRP , SMTC period))  CSSFinter

- Conclusion

This contribution provides discussion on RRM core requirements for XR (eXtended Reality) for NR Phase 3. The observations and proposals are:

Proposal 1: for the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms:

- If concurrent gaps are separated by less than X ms and X is 3ms, it is the colliding case, and MG cancelling DCI applies to the MG surviving the collision handling rules.

- If concurrent gaps are separated by less than X ms and X is 5ms, MG cancelling DCI cancels the MG occasion which is the first occasion that satisfy the X ms from the cancelling indication reception.

Proposal 2: taking FR1 inter-frequency measurements with gap as an example, for the case of no DRX, and not considering Type 2 gaps, the measurement delay extension due to measurement skipping is proposed as following:

- Max(200 ms, ( 8 + Nskip ) Max(MGRP , SMTC period))  CSSFinter

- Reference

- RP-234080, New WID: XR (eXtended Reality) for NR Phase 3

- R4-2508285, WF on RRM requirements for XR_Ph3, Nokia

- R4-2504911, WF on RRM requirements for XR_Ph3, Nokia

- R4-2502700, WF on RRM requirements for XR_Ph3, Nokia

- R4-2420103, WF on RRM requirements for XR_Ph3, Nokia

- R4-2416864, WF on RRM requirements for XR_Ph3, Nokia

- R1-2209536, XR-specific capacity enhancements, Nokia, Nokia Shanghai Bell

- R4-2414043, WF on RRM requirements for XR_Ph3, Nokia

- R1-2405736, LS on UE assistance information

- R1-2407561, LS on gaps/restrictions that are caused by RRM measurements