# 3GPP TSG-RAN WG4 Meeting #116                              R4-2510152

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.21.4.1

Source: 	CMCC

Title: 			 Discussion on RRM requirements for UE-to-UE CLI handling

Document for:	Discussion

- Introduction

In RAN #104 meeting, the revised WID on SBFD was approved [1]. According to the WID, the objectives having RAN4 impact are duplicated as following.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In last meeting, there was discussion on SBFD, and a WF was agreed [2]. This contribution provides further discussion on UE-to-UE CLI handling.

- Discussion

Issue 1-1-1: Requirement applicability - Agreement (Monday online session) - For method #2, 3 and non-basic UE operation of method #1 - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is configured within active DL BWP - when CLI-RSSI measurement resource is configured within active DL BWP - FFS for basic UE operation for method #1 - Note: The assumption for basic UE operation is that UE cannot receive simultaneous PDSCH reception on two DL subbands, and whether the basic UE operation exists or not is pending on RAN1 discussion.

The requirement applicability for Rel-16 L3 CLI measurement is duplicated as following. In general, it is OK to follow the similar requirement applicability for L1 CLI measurement. But some further clarification is needed.

TS 38.133, 9.7 Cross Link Interference measurements CLI measurements are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resource is configured within active BWP

There are three methods. L1-SRS-RSRP (Method #2) is measured within UL subband. It is better to make it clear that the L1-SRS-RSRP is performed within UL sunband. L1-CLI-RSSI can be measured within DL subband (Method #1) or within UL subband (Method #1). It is prefered to make it clear.

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- Method#3: UE measures RSSI within UL subband

- Our initial consideration is that forL1-SRS-RSRP (Method #2), making it more clear that it is measured within UL subband of DL active BWP. However, according to the previous discussion, companies prefer to only mention that SRS-RSRP measurement resource is fully confined within BW of DL active BWP.

- As for method #1, the assumption for basic UE operation is that UE cannot receive simultaneous PDSCH reception on two DL subbands, and whether the basic UE operation exists or not is pending on RAN1 discussion. In our view, from the perspective of applibility, there is no difference from non-basic UE operation of method #1, the measurement is performed within active DL BWP.

Proposal 1: for basic UE operation for method #1, the requirements applibility is proposed as that when L1-CLI-RSSI measurement resource is configured within DL active BWP

Issue 1-2-3: Measurement restriction - Agreement - Take into account following RAN1 agreement in RAN1#121, and further discuss the measurement restriction between Method #1 and Method #2. Agreement A UE does not expect to be configured with L1-CLI-RSSI measurements resources within an UL subband and L1-SRS-RSRP measurement resources on the same symbol..

- RAN1 agreements states that a UE does not expect to be configured with L1-CLI-RSSI measurements resources within an UL subband and L1-SRS-RSRP measurement resources on the same symbol. From perspective of network, it is better to avoid to configure L1-CLI-RSSI measurements resources within an UL subband and L1-SRS-RSRP measurement resources on the same symbol. However, there is no restriction on network configuration. From the point of RAN4 requirements, it is better to clarify UE behaviour when there is collision between L1-CLI-RSSI measurement and L1-SRS-RSRP measurement. In this case, it can be leave to UE implementation.

- Proposal 2: for collision between L1-SRS-RSRP measurements and L1-CLI-RSSI measurements, it is proposed to leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI.

- Conclusion

This contribution provides discussion on UE-to-UE CLI handling. The proposals are:

Proposal 1: for basic UE operation for method #1, the requirements applibility is proposed as that when L1-CLI-RSSI measurement resource is configured within DL active BWP

- Proposal 2: for collision between L1-SRS-RSRP measurements and L1-CLI-RSSI measurements, it is proposed to leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2502699, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, Apple

- R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2416919, LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation

- R1-2410890, Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R4-2502633, Updated summary of simulation results for L1-SRS-RSRP, Huawei, HiSilicon

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2413900, WF on SBFD RRM, Huawei, HiSilicon

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R1-2501620, LS on L1 based UE-to-UE CLI measurement and reporting