# 3GPP TSG-RAN WG4 Meeting #116                             R4-2510154

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.14.3.1

Source: 	CMCC

Title: 			 Discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In RAN #103 meeting, the new WID on RRM enhancements was approved [1]. According to the WID, one of the objectives is FR2-1 SSB based L3 measurement delay reduction for connected mode, the details are duplicated as following.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

There is discussion on FR2-1 SSB based L3 measurement delay reduction for connected mode in last meeting, and a WF was approved [2]. This contribution provides discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor.

- Discussion

There is good progress on the core requirements for L3 measurement delay reduction by optimizing Rx beam sweeping factor. One remaining issue is the UE capability for Rx BSF optimization.

Issue 1-1-3: UE capability for Rx BSF optimization Agreement: For FBS capability design: - UE capability is per-band and optional - TDD only - FR2-1 only - FFS: in the same FG, whether to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency)

In RAN4 #113 meeting, it was agreed that for UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability, and FFS the details of the capability. In RAN4#114bis meeting, it was agreed to introduce the UE capability of reduced N, the values of which are {2, 4 and 6}. And UE chooses one value to report according to its capability. FFS the granularity of the UE capability. In last meeting, it was agreed that the UE capability is per-band. In our view, we do not see the necessity to introduce separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency).

Proposal 1: it is not proposed to introduce separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency).

- Conclusion

This contribution provides discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor. The proposals are:

Proposal 1: it is proposed not to introduce separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency).

- Reference

- RP-240830, New WID: WI resulting from discussion on RRM enhancements

- R4-2508266, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2504900, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2502593, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2416853, WF for NR_RRM_Ph5_Part1, Apple

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5

- R4-2406392, WF for [110bis][230] NR_RRM_Ph5, Apple

- RP-240817, Moderator's summary for discussion on RRM enhancements