# 3GPP TSG-RAN WG4 Meeting #116                             R4-2510156

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.14.4

Source: 	CMCC

Title: 			 Discussion on fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

- Introduction

In RAN #103 meeting, the new WID on RRM enhancements was approved [1]. According to the WID, one of the objectives is fast SCell activation for UE supporting Rel-18 EMR, the details are duplicated as following.

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In last meeting, there was discussion on fast SCell activation for UE supporting Rel-18 EMR, an a WF was agreed [2]. Most of the open issues about core part are solved, this contribution provides discussion on UE capability about fast SCell activation for UE supporting Rel-18 EM.

- Discussion

Issue 2-1-1: UE capability for EMR-based fast SCell activation FFS: - Option 1: Introduce a UE capability for R19 SCell activation based on EMR - Prerequisite feature groups: 39-8 or 39-9 in R18 RAN4 feature list - Need for the gNB to know if the feature is supported: No - Granularity: Per-UE - FDD/TDD differentiation: No - FR1/FR2 differentiation: Yes - Option 2: Not to introduce additional UE capability

According to the WID [1], the objective is to reduce the SCell activation delay with valid EMR reporting. The motivation is that existing known/unknown SCell condition during SCell configuration/activation does not consider whether UE has valid EMR for the target SCell or not, which may cause unnecessary long delay requirement for the target SCell activation.

The SCell activation delay requirements for the case that target SCell is unknown is very long. Taking FR1 as example, if the SCell is known, Tactivation_time is TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms. However, if the FR1 SCell is unknown, Tactivation_time is 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting.

According to the agreements on fast SCell activation for UE supporting Rel-18 EMR [2] [3], with valid EMR reporting, taking unknown FR1 SCell as an example, the activation delay could be reduced to TFirstSSB_MAX + Trs + 5ms if the following conditions are met:

-	During the period equal to 5s for FR1 before the reception of the SCell activation command:

-	the UE has sent a measurement report with SSB index

-	the SSB measured is detectable from the earliest measurement to the end of the defined SCell activation latency (i.e., a valid CQI reporting)

-	If measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 is configured, the earliest measurement refers to the time instance measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 before Msg1 transmission.

-	If only measIdleDuration-r16 is configured, the earliest measurement refers to the time instance when the UE obtained the configuration parameter.

With valid EMR report, the unknown SCell activation delay are reduced provided the conditions are met, and these conditions could ensure that the previous measurement results can be used to reduce the SCell activation delay. It can be seen that UE only use the EMR measurement to reduce activation delay, no additional effort are needed. While for EMR, the related UE capabilities are already introduced in previous releases.

According to the discussion about applicability, fast SCell activation delay requirements apply to following cases:

- UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or,

- UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or,

- neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode,provided that the side condition Ês/Iot ≥ -2dB

The above first two cases are about Rel-18 eEMR (i.e. fast CA/DC setup) , which mainly focus on the validity check in order to report the valid measurement results. Measurement validation and report based on both EMR measurement and reselection measurements are supported, and UE capability measValidationReportEMR-r18 for valid report based on EMR measurement and UE capability measValidationReportReselectionMeasurements-r18 for valid report based on reselection measurements are introduced in Rel-18.The above third case is about Rel-16 EMR, and UE capability idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 are introduced.

Based on above analysis, it can be seen that unknown SCell activation delay is reduced based on the EMR/eEMR measurement, no additional UE effort are needed. And the UE capabilities about EMR/eEMR are already introduced in previous releases.

We do not see the necessity to have new UE capability. For Rel-19 fast Scell activation, fast SCell activation is acheived by using the measurement results for Rel-16/18 EMR. Since UE already have the measurement results, we do not see too much effort are needed to support Rel-19 fast Scell activation based on Rel-16/18 EMR.

Proposal 1: it is not necessary to introduce additional UE capability for fast SCell activation with Rel-18 eEMR report.

- Conclusion

This contribution provides discussion on fast SCell activation for UE supporting Rel-18 EMR. The proposals are:

Proposal 1: it is not necessary to introduce additional UE capability for fast SCell activation with Rel-18 eEMR report.

- Reference

- RP-240830, New WID: WI resulting from discussion on RRM enhancements

- R4-2508266, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2504900, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2502593, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2420094, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2420132, draftCR on Fast SCell activation for UE supporting Rel-18 eEMR

- R4-2416854, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2414041, WF on NR_RRM_Ph5_Part2 - fast SCell activation, CATT