# 3GPP TSG-RAN WG4 Meeting #116                             R4-2510161

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.17.3

Source: 	CMCC

Title: 			 Discussion on CSI reporting requirement and testing framework for CSI prediction

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion core requirements for CSI prediction.

- Discussion

RAN4 agreements to use relative throughput as requirement metric for CSI prediction is for inference. Inference performance is eventully performance, which is impacted not only by the AI/ML model performance but may also be impacted by other factors. While model monitoring targets to evaluate whether a model works well under a centain configuation/scenario. From this point of view, intermediate KPI, e.g. cosine similarity (SGCS), ranging within [0, 1], can be considered as metrics for performance monitoring. RAN1 also agreed that for CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric.

For performance monitoring, RAN1 agreed to support SGCS as a performance metric for performance monitoring type 3 (i.e. UE reports performance metric to NW). In detail, reporting contents of UE assisted performance monitoring includes two SGCS: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

Taking above into account, it is proposed to follow RAN1 agreements to use SCGS as metric for performance monitoring.

RAN1 agreements in RAN1#120bis meeting Agreement For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric. RAN1 agreements in RAN1#121 meeting Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, - one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCS is reported - wideband frequency granularity - only for one prediction instance configured by NW - per layer, and the total number of layers is the same as the reported RI in the associated inference report - Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’ - Each SGCS value is quantized with 4-bit - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale. - One codepoint is used to indicate the value range of (0 0.3] - Only one monitoring resource set is configured in the CSI-ReportConfig - The one configured time instance (i.e. f-th doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1 Agreement For CSI prediction using UE-side model, support single UCI part with the following report format (i.e., CSI field mapping order) among a set of SGCSs. SGCS1 #1. SGCS1 #2. …. SGCS1 #v. SGCS2 #1. SGCS2 #2. …. SGCS2 #v. - SGCS1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI, - SGCS2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCSi #k is the SGCS of k-th layer of i-th SGCS where k ={1, …., v}, and i ={1, 2}. - v is the value of the reported RI in the associated inference report.

Proposal 1: for CSI prediction, it is proposed to follow RAN1 to use SGCS as metric for performance monitoring.

Normally, RAN4 need to define reporting mapping for the reported quantity. SGCS is a newly introduced, and reporting mapping is required. According to RAN1 agreements, the reported SGCS value is quantized with 4-bit. 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale, and one codepoint is used to indicate the value range of (0 0.3]. RAN4 can define the report mapping based on RAN1 agreements.

Proposal 2: it is proposed to define report mapping for SGCS based on RAN1 agreements.

RAN1 agreements in RAN1#117 meeting Agreement For the boundary between Type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively. - The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured. Observation For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified compared to that has been captured in TR38.843, - Type 1 - Definition/configuration of performance metric - Definition of threshold criterion, if configured - Definition/configuration and report of monitoring output, and corresponding report mechanism - Type 2 - Definition/configuration and report of ground truth CSI, and corresponding report mechanism. - Type 3 - Definition/configuration and report of performance metric, and corresponding report mechanism. - For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action RAN1 agreements in RAN1#121 meeting Agreement - For CSI prediction using UE-side model, for UE assisted performance monitoring, - If the inference measurement resource is aperiodic CSI-RS, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of K inference measurement resources that are no later than the CSI reference resource of the associated inference report; - Additional active resource/port counting time for the aperiodic resource of the inference report is from the end of the inference report to the end of the triggered monitoring report - If inference report is aperiodic and the inference measurement resource is periodic or semi-persistent, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of the most recent Kp inference measurement resource transmission occasions that are no later than the CSI reference resource of the associated inference report

It can be seen that UE performance monitoring action is triggered by network indication. If network trigger UE to perform performance monitoring, with Type 3, UE need to report performance metric to network. There is interaction between network and UE. It is proposed to define monitoring delay requirements for this procedure.

Proposal 3: it is proposed to define monitoring delay for CSI prediction performance monitoring.

- Conclusion

This contribution provides discussion on CSI prediction. The proposals are:

Proposal 1: for CSI prediction, it is proposed to follow RAN1 to use SGCS as metric for performance monitoring.

Proposal 2: it is proposed to define report mapping for SGCS based on RAN1 agreements.

Proposal 3: it is proposed to define monitoring delay for CSI prediction performance monitoring.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated

- R4-2505105, WF on requirements for AI/ML air interface normative work, Qualcomm Incorporated

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2502970, Baseline Simulation assumptions for AIML based CSI prediction