3GPP TSG-RAN4 Meeting #116          		R4-2510174

Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on measurement period for L1-SRS-RSRP measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding measurement restriction for the case that measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding measurement restriction for the case that measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.x.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of 1st change >>

#### 9.x.2.5	L1-SRS-RSRP measurement period

The UE shall be capable of performing L1-SRS-RSRP measurement based on the configured SRS resource. For periodic and semi-persistent SRS resource, the UE shall be capable of reporting L1-SRS-RSRP measured over measurement period of TL1_SRS_RSRP_measurement_period for FR1 and FR2.

Table 9.x.2.5-1 Measurement period TL1_SRS_RSRP_measurement_period

| Configuration | TL1_SRS_measurement_period (ms) |
| --- | --- |
| No DRX | Max(TReport, M X TSRS) |
| DRX cycle ≤ 320 ms | Max(TReport, Ceil(1.5 X M) X max(TSRS, TDRX)) |
| DRX cycle > 320 ms | M X TDRX |
| NOTE 1: TSRS is SRS measurement periodicity configured SRS-PeriodicityAndOffset, and TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise. |  |

If the SRS resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TL1_SRS_RSRP_measurement_period.

A UE does not expect to be configured with L1-CLI-RSSI measurements resources within an UL subband and L1-SRS-RSRP measurement resources on the same symbol. When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, it is up to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI.

## << End of 1st change >>