3GPP TSG-RAN WG4 Meeting #116                                                                R4-2510182

# Bengaluru, India, August 25th – 29th, 2025

Source: 	MediaTek Inc.

Title: 	Discussion on NTN Ku band coexistence study

Agenda Item:	7.7.3

Document for:	Approval

Introduction

This document provides Ku band coexistence simulation results focusing on adjacent channel interference for coexistence between NTN and TN based on the NTN coexistence study [1].

Simulation scenarios and assumptions

Simulation scenario

According to WF [2], RAN4 to evaluate the impact on the system performance of TN and NTN for coexistence scenarios shown in Table 1. e.g., For scenario#1, the aggressor is NTN UL (VSAT). The Victim is TN UL from base station (BS). The ACLR of the VSAT is defined based on agreed ACIR. This document presents the simulation observations relating to the coexistence of NTN with TN in terms of ACIR requirement on both DL and UL scope.

WF [2] Agreement - Practical considerations For the purpose of writing the TR, at the time of this 3GPP co-existence study, there is no TN band defined near the simulated frequencies. On the Throughput Performance Use 5% percentile for the first preference and if no data is available then use average throughput loss results. Note: in real satellite networks, the cell size is not well defined, and beams are overlapping Guard band was not considered, and it is not necessary for the co-existence analysis. …etc.

Table 1: Target scenarios for simulation

| Scenarios | Satellite | Aggressor | Victim | Note | ACIR scope |
| --- | --- | --- | --- | --- | --- |
| 1 | GEO or LEO-600 | NTN UL | TN UL | Ku Band VSAT uplink interfering with gNodeB receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the gNodeB is fixed. |
| 4 | GEO or LEO-600 | TN DL | NTN UL | gNodeB downlink interfering with Ku Band Satellite receiver. | The ACLR of the gNodeB is fixed. The ACS of the Satellite to be varied/defined |
| 5 | GEO or LEO-600 | TN DL | NTN DL | gNodeB downlink interfering with Ku Band VSAT receiver. | The ACLR of the gNodeB is fixed. The ACS of the VSAT to be varied/defined. |
| 6 | GEO or LEO-600 | NTN DL | TN DL | Ku Band Satellite downlink interfering with UE receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the UE is fixed. |

We refer to the simulation procedure and assumption [3] as below in details. Results presented are for urban macro environments described in TR38.901, using extended AAS antenna models for TN BS defined in TR38.922.

Ku band coexistence assumption [3] Table 6a.2.2.1-2: Satellite parameters for co-existence study Satellite orbit; GEO; LEO-600. Satellite altitude; 35786 km; 600 km. Satellite antenna pattern; Section 6.4.1 in [2]; Section 6.4.1 in [2]. Payload characteristics for DL transmissions. Equivalent satellite antenna aperture (Note 1); Ku-band (i.e. 11 GHz for DL); 4 m; 0.91 m. Satellite EIRP density; 43 dBW/MHz; 7 dBW/MHz. Satellite Tx max Gain; 43 dBi; 38.5 dBi. 3dB beamwidth; 0.400 deg; 1.768 deg. Satellite beam diameter (Note 2); 250 km; 19 km. Payload characteristics for UL transmissions. Equivalent satellite antenna aperture (Note1); Ku-band (i.e. 14 GHz for UL); 4 m; 0.714 m. G/T; 16.5 dB K-1; 10.9 dB K-1. Satellite RX max Gain; 44 dBi; 38.5 dBi. Table 6a.2.2.2-1: NTN UE antenna characteristics for system level simulations 1; VSAT Antenna Characteristics. 1.1; Antenna pattern; TR 38.921; TR 38.921; Circular Aperture TR 38.863. 1.2; Element gain (dBi) (Note 1); 3.5; 3.5; N/A. 1.3; Y axis / Z axis element 3 dB beam width of single element (degree); 90° for Y axis 90° for Z axis; 90° for Y axis 90° for Z axis; N/A. 1.4; Y axis / Z axis element front-to-back ratio (dB); 30 dB; 30 dB; N/A. 1.5; Antenna polarization (Note 5); Circular (RHCP or LHCP); Circular (RHCP or LHCP); Circular (RHCP or LHCP). 1.6; Antenna array configuration (Y axis × Z axis)(Note 3); 45x45 elements; 78x78 elements; 60 cm diameter aperture. 1.7; Number of supported polarizations, P; 1; 1; 1. 1.8; Y axis / Z axis radiating element spacing; 0.5 lambda; 0.5 lambda; N/A. 1.9; Array Ohmic loss (dB) (Note 1); 2; 2; N/A. 1.10; Conducted power (before Ohmic loss) per antenna element (dBm) (Note 2); 6.9; 9.1; N/A. 1.11; Tx Transmit power; 10 Watts for GEO and for LEO; 50 Watt for GEO; 10 Watts for GEO and for LEO. 1.12; Maximum coverage angle in the horizontal plane (degrees); 0~360 degrees; 0~360 degrees; 0~360 degrees. 1.13; Vertical coverage range (degrees) between beam direction and normal direction; 0~60 degrees; 0~60 degrees; 0~60 degrees. 1.14; Normal direction; Toward X+ axis; Toward X+ axis; Toward X+ axis. 1.15; Max EIRP Value; 76.6 dBm; 88.2 dBm; 76.6 dBm. 1.16; Noise Figure; 6 dB; 6.0 dB for GEO; 2.5 dB for both LEO and GEO. Note 1: The element gain in row 1.2 includes the loss given in row 1.9. Note 2: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 3: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 4: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Note 5: Antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization evaluations are conducted by converting to linear polarization. Table 6a.2.3.2-1: TN BS antenna modelling for Urban macro scenario for 11 GHz Parameters; Values for 11 GHz. Scenario; Urban macro. Antenna pattern; Extended AAS model see below (from TR 38.922). Element gain GE,max (dBi); 6.4. Horizontal 3dB /vertical 3dB beam width of single element; 90º for H 65º for V. Horizontal/vertical front-to-back ratio (dB); 30 for both H/V. Antenna polarization; Linear ±45º. Antenna array configuration (Row × Column) (Note 4); 8x16. Horizontal/Vertical radiating sub-array or element spacing (Note 5); 0.5 of wavelength for H, 2.1 of wavelength for V. Number of element rows in sub-array; 3. Vertical element separation in sub-array (dv,sub); 0.7 of wavelength for V. Pre-set sub-array down-tilt (degrees) (Note 6); 3. Array Ohmic loss LE (dB); 2. Conducted power (before Ohmic loss) per antenna element (dBm); 22. Mechanical downtilt (degrees); 6. Table 6a.2.3.2-3: TN BS antenna modelling for Urban macro scenario for 14 GHz Parameters; Values for 14GHz. Scenario; Urban macro. Antenna pattern; Extended AAS model see below (from TR 38.922). Element gain GE,max (dBi); 6.4. Horizontal 3dB /vertical 3dB beam width of single element; 90º for H 65º for V. Horizontal/vertical front-to-back ratio Am (dB); 30 for both H/V. Side lobe suppression SLAv (dB); 30. Antenna polarization; Linear ±45º. Antenna array configuration (Row × Column); 16 × 24 elements. Horizontal/Vertical radiating element spacing; H = 0.5 λ V = 2.8λ. Number of element rows in sub-array; 4. Vertical element separation in sub-array (dv,sub); 0.7 of wavelength of V. Pre-set sub-array down-tilt (degrees); 3. Array Ohmic loss LE (dB); 2. Conducted power (before Ohmic loss) per antenna element (dBm); 17.14. Base station horizontal coverage range (degrees); +/-60. Base station vertical coverage range (degrees) (Note 1); 90-100. Mechanical down-tilt (degrees); 6. …etc. - 6a.3.1 Simulation procedure See Section 6.3.1 of TR 38.863, noting the FRF and network deployment for NTN and TN in above 10GHz are specified accordingly. Following approaches for the deployment of NTN UE and satellite are used: - The satellite should be generated in the visible area/sky of NTN UE; - NTN UEs point to the satellite accurately; - The position of the satellite should guarantee that NTN UE vertical angle towards the satellite; - For calibration, use 35 degree instead of the range; - SAN elevation angle value, 35, & 90 degrees (computed from the centre of the beam), for co-existence -simulation. - Horizontal angle of NTN UEs should be calculated based on the satellite position; - Use 35m as minimum distance assumed between VSAT and TN BS for co-ex study.

Network layout model

The system level simulation deployment is implemented as Network layout model as below Table 6a.2.1.1-1:

Ku band coexistence assumption [3] Table 6a.2.1.1-1: Network and UE deployment No.; Combination; Aggressor; Victim; Which NTN cell/UE to observe?; Which TN/UE to observe?; Which TN cells in a TN to observe?. 1a; TN with NTN; NTN UL; TN UL; NTN cell: Nadir point. NTN UE: NTN UEs randomly dropped in TN clusters.; TN randomly placed in this NTN beam; Only the active TN clusters which contain NTN UE(s). 4a; TN with NTN; TN DL; NTN UL; NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters; Consider the active rate of 20% for TN.; All active TN cells in central NTN beam. 5a; TN with NTN; TN DL; NTN DL; NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters.; One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam; Only the active TN clusters which contain the NTN UE(s). 6a; TN with NTN; NTN DL; TN DL; NTN cell: Nadir point. NTN UE: NTN UEs randomly dropped in TN clusters.; TN clusters randomly placed in this NTN beam; All in central NTN beam. NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO, interested companies can bring analysis and results for other values) NTN UE: NTN UEs randomly dropped in TN clusters; TN clusters randomly placed in this NTN beam. Table 6a.2.2.3-1: Simulation assumptions of TN NR Carrier frequency in GHz; 11 and 14. Environment; Urban macro. Network layout; hexagonal grid, 19 macro sites, 3 sectors per site with wrap around. Inter-site distance in meter; 450m. UE number per cell; 3, 100% outdoor. Number of scheduled UE per cell (DL); 1. Number of scheduled UE per cell (UL); 1. BS height; 25. UE antenna height in meter; 1.5. UE TX power in dBm; -40 to 23. Cell selection margin in dB; 3. BS-MS min distance in meters; 35. BS-UE path-loss model; TR 38.901. UE distribution; Uniform. Evaluation metrics; See Section 6a.2.8 [3]. Simulation assumption of NTN (satellite and VSAT) Parameters; NTN satellite. Carrier frequency in GHz; 14 GHz (UL) / 11GHz (DL). The number of active UE (UL); 10 UEs and 27RBs per UE for GEO and LEO. The number of active UE (DL); 1. Traffic model; Full buffer. DL power control; NO. UL power control; See Session 6a.2.6.2. NTN satellite Noise Figure in dB; See Table 6a.2.2.1-4.

In this deployment, we assumed there’re 3 TN UEs in each TN sector and total 10 NTN UEs in a TN cluster. The ISD for Urban Macro is 450 meters. NTN UEs randomly placed in the TN cluster as shown in Figure 1 for an example.

Figure 1: An example for urban macro deployment for NTN UEs and TN UEs.

Simulation results

The key performance metric as highlighted in simulation assumption [3] is the throughput loss ratio, thus simulation results in this document will focus on this.

Throughput Loss Ratio vs ACIR

The throughput loss ratio of each scenario is determined across different ACIR values to determine the minimum required ACIR of the victim system to prevent impact from interference.

Table 2 Simulation results of Ku Band Co-existence for 5%-tile Tput loss

| Scenarios | Simulation setting |  | ACIR Requirement @ 5%-tile Throughput Loss Ratio （worst case between 35/90 degree） |
| --- | --- | --- | --- |
| scenario 1 NTN UL aggressor TN UL victim VSAT ACLR to be defined | GEO | Phase array 45*45 | 21.16/17.92 |
|  |  | Phase array 78*78 | 21.45/3.04 |
|  |  | Parabolic | 21.53/15.51 |
|  | LEO-600 | Phase array 45*45 | 0/0 |
|  |  | Phase array 78*78 | 0/0 |
|  |  | Parabolic | 0/0 |
| scenario 4 TN DL aggressor NTN UL victim SAN ACS to be defined | GEO | Phase array 45*45 | 0/0 |
|  |  | Phase array 78*78 | 0/0 |
|  |  | Parabolic | 0/0 |
|  | LEO-600 | Phase array 45*45 | 10.72/14.51 |
|  |  | Phase array 78*78 | 10.72/14.51 |
|  |  | Parabolic | 10.72/14.51 |
| scenario 5 TN DL aggressor NTN DL victim VSAT ACS to be defined | GEO | Phase array 45*45 | 35.92/35.3 |
|  |  | Phase array 78*78 | 31.01/30.07 |
|  |  | Parabolic | 35.53/34.35 |
|  | LEO-600 | Phase array 45*45 | 36.26/35.3 |
|  |  | Phase array 78*78 | 32.44/30.77 |
|  |  | Parabolic | 35.88/34.36 |
| scenario 6 NTN DL aggressor TN DL victim SAN ACLR to be defined | GEO | Phase array 45*45 | 0/0 |
|  |  | Phase array 78*78 | 0/0 |
|  |  | Parabolic | 0/0 |
|  | LEO-600 | Phase array 45*45 | 0/0 |
|  |  | Phase array 78*78 | 0/0 |
|  |  | Parabolic | 0/0 |

Table 3 Simulation results of scenario#5 for average Tput loss

| Scenarios | Simulation setting |  | ACIR Requirement @ average Throughput Loss Ratio （worst case between 35/90 degree） |
| --- | --- | --- | --- |
| scenario 5 TN DL aggressor NTN DL victim VSAT ACS to be defined | GEO | Phase array 45*45 | 29.11/30.66 |
|  |  | Phase array 78*78 | 22.47/19.84 |
|  |  | Parabolic | 32.24/30.74 |
|  | LEO-600 | Phase array 45*45 | 30.73/30.73 |
|  |  | Phase array 78*78 | 24.23/22 |
|  |  | Parabolic | 33.21/30.85 |

Table 2 lists the ACIR requirements for each scenario and various antenna configurations. The ACIR requirement is determined based on the 5%-tile throughput loss which should be less than 5%. All the figures of throughput loss are given in Annex 1.

Proposal 1: Simulation results based on the 5%-tile throughput are provided for scenarios 1, 4, 5, and 6, as listed in Table 2.

Observation 1: Based on the simulation results in Table 2, there is no stringent ACIR requirement for the following scenarios:

- Scenario 1: From NTN UL aggressor to TN UL victim.

- Scenario 4: From TN DL aggressor to NTN UL victim.

- Scenario 6: From NTN DL aggressor to TN DL victim.

Observation 2: Based on the simulation results in Table 2 for the VSAT side:

- ACIR requirement for scenario 1 with a required ACIR of < 22 dB (the worst ACIR is 21.53 dB for scenario 1 GEO with VSAT parabolic at 35° elevation angle).

- Very high ACIR requirement for scenario 5 with required ACIR of < 37 dB (the worst ACIR is 36.26 dB for LEO-600 with VSAT phase array 45*45 at 35° elevation angle)

According to [4], which is the LS for 15 GHz from SI [5] regarding R19 IMT parameters for 15GHz, the UE ACLR value is derived considering that the UE needs to meet the occupied bandwidth (OBW) requirements [6] and is higher than the ACLR needed for coexistence. Therefore, an ACLR of 24 dB is proposed for Ku-band VSATs

Proposal 2: RAN4 to consider ACLR of 24dB for Ku-band VSATs.

Since the ACIR of 5% throughput loss ratio is too stringent for scenario 5, we provided the ACIR of average throughput loss ratio for scenario 5 as listed in Table 3.

Proposal 3: Simulation results based on the average throughput loss ratio are provided for scenario 5, as listed in Table 3.

Observation 3: Based on the simulation results in Table 3:

- The ACIR requirement for scenario 5 with a required ACIR of < 34 dB (the worst ACIR is 33.21 dB with VSAT parabolic and LEO-600 at 35° elevation angle).

Observation 4: ACIR requirements of average throughput loss ratio is also too stringent for scenario 5.

Proposal 4: FFS on how to relax VSAT ACS requirement.

Conclusion

Proposal 1: Simulation results based on the 5%-tile throughput are provided for scenarios 1, 4, 5, and 6, as listed in Table 2.

Observation 1: Based on the simulation results in Table 2, there is no stringent ACIR requirement for the following scenarios:

- Scenario 1: From NTN UL aggressor to TN UL victim.

- Scenario 4: From TN DL aggressor to NTN UL victim.

- Scenario 6: From NTN DL aggressor to TN DL victim.

Observation 2: Based on the simulation results in Table 2 for the VSAT side:

- ACIR requirement for scenario 1 with a required ACIR of < 22 dB (the worst ACIR is 21.53 dB for scenario 1 GEO with VSAT parabolic at 35° elevation angle).

- Very high ACIR requirement for scenario 5 with required ACIR of < 37 dB (the worst ACIR is 36.26 dB for LEO-600 with VSAT phase array 45*45 at 35° elevation angle)

Proposal 2: RAN4 to consider ACLR of 24dB for Ku-band VSATs.

Proposal 3: Simulation results based on the average throughput loss ratio are provided for scenario 5, as listed in Table 3.

Observation 3: Based on the simulation results in Table 3:

- The ACIR requirement for scenario 5 with a required ACIR of < 34 dB (the worst ACIR is 33.21 dB with VSAT parabolic and LEO-600 at 35° elevation angle).

Observation 4: ACIR requirements of average throughput loss ratio is also too stringent for scenario 5.

Proposal 4: FFS on how to relax VSAT ACS requirement.

Reference

[1] 3GPP TR 38.863 Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

[2] R4-2508773, Way Forward for [115][313] NR_NTN_Ku_Band_Coexistene

[3] R4-2508774, Ku band co-existence simulation assumptions (running document)

[4] R4-2420385, LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27

[5] RP-240787, New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz

[6] CFR Title 47, Part 2

Annex

Annex – 1 Throughput Loss Ratio vs ACIR for all scenarios.

scenario#1 (aggressor: NTN UL, victim: TN UL)

Table 3 Throughput loss @ACIR requirement for GEO 90/35 degrees

| VSAT | GEO |  |
| --- | --- | --- |
|  | 90 degrees | 35 degrees |
| Phase array 45*45 |  |  |
| Phase array 78*78 |  |  |
| Parabolic |  |  |

Table 4 Throughput loss @ACIR requirement for LEO-600 90/35 degrees

| VSAT | LEO-600 |  |
| --- | --- | --- |
|  | 90 degrees | 35 degrees |
| Phase array 45*45 |  |  |
| Phase array 78*78 |  |  |
| Parabolic |  |  |

scenario#4 (aggressor: TN DL, victim: NTN UL)

Table 5 Throughput loss @ACIR requirement for GEO 90/35 degrees

| VSAT | GEO |  |
| --- | --- | --- |
|  | 90 degrees | 35 degrees |
| Phase array 45*45 |  |  |
| Phase array 78*78 |  |  |
| Parabolic |  |  |

Table 6 Throughput loss @ACIR requirement for LEO-600 90/35 degrees

| VSAT | LEO-600 |  |
| --- | --- | --- |
|  | 90 degrees | 35 degrees |
| Phase array 45*45 |  |  |
| Phase array 78*78 |  |  |
| Parabolic |  |  |

scenario#5 (aggressor: TN DL, victim: NTN DL)

Table 7 Throughput loss @ACIR requirement for GEO 90/35 degrees

| VSAT | GEO |  |
| --- | --- | --- |
|  | 90 degrees | 35 degrees |
| Phase array 45*45 |  |  |
| Phase array 78*78 |  |  |
| Parabolic |  |  |

Table 8 Throughput loss @ACIR requirement for LEO-600 90/35 degrees

| VSAT | LEO-600 |  |
| --- | --- | --- |
|  | 90 degrees | 35 degrees |
| Phase array 45*45 | a |  |
| Phase array 78*78 |  |  |
| Parabolic |  |  |

scenario#6 (aggressor: NTN DL, victim: TN DL)

Table 9 Throughput loss @ACIR requirement for GEO 90/35 degrees

| VSAT | GEO |  |
| --- | --- | --- |
|  | 90 degrees | 35 degrees |
| Phase array 45*45 |  |  |
| Phase array 78*78 |  |  |
| Parabolic |  |  |

Table 10 Throughput loss @ACIR requirement for LEO-600 90/35 degrees

| VSAT | LEO-600 |  |
| --- | --- | --- |
|  | 90 degrees | 35 degrees |
| Phase array 45*45 |  |  |
| Phase array 78*78 |  |  |
| Parabolic |  |  |