3GPP TSG-RAN WG4 Meeting #116                                   R4-2510183

# Bengaluru, India, August 25th – 29th, 2025

Source: 	MediaTek Inc.

Title: 	NTN HD-FDD VSAT for Ka/Ku bands work scope in Rel-20

Agenda Item:	11

Document for:	Information

- Introduction

In the RAN #108 meeting, the WF on RAN4 R20 package was endorsed [1] as shown below. In this paper, we share our views on the candidate WI for NTN HD-FDD VSAT for Ka/Ku bands.

- NTN HD-FDD VSAT for Ka/Ku bands - RAN4 will start with HD-FDD mobile VSAT with small antennas (e.g 20x20cm). - Once the mobile VSAT with small antennas work is done, - NTN HD-FDD for fixed VSAT and the mobile VSAT with large/normal antenna will be discussed, including how the agreements for the mobile VSAT with small antenna can be leveraged as applicable - RAN1/2 impact is expected and will be further discussed in RAN#109 - Leverage RedCap over NTN HD-FDD solution to NTN as applicable.

- Discussion

Satellite communication technology is essential for the automotive market, especially when integrated with terrestrial networks to ensure stable and seamless connectivity.  Ku/Ka-band satellite systems play a vital role in providing reliable communication, connectivity, and entertainment for vehicles, particularly in areas where terrestrial coverage is limited or unavailable. For mass adoption in the automotive sector, form factor and cost are critical considerations. As a result, compact and lightweight antennas—such as phased-array designs—are preferred for their ability to maintain connectivity while vehicles are in motion.

A key enabler for reducing phased array antenna size is HD-FDD operation, which allows a single antenna to be used for both uplink and downlink, effectively halving the form factor. Since the satellite access network (SAN) remains FDD, network capacity is not compromised, and the EIRP and G/T of the VSAT are broadly similar to those of traditional FDD types.

To further minimize antenna size (e.g., down to 20x20 cm) , link budget considerations become increasingly important and must be carefully studied. For broadband communications, such small antennas are best suited for use with LEO SANs, as GEO SANs typically require larger arrays.

Currently, RAN4 has defined five VSAT types based on FDD, all of which have relatively large antenna sizes (potentially six types by RAN4 #116). HD-FDD operation needs to be studied mostly in terms of RRM, and the requirements for small antenna VSAT (e.g., 20x20 cm) must be addressed in terms of RRM and RF performance. This can be facilitated by defining a new VSAT type with a small antenna, based on HD-FDD and supporting LEO-only operation.

RedCap NTN HD-FDD is currently supported in FR1, but its implementation in FR2 has not yet been addressed in RAN1. Therefore, RAN1 will need to define certain RAN-related aspects to enable HD-FDD operation in FR2. Priority should be given to FR1 numerology in the Ku band, while further studies are required for FR2 numerology in both Ku and Ka bands for NTN HD-FDD VSAT.

Based on these considerations, the objectives of this work item are as follows:

Work Item Objectives for NTN HD-FDD VSAT for Ka/Ku bands

- Specify RF and RRM requirements to support HD-FDD VSATs

- Phase 1: Starting from Ku band with FR1 numerology

  - Introduce requirements for HD-FDD for VSAT type 6

    - VSAT type 6: Mobile VSAT communicating with LEO only with electronic steering antenna.

  - Introduce requirements for HD-FDD VSAT with small antennas (e.g., 20x20cm)

- Phase 2:

  - Introduce NTN HD-FDD for VSAT type 1 to 5 for FR1-numolonogy Ku band

  - Introduce NTN HD-FDD for all defined VSAT types for FR2-numolonogy Ku and Ka-band

Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

- 3	Conclusion

In this contribution, we provide our views on the candidate WI for NTN HD-FDD VSAT for Ka/Ku bands.

- 4	References

- RP-251879, “Way Forward on RAN4 R20 packages” RAN4 Chair (Apple), 3GPP TSG RAN Meeting #108, Prague, June 09-13, 2025.