3GPP TSG-RAN WG4 Meeting #116                                                             R4-2510184

Bengaluru, India, Aug 25th – 29th, 2025

Title:	CSI compression on Low‑Complexity Encoders under SGCS‑4

Source:	Korea Testing Laboratory

Agenda item:			7.18.3

Document for:	Discussion

- Introduction

This contribution discusses Option 3, Track 2, considering lower complexity encoders and follows the guidance outlined in R4-2508084. We can confirm that there are technical viewpoints available, although not all results are presented due to the required computation time.

The objective is to check whether one common test decoder can work effectively with multiple low-complexity UE encoders that utilize different backbones (Transformer, CNN, MLP). The focus is on SGCS convergence rather than absolute performance numbers, in alignment with the RAN4 Way-Forward from Malta. This run corresponds to Option 3, Track 2, where UE vendors train their encoders against a frozen test decoder, with convergence across vendors, backbones, and datasets serving as the main success criterion.

Table 1. WF captured [R4-2508051]

Further simulations (not agreement): In order to further elaborate option 3 track 2 considering lower complexity encoders, interested companies may contribute simulations to RAN4#116. The following guidance, and also the guidance in R4-2508084 is provided and recommended for companies to follow in order to obtained aligned simulations. For option 3, track 2 consider the following investigations for lower complexity encoder structures. - Performance assuming training and testing with the Samsung frozen decoder - Performance achieved with a jointly trained low complexity encoder and decoder - Performance achieved when a frozen decoder is trained based on all of the encoder structures (i.e. different encoder complexity) - Consider transformer, CNN, MLP

2. Simulation

The motivation for this study stems from several key considerations. First, the agreed encoder is impractical due to its extremely high complexity, and different UE vendors may employ encoders with different backbones and varying complexity levels.

For Option 3, Track 2, we consider the following investigations for lower complexity encoder structures: performance assuming training and testing with the Samsung frozen decoder; performance achieved with a jointly trained low-complexity encoder and matching decoder; performance achieved when a frozen decoder is trained based on all encoder structures (i.e., different encoder complexity levels); and consideration of Transformer, CNN, and MLP architectures.

Additionally, verifying whether a common decoder can be compatible with encoders of different backbones and varying complexity levels can provide a more robust feasibility assessment. Finally, we have obtained results from two-side retained models of five companies' given models.

2.1 Decoder (test decoder)

- Structure: fixed to the Samsung “test decoder” structure used in RAN4 studies (the “agreed decoder” in figures).

- Training state per SGCS‑4:

  - Step‑1: the decoder is trained jointly with all encoders (N‑to‑1 joint training on the mixed dataset).

  - Step‑2: the Step‑1 decoder is frozen and used to (re)train/evaluate each encoder separately.

2.1 Encoders

- Encoder‑1 (E1, “given model”). Two vendor baselines are used as anchors in separate Step‑1 runs: Samsung E1 and Vivo E1. These are reused “as is” from prior submissions and then included in the SGCS‑4 procedure.

- Low-complexity encoders implemented per the RAN4 simulation‑assumptions note:

  - Encoder-2(Transformer): 3.63 M FLOPs, model size 1.6 MB; d_model = 64, FF = 256, 8 heads, 4 Transformer blocks.

  - Encoder-3(CNN, ResNet like design): 4.14 M FLOPs, model size 92 KB; 2 ResNet blocks; default conv hyper‑parameters; feature map size (13,32).

  - Encoder-4(MLP): 0.22 M FLOPs, K = 64; simple FC‑based design.

2.2 Training

- Perform training with SGCS-4 manner.

  - Data set: Mixed data set in “MIXEDR4_114dsei00.zip” (50 0000 samples).

    - Training data set = The first 70% of mixed data set (from the 1st to 350 000th sample).

    - Test dataset = The last 10% of mixed data set (from the 450 000th to 500 000th sample).

- Step-1: N-to-1 Joint training (mixed dataset).

  - Inputs: E1 (Samsung or Vivo, depending on the run) + E2 + E3 + E4  single decoder.

  - Update rule per iteration:

    - Compute a composite loss (see below).

    - Update the decoder once using the total loss gradient.

    - Update each encoder once using the back‑propagated gradients from the decoder.

- Step‑2: Separate training with the frozen decoder (mixed dataset).

  - Freeze the Step‑1 decoder.

  - Train each encoder independently (E1, E2, E3, E4) against the frozen decoder.

  - Record the SGCS per encoder on the mixed test set.

Table 2. R4-2508084, “Simulation assumptions on AI/ML CSI compression”

Figure 1 Procedures to derive SGCS-1/2/3/4 of The detailed training procedure of SGCS-4 Step-1 is described as follows. - For each loop, use the Loss to update parameters of the decoder, in addition, - update parameters of Encoder-1 - update parameters of Encoder-2 - update parameters of Encoder-3 - update parameters of Encoder-4 Note: In each loop, decoder parameters are updated for one time based on the total loss, each Encoder (e.g., Encoder-1,2,3,4) is also updated for one time based on the backpropagation results from the decoder. - One of the candidate methods to calculate the Loss: Loss=n1* Loss1+ n2* Loss2, n1+n2=1 Note: n1> n2, using n1=0.7, n2=0.3 as a starting point Loss1=average (lossV,V1+lossV,V2+lossV,V3+lossV,V4) Loss2=average (lossV1, V2+lossV1, V3+lossV1, V4+lossV2, V3+lossV2, V4+lossV3, V4) - Note: Introducing Loss2 is to ensure that comparable SGCSs can be achieved for each encoder. Otherwise, after N-to-1 joint training, SGCS of some encoders (SGCS-4 in Step-1) may be much lower than that of others. - Note: Each lossX,Y can be the reconstruction loss function used in training of SGCS-0/1/2/3, e.g. SGCS loss. - Note: Other methods of calculating the Loss are not precluded. Companies are encouraged to report other methods if they use.

3. Results

- The SGCS values by testing on 2-step trained Encoder 1/23/4

  - It is implemented encoder given by Samsung and VIVO as Encoder 1, respectively

  - For step 1, Epochs 10 training

  - For step2, Epochs 10 training

- Note:  For evaluation focus, in accordance with the Malta Way‑Forward, analysis emphasizes convergence behavior (comparability across encoders/vendors when trained on the mixed dataset) rather than absolute SGCS, and uses Transformer/CNN low‑complexity baselines as reference structures.

Step‑1 and Step‑2 were each trained for 10 epochs, and the observed ranking among low‑complexity encoders (MLP > Transformer > CNN) may reverse with longer training. Hence, endpoint‑only reporting can mislead; contributions should demonstrate learning‑curve stability to rule out budget‑induced ranking artifacts.

In SGCS-4 Step-1, the choice of Encoder-1 as the anchor has a noticeable impact on the compatibility of the other low-complexity encoders. While the original comparison between Samsung E1 and VIVO E1 already showed a bias (e.g., E2: 0.1810 → 0.2569, E3: 0.1434 → 0.2567, E4: 0.5282 → 0.5430), the extended results indicate that this phenomenon exists across all vendors. For example, with Nokia E1 as the anchor, E2/E3/E4 achieve 0.2609/0.2670/0.5204, while MediaTek E1 yields 0.1722/0.1856/0.5600, and CATT E1 yields 0.1643/0.0764/0.5314. The variation range across anchors suggests that anchor-bias is an inherent characteristic of the current training recipe and should be addressed to ensure fairness.

From Table 3 and Table 4, when the encoder and decoder come from the same vendor, the SGCS results tend to be noticeably higher for that vendor’s model, while the other vendor’s model performance drops. For example, with Samsung’s decoder, Samsung E1 achieves 0.5698 while VIVO E1 drops to 0.0289; conversely, with VIVO’s decoder, VIVO E1 improves to 0.2435 while Samsung E1 drops to 0.4501. This suggests that encoder–decoder compatibility plays a significant role in the observed performance, and results may be biased in favor of matched vendor pairs. Future evaluations should consider normalizing for such matching effects to ensure fair cross-vendor comparison.

Table 3. Result for comparison of testing on low complexity encoders (Samsung’s decoder).

| Category | Testing on Encoder 1 (Step-2) | Testing on Encoder 2 (Step-2) | Testing on Encoder 3 (Step-2) | Testing on Encoder 4 (Step-2) | Testing on given two-side model |
| --- | --- | --- | --- | --- | --- |
| VIVO | 0.0289 | 0.1810 | 0.1434 | 0.5282 | 0.0447 |
| Nokia | 0.2286 | 0.2609 | 0.2670 | 0.5204 | 0.4942 |
| Samsung | 0.5698 | 0.2569 | 0.2567 | 0.5430 | 0.5374 |
| MediaTek | 0.2815 | 0.1722 | 0.1856 | 0.5600 | 0.5189 |
| CATT | 0.2325 | 0.1643 | 0.0764 | 0.5314 | 0.5517 |

Table 4. Result for comparison of testing on low complexity encoders (VIVO’s decoder).

| Category | Testing on Encoder 1 (Step-2) | Testing on Encoder 2 (Step-2) | Testing on Encoder 3 (Step-2) | Testing on Encoder 4 (Step-2) | Testing on given two-side model |
| --- | --- | --- | --- | --- | --- |
| VIVO | 0.2435 | 0.2939 | 0.3006 | 0.5043 | 0.0447 |
| Samsung | 0.4501 | 0.2471 | 0.2580 | 0.4570 | 0.5374 |

In SGCS‑4 Step‑1, a single decoder is trained jointly with several encoders, and it receives gradients from all of them at once. If the encoder-agreement term in the loss (often called,  Loss2) is given a very small weight or is omitted, the decoder tends to specialize to the distribution of the current anchor encoder (E1). When that happens, the other low-complexity encoders (E2/E3/E4) inherit a handicap in Step-2: after the decoder is frozen and they are retrained against it, their SGCS depends strongly on which anchor was used in Step‑1. We can see this effect as a large “anchor‑spread,” defined as the maximum minus the minimum SGCS across different anchors.

Raising the weight on the encoder-agreement term (the coefficient n₂ in the composite loss) counteracts that specialization. A higher n2 increases the penalty for discrepancies between encoder outputs, which nudges all encoders toward a shared reconstruction representation. Because the decoder is updated with a loss that now values inter‑encoder consistency, it stops drifting toward any single anchor’s statistics and instead learns features that serve the whole encoder set. In practice this acts like a regularized against anchor bias during the joint Step- training loop.

As the encoder outputs become better aligned, switching the anchor in Step-1 causes much smaller changes in Step-2 performance. In other words, the anchor‑spread shrinks, and the evaluation becomes fairer to each encoder. This is exactly why the document introduces the  Loss2 term: it is there to ensure that comparable SGCS can be achieved for every encoder, rather than allowing some encoders to be favored by the training recipe. Treating anchor-spread as a fairness metric and reducing it by tuning n2 is therefore both principled and consistent with the stated training objective.

Table 5. Anchor-spread = (max -min) on Samsung’s decoder

| Encoder | Max (anchor, value) | Min (anchor, value) | anchor-spread (=max−min) |
| --- | --- | --- | --- |
| E1 | (Samsung, 0.5698) | (VIVO, 0.0289) | 0.5409 |
| E2 | (Nokia, 0.2609) | (CATT, 0.1643) | 0.0966 |
| E3 | (Nokia, 0.2670) | (CATT, 0.0764) | 0.1906 |
| E4 | (MediaTek, 0.5600) | (Nokia, 0.5204) | 0.0396 |

Table 6. Anchor-spread = (max -min) on VIVO’s decoder

| Encoder | Max (anchor, value) | Min (anchor, value) | anchor-spread (=max−min) |
| --- | --- | --- | --- |
| E1 | (Samsung, 0.4501) | (VIVO, 0.2435) | 0.2066 |
| E2 | (VIVO, 0.2939) | (Samsung, 0.2471) | 0.0468 |
| E3 | (VIVO, 0.3006) | (Samsung, 0.2580) | 0.0426 |
| E4 | (VIVO, 0.5043) | (Samsung, 0.4570) | 0.0473 |

As shown in Table 7, the experimental design specifically examines SGCS convergence behavior rather than absolute performance values. A key determinant of system performance is identified as dataset alignment between training and testing phases. Under conditions where mixed-dataset configurations maintain consistent alignment across both training and testing procedures, convergence stability is achieved with deviation parameters confined to approximately ±1.5% to +1.4%. Conversely, when dataset misalignment is introduced between training and testing phases, the system exhibits substantial performance variations with deviation ranges of 30-50%. This disparity demonstrates the critical importance of dataset consistency in maintaining system reliability and ensuring reproducible experimental results.

Table 7. WF on AI/ML part 2, R4-2508051

Simulation result observations Agreements: During the ad-hoc, it was agreed that the moderator should propose some general observation that the study has focussed on SGCS convergence, not absolute SGCS. The following text is provided: The focus in this study has been on convergence between SGCS reported for different company UE encoders when operating with test decoders, in order to understand whether it is feasible to specify a test decoder (or, for option 4, information used to create a test decoder) that can be used for consistent performance testing across multiple UE vendor implementations. Absolute SGCS has not been considered, since the assumed model structure is somewhat arbitrary and the purpose of the study has been to consider testing.

Observation 1: In SGCS-4 Step-1, the choice of the anchor (Encoder-1) materially affects the SGCS of the other encoders.

Observation 2:  With a limited budget (10 epochs in Step‑1 and 10 in Step‑2), the observed ranking is E4 (MLP) > E2 (Transformer) > E3 (CNN), but longer training may reverse it. To avoid over‑crediting lightweight models, report the learning-curves stability for each encoder(SGCS versus epochs), not just endpoints.

Observation 3:  Relative to each vendor’s two‑side baseline, matched encoder–decoder pairs tend to improve while mismatched pairs tend to degrade. However, absolute rankings within a given decoder do not necessarily favor the matched vendor. Therefore, baseline-normalized reporting should be used to ensure fair cross-vendor comparison.

Observation 4: In line with WF [R4-2508051], convergence across encoders is typically within a few percent under aligned training/testing data, whereas misalignment can cause swings on the order of tens of percent

Proposal 1: Considering to require reporting of learning-curve stability (SGCS vs. epochs) for each encoder to rule out budget-induced ranking artifacts, since SGCS depends on the training-epoch budget.

Proposal 2: Discuss to adopt an anchor-fairness check as part of our evaluation framework.

3. Conclusion

In this contribution, we observed from the simulation results regarding Option 3, Track 2, considering lower complexity encoders followed by R4-2508084.

Observation 1: In SGCS-4 Step-1, the choice of the anchor (Encoder-1) materially affects the SGCS of the other encoders.

Observation 2:  With a limited budget (10 epochs in Step‑1 and 10 in Step‑2), the observed ranking is E4 (MLP) > E2 (Transformer) > E3 (CNN), but longer training may reverse it. To avoid over‑crediting lightweight models, report the learning-curves stability for each encoder(SGCS versus epochs), not just endpoints.

Observation 3:  Relative to each vendor’s two‑side baseline, matched encoder–decoder pairs tend to improve while mismatched pairs tend to degrade. However, absolute rankings within a given decoder do not necessarily favor the matched vendor. Therefore, baseline-normalized reporting should be used to ensure fair cross-vendor comparison.

Observation 4: In line with WF [R4-2508051], convergence across encoders is typically within a few percent under aligned training/testing data, whereas misalignment can cause swings on the order of tens of percent

Proposal 1: Considering to require reporting of learning-curve stability (SGCS vs. epochs) for each encoder to rule out budget-induced ranking artifacts, since SGCS depends on the training-epoch budget.

Proposal 2: Discuss to adopt an anchor-fairness check as part of our evaluation framework.

4. Reference

[1] WF on AI/ML part 2, R4-2508051

[2] Simulation assumptions on AI/ML CSI compression, R4-2508084.

5. Contact points

- Kangyu Gao(gaokangyu@hanyang.ac.kr); Zhaokun Shao(shaozhaokun@hanyang.ac.kr);  Sang-Woon Jeon (sangwoonjeon@hanyang.ac.kr)-Hanyang University; Jae-suk Lee(jslee@ktl.re.kr); Hoon-Geun Song(hgsong@ktl.re.kr)- KTL