3GPP TSG-RAN WG4 Meeting #116                                                                R4-2510185

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.7.5

Source:	Sharp

Title:	EIS requirement

Document for:	Discussion

# Introduction

At the last meeting, the following agreement has been reached for EIS requirements [1].

#### Issue 1-3-1: EIS requirement Wayforward: - Consider calculate the EIS requirement for 50MHz channel bandwidth based on the DL SNR target -1 dB (MCS for QPSK 1/3) - At least 2dB implementation margin and will be taken into account. - At least the beam gain reduction around [3.5dB] at the 30~35 degree elevation angle can be taken into account. - Companies are encourage to provide views on the assumed VSAT G/T value for the EIS requirement in the next RAN4 meeting

# Discussion

As agreed at the last meeting, companies are encouraged to provide views on the assumed VSAT G/T value for the EIS requirement. From Sharp’s point of view, the beam gain reduction at the lower elevation angle (as agreed at the last meeting) should be considered for G/T value.

Table 1 shows EIRP for each satellite angle in our measurement setup [2], which yields more than 3.5 dB beam gain reduction at lower satellite angles. We propose 4 dB beam gain reduction for G/T.

Proposal 1: 4 dB beam gain reduction at lower satellite angles should be considered for G/T for VSAT connected only to LEO.

Table 1: Measurement results in Sharp’s measurement setup [2]

| Angle [degree] | 0 | 10 | 20 | 30 | 40 | 50 | 60 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EIRP [dBm] | 67.94 | 68.00 | 67.42 | 67.28 | 66.87 | 65.71 | 64.40 |
| Beam gain reduction [dB] | N/A | -0.06 | 0.52 | 0.66 | 1.07 | 2.23 | 3.54 |

On the other hand, G/T value can be calculated by Eq. (1) as in TR38.811.

G/T=Ga-NF-10*log10T0+Ta-T0/100.1*NF#1

Here, Ga is the antenna gain in dBi, NF is the noise figure, T0 is the ambient temperature (usually 290 K), and Ta is the antenna temperature (typically 290 K with 0 dBi gain and 150 K with >30 dBi gain).

EIS is derived by Eq. (2) below.

EIS=10*log10k+30+10*log10BW+SNR+IM-G/T#2

Here, k is the Boltzmann constant, BW is the channel bandwidth (50 MHz as agreed), SNR is the target SNR for QPSK with coding rate = 1/3 (-1 dB as agreed), and IM is the implementation margin (2 dB as agreed).

Table 2 shows EIS calculation for VSAT connected only to LEO and connected to GEO and LEO, assuming 50 MHz channel bandwidth. Based on our calculations, we propose the following:

Proposal 2: EIS = -121.13 dBm for 50 MHz channel bandwidth for VSAT connected only to LEO.

Proposal 3: EIS = -125.83 dBm for 50 MHz channel bandwidth for VSAT connected to GEO and LEO.

Table 2: EIS calculation for VSAT connected only to LEO and connected to GEO and LEO

|  | VSAT connected only to LEO | VSAT connected to GEO and LEO |
| --- | --- | --- |
| (1) Rx beam gain | 34.3 dBi | 39 dBi |
| (2) Beam gain reduction at lower satellite angles | 4 dB | 4 dB |
| (3) Effective beam gain at lower satellite angles (Ga) | 30.3 dBi | 35 dBi |
| (4) Noise figure (NF) | 6 dB | 6 dB |
| (5) G/T | 0.3 dB/K | 5 dB/K |
| (6) Channel bandwidth | 50 MHz | 50 MHz |
| (7) Effective BW (66*12*0.06) | 47.52 MHz | 47.52 MHz |
| (7) Target SNR | -1 dB | -1 dB |
| (8) IM | 2 dB | 2 dB |
| (9) EIS | -121.13 dB | -125.83 dB |

# Reference

- R4-2508778, Meeting minutes for [115][314] NR_NTN_Ku_Band_UE_SAN_RF, May 2025

- https://global.sharp/corporate/news/240911-a.html