3GPP TSG-RAN WG4 Meeting #116	R4-2510186

Bengaluru, India, 25 August – 29 August, 2025

Source:	vivo

Agenda item:			7.14.3.2

Title:	Discussion on remaining issues on FR 2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised WID on enhancements for NR and MR-DC RRM requirements was approved [1]. The objectives for RAN4 in the WID are as following

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last RAN4 meeting, the topic of FR2-1 SSB based L3 measurement delay reduction for connected model is discussed and the way forwards are captured in [2]. In this contribution, we provided our views on on FR 2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC.

- Discussion

- At RAN4 114bis meeting, the following agreements are available for this WI:

Issue 1-2-1: detailed design for solution 3 (by-default option 3 vs. need network indication for specific SCC)

Agreement on Monday:

  - For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the second searcher sharing.

Issue 1-2-2: applicability limitation for solution 3

Agreement in AdHoc:

  - For UE supporting both per-FR measurement gap and new 3-searcher capability,

    - If network configures per-FR measurement gap, UE will use legacy requirement (i.e., 2-searcher requirement) when at least one SMTC occasion of outside gap measurement objects in one FR are overlapped by per-FR measurement GAP in another FR. Otherwise, the 3-searcher requirement applies.

  - For UE supporting new 3-searcher capability,

    - If network configures per-UE measurement gap and does not configure per-FR measurement gap, the 3-searcher requirement always applies.

Issue 1-3-1: UE capability for CSSF enhancement option 1 (measure one carrier per band)

Agreement in AdHoc:

  - For CSSF enhancement solution 1 in Rel-19, a new optional per-UE capability, apply for FR2-1 only.

Note: the UE capability can be revisited if this feature is expanded to FR1 case.

- In this contribution we discuss the following open issues:

Issue 1-1-1: revision on solution 1

- Option 1 (vivo, OPPO): For solution 1, remove the sub-case and update the agreements as follows:

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

- Option 2 (Ericsson, ZTE, Nokia): Not to remove the phrase ‘if there is no SCCs on which UE is configured to report SSB based measurements in band, or’.

  - (ZTE)NW indication of interested SCC is meaningful even only multiple SCCs without SSB based reporting are configured in the band. Under such case, the NW indication is used to indicate which serving cell to be measured and reported at least.

  - (Nokia): UE shall follow network indication to measure the SCC if there is no SCCs on which UE is configured to report SSB based measurements in the band

- Option 3 (Huawei): Discuss this revised Solution 1 update after the indication design is finalized.

For this issue our understanding is if ther eis no SCCs on which UE is configured to report SSB, the meaning for a network to further indicate which SCC to be measured is limited. It can be argued that network can still indicate interested SCC to be measured even only multiple SCCs without SSB based reporting are configured in the band. However if this interest exists, at the beginning the network will configure this SCC to be measured. Hence option 1 is still preferred

Proposal 1: For revision on solution 1, option 1 is preferred.

Issue 1-1-3: Expand solution 1 to FR1 case

- Option 1 (vivo, Apple, Nokia, CTC, Ericsson (only limit to serving cell measurement), CMCC, ZTE, MTK): Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

  - (Nokia) Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately.

  - (Ericsson): limit the applicability to the serving cell measurements

  - (Nokia) RAN4 to discuss how to handle the neighbor cell measurement when network indicates to enable solution 1 for FR1 bands.

- Option 2 (OPPO, CATT, HW, MTK): Not to expand solution 1 to FR1 band

[Wayforward]: Further discuss in next meeting:

If extend for FR1:

Aspect 1: co-located or non-collocated

- Option 1: only limit to serving cell measurement (E///)

- Option 2: cover both serving cell and neighboring cell measurements

  - In clause 7.6.4 of TS 38.133, it clearly sys only co-located deployment is applied for intra-band contiguous CA.

  - Companies are encouraged to check the spec in this meeting.

Aspect 2: Network configuration on enable/disable the feature

- Option 1: the configuration is the same for different FRs

- Option 2: the configuration can be different for different FRs (Nokia, ZTE, China Telecom, Apple, HW, E///)

  - Under the condition that the UE capability is per FR [or per band].

- Option 2 will be adopted if the solution 1 will be extended to FR1.

Regarding whether to extend solution 1 to FR1 case, our initially understanding is solution 1 is more suitable for FR2 scenario. For FR1, for the intra-band contiguous CA case, the interference experienced by different SCC could be similar to some extent hence option 1 can be used as a compromise.

Proposal 2: If expand to FR1, expand the solution 1 to FR1 intra-band contiguous CA, co-located.

- Summary

In this contribution, we provided our views on FR2-1 SSB based L3 measurement delay reduction for connected mode. Based on above analysis, following observations and proposals are present.

Proposal 1: For revision on solution 1, option 1 is preferred.

Proposal 2: If expand to FR1, expand the solution 1 to FR1 intra-band contiguous CA, co-located.

- References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, RANP 105

- R4-2504899, WF on RRM requirements for NR_RRM_Ph5_Part1, RAN4 114bis, Apple