3GPP TSG-RAN WG4 Meeting #116	R4-2510190

Bengaluru, India, 25 August – 29 August, 2025

Source: 	vivo

Title: 	Discussion on remaining issues for on-demand SSB SCell operation in R19 NES

Agenda Item:	7.23.3.1

Document for:	Discussion

Introduction

In RAN4 #114bis, the WF [1] was agreed and topic summary is provided in [2].

Based on all above information, we provide our views on RRM requirement impacts for on-demand SSB SCell operation in R19 NES.

Discussion

OD-SSB based deactivated SCell L3 measurement procedure (Case 1)

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW

Sub-issue 2: OD-SSB in FMW collided with MG

- Option 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion.

- Option 2: UE to prioritize the OD-SSB carrier/layer within FWM

For the scenario when OD-SSB measurement within FWM collides with MG occasion, since bother items are configured by NW, the collision could be avoided by NW configuration. Under the scenario when they collides, MG should have higher priority.

Proposal 1: The collision could be avoided by NW configuration. If collision happens, UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion.

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure

For issue 1-1-2, the following agreements were achieved at RAN4 115 meeting.

Sub-issue 1: The start point of T2

Agreement:

- The time point of OD-SSB deactivation.

Sub-issue 2: The minimum interval(T2) between two FMWs

Agreement:

- For FR1, 5*max (measCycleSCell, DRX cycles)

- For FR2, 4s for power class 1/5 and 3s for power class 2/3/4

Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2

Agreement:

- The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2.

Sub-issue 4 – UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs

The issue can be further discussed based on contribution driven, and the discussion will not impact the WI completion.

- Option 1: When OD-SSB further activation/reconfiguration occurs within T2, the UE should perform ‘fast measurement’ immediately after T2.

- Option 2: when OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2.

- Other option is not precluded.

For the sub-issue 4, since the OD-SSB is further activated within T2, it could be expected that a fast deactivated SCell activation is strongly preferred hence it could be better to consider option 1.

Proposal 2: When OD-SSB further activation/reconfiguration occurs within T2, it is preferred that the UE should perform ‘fast measurement’ immediately after T2.

Sub-topic 1-2: OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-2-2: Requirement for RAN1 Alt Time-C2

Agreement

- Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity.

- Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if

  - The periodicity of AO-SSB and OD-SSB are same

- It doesn’t impact the WI’s completion.

For this issue, to our understanding the motivation of NW to have this kind of configuration is still not clear. Anyway, the OD-SSB from NW must be sent for a purpose and when it was sent, it is reasonable for UE to measurement OD-SSB hence option 1 is ok.

Proposal 3: For requirement for RAN1 Alt Time-C2, option 1 is ok.

Issue 1-2-3: Requirement for scenario 3 (OD-SSB and AO-SSB are within different frequencies)

Sub issue 1: RAN2 LS for scenario 3

Agreement:

- RAN4 to confirm RAN2’s WS to introduce a new servingCellMO.

Sub issue 2: Requirement for scenario 3

Agreement:

RAN4 will further discuss whether and how to define the requirements for scenario 3, and meanwhile the discussion will not impact the WI completion.

- Proposals

  - Option 1: Qualcomm, CATT, Ericsson, Nokia, China Telecom, LG, CMCC, ZTE, Huawei

    - RAN4 to define the requirement for scenario 3.

  - Option 1a: CATT

    - RAN4 to define the requirement if ‘RAN4 can reach consensus on the understanding of ‘UE is not required to measure both AO-SSB and OD-SSB’.

  - Option 2: vivo, OPPO, Apple

    - RAN4 to not define requirements for scenario 3.

Regarding scenario 3, RAN1 has the following agreements:

RAN1#120 Agreement: Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability Note: UE is not required to measure both AO-SSB and OD-SSB RAN1#121 Agreement: Agreement For a cell supporting on-demand SSB SCell operation, - The following cases are NOT supported in Rel-19. - AO-SSB is CD-SSB and not on a synchronization raster. - OD-SSB is CD-SSB and on sync-raster. - When AO-SSB is not CD-SSB, the center frequency of AO-SSB is different from the center frequency of OD-SSB. - When the center frequency locations of always-on SSB and on-demand SSB are different, OD-SSB is not CD-SSB.

From former RAN1 agreement, for scenario 3 where AO-SSB and OD-SSB on different frequencies, since a UE is not required to measure both AO-SSB and OD-SSB simultaneously, requirements, if to be defined, could be based on either AO-SSB or OD-SSB based on technical aspects, as suggested by a few proposals. In addition, if requirements are based on OD-SSB, the FMW principle introduced for case 1 can be reused. However the potential impact is on the legacy neighbour cell measurement where existing legacy intra-frequency or inter-frequency measurement will be impacted when the measurement is based on the OD-SSB for serving cell and/or deactivated SCell measurement. Considering the impact on the legacy neighbor cell measurement, option 2 is still preferred.

Proposal 4: For requirements for scenario 3, option 2, not define requirements for scenario 3, is still preferred.

Sub issue 3: Intra-frequency definition, if agreed to define, for scenario 3

Sub issue 4: Neighbour cell measurement, if agreed to define, for scenario 3

- Proposals

  - Option 1: Ericsson

    - Intra-frequency is indicated by OD-SSB activation/deactivation if UE supports the scenario 3. Otherwise, the intra-frequency shall be always CD-SSB(AO-SSB) frequency.

  - Option 2: ZTE

    - No impact on the definition of legacy intra/inter-frequency. The OD-SSB based MO is deemed as deactivated SCell MO and the AO-SSB based MO is deemed as intra-frequency MO similar as other active PCell/SCell.

    - When UE detects the configured triggering event with serving cell quality involved in, take the OD-SSB frequency as the serving cell of this deactivated SCell.

  - Option 3: Samsung

    - RAN4 to discuss which SSB can be taken as the baseline to define intra-frequency.

For this issue, our preference is the intra-frequency definition is not impacted. Typically, the intra-frequency neighbour cells is defined for cells that are at the same frequency as that of a CD-SSB on a particular synchronization raster. From latest RAN1 121 agreement, this CD-SSB is also a AO-SSB to our understanding. Then the preference from our side is the neighbor cell which is treated as the intra-frequency cells based on legacy definition, is still treated as intra-frequency cell even when a UE measures OD-SSB for serving cell/deactivated SCell measurement. Follow this method, the impact on legacy neighbor cell measurement, which means a cell may switch between intra-frequency and inter-frequency measurement depending on whether AO-SSB or OD-SSB is measured, can be avoided.

Proposal 5: For neighbour cell measurement, whether a neighbour cell is treated as intra-frequency cell or inter-frequency cell follows its legacy status, i.e., the status when OD-SSB is not configured or activated.

Proposal 6: Whether a neighbour cell is treated as intra-frequency cell or inter-frequency cell is not impacted by whether AO-SSB or OD-SSB is measured.

Sub issue 5: Detail deactivated SCell measurement requirement, if agreed to define, for scenario 3

- Proposals

  - Option 1: Qualcomm, Nokia

    - The measurement requirement is based on different frequencies other than based on the combination of measurements, such as the measurement requirement is defined based on OD-SSB periodicity for FMW.

  - Option 2: Nokia

    - The UE follows legacy requirements based on measCycleSCell if UE has sent a valid measurement report before OD-SSB activation indication within the time window as follow.

      - max(5*measCycleSCell, 5*DRX cycles) for FR1, or

      - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2.

    - Otherwise, the same approach in Case 1 can be reused

  - Option 3: LG

    - RAN4 to discuss how to prioritize OD-SSB measurements, e.g.

      - If the time offset between AO-SSB and OD-SSB is shorter than certain threshold, prioritize OD-SSB measurement.

  - Option 4: CMCC

    - Case 2 deactivated SCell measurement procedure can be reused with clarification that the serving cell measurement shall be based on OD-SSB if OD-SSB is activated and the new servingCellMO is configured.

  - Option 5: ZTE

    - Apply similar strategy for the different frequency case of Case 2 as Case 1,

      - When the OD-SSB is activated, UE enters a FMW and measures the frequency of OD-SSB with the OD-SSB periodicity during the FMW with CSSF = 1.

      - After the FMW, UE fallbacks to slow measurement at the frequency of OD-SSB with MeasCycleSCell. For the frequency of AO-SSB, UE treats it as legacy intra-frequency MO and it is counted in the CSSF.

  - Option 6: Ericsson

    - UE needs to perform measurement without gap in both AO-SSB and OD-SSB frequencies if UE supports the scenario 3.

      - When OD-SSB is activated, UE shall perform intra-frequency measurement in OD-SSB frequency.

      - Before OD-SSB activation or after OD-SSB deactivation, AO-SSB frequency measurement is the intra-freq measurement. UE needs to perform inter-frequency measurement on OD-SSB frequency.

    - Otherwise, UE treats OD-SSB frequency measurement as inter-frequency with/without gap measurement.

  - Option 7: Huawei

    - For Scenario 2: Upon OD-SSB is activated, UE is only required to measure OD-SSB and is not required to measure AO-SSB.

    - For Scenario 2A: Define SCell activation requirements based on OD-SSB. For known conditions, at least for the case when with same QCL assumption, the known conditions can be based on either AO-SSB or OD-SSB.

  - Option 8: Samsung

    - When the two SS blocks (AO-SSB and OD-SSB) are within the UE operating bandwidth, if to perform OD-SSB inter-frequency/intra-frequency measurement, no gaps needed.

    - The AO-SSB and OD-SSB within different frequencies need to be placed in the first active DL BWP.

    - It is not expected that the UE is allowed to perform measurement for the new servingCellMO for OD-SSB in ServingCellConfig when OD-SSB transmission is not activated if multiple serving cell MOs are configured for a given SCell.

    - To enable the UE to prioritize OD-SSB based SCell measurements (e.g., FMW-based fast deactivated SCell measurement) when OD-SSB transmission is activated.

      - Alt.1 UE to measure the serving cell MO for OD-SSB when the UE-specific level OD-SSB signalling is indicated.

      - Alt.2 UE to measure the serving cell MO for OD-SSB when new network signalling of a specific OD-SSB SCC for measurement is indicated.

For the deactivated SCell measurement for scenario 3, firstly we agree with option 1, the requirements could be based on either AO-SSB or OD-SSB, depending on the rules when one of them will be measured, instead of the combination of AO-SSB and OD-SSB.

When OD-SSB is activated, the deactivated SCell measurement should be based on OD-SSB. The FMW and corresponding conclusions introduced for case 1, like lower bound for measurement in FMW, the start point and minimal interval of T2, UE’s behaviour when OD-SSB further activation/reconfiguration within T2 can be reused when measurement in scenario 3 is based on OD-SSB.

Proposal 7: For the deactivated SCell measurement requirements for scenario 3, the requirements could be based on either AO-SSB or OD-SSB, instead of the combination of AO-SSB and OD-SSB.

Proposal 8: For the deactivated SCell measurement requirements, when OD-SSB is activated, the deactivated SCell measurement should be based on OD-SSB. The FMW and other conclusions for OD-SSB measurement introduced for case 1 can be reused.

At RAN4 115 meeting, the known cell condition for case 2 was agreed as below:

Agreement

- Reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4  in FR2.

For scenario 3, although AO-SSB and OD-SSB are at different frequency location which means when OD-SSB is measured, scenario 3 is more like case 1. However if AO-SSB and OD-SSB have the same QCL, the second bullet of option 7, i.e., the known conditions can be based on either AO-SSB or OD-SSB, is ok. And the known conditions agreed before can be reused.

Proposal 9: For the known cell conditions for scenario 3, when AO-SSB and OD-SSB are QCLed, the known cell conditions agreed for case 2 can be reused.

For scenario 2A for scenario 3, SCell activation requirements is based on OD-SSB.

Proposal 10: For scenario 2A for scenario 3, SCell activation requirements is based on OD-SSB.

OD-SSB based SCell activation

Issue 1-3-1: The known cell condition in Case 2

Agreement

- The current known SCell activation delay methodology can be reused in Case 2 at least for Alt Time-C1 with the following combination.

  - When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB.

- FFS whether apply to Alt Time-C2, if the requirement is defined.

- FFS whether apply to scenario 3, if the requirement is defined.

- FFS whether additional AGC time should be included in the known SCell activation delay requirement for scenario 3, if defined.

For Alt Time-C2, if the requirements is defined based on OD-SSB, the former agreement can apply.

Proposal 11: For Alt Time-C2, if the requirements is defined based on OD-SSB, the former agreement can apply.

General issues

Issue 1-4-3: Additional Processing time for OD-SSB activation

Agreement

- RAN4 should introduce UE capability with values of 2 ms and 5 ms.

Issue 1-4-4: Additional Processing time for OD-SSB parameter update

Agreement

- Option 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

  - The additional processing time is 5ms or 2ms based on UE capability reporting.

  - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated].

- Option 2: When NW updates the OD-SSB parameters for an active Scell by MAC-CE, additional 5ms processing time is applied on top of THARQ+3ms after UE receiving the MAC-CE.

  - The additional processing time is 5ms

  - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated].

  - Note 2: No capability is needed

For issue 1-4-4, it is reasonable to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates. In addition 2 ms and 5 ms have already been agreed to be introduced for additional processing time for OD-SSB activation. It is straightforward to reuse this conclusion.

Proposal 11: For Additional Processing time for OD-SSB parameter update, support option 1.

Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline

Agreement

- RAN4 to confirm RAN1’s working assumption about OD-SSB deactivation processing time. Further discuss the wording in the reply LS.

- Further discuss in RAN4 the UE additional processing time for OD-SSB deactivation.

  - Option 1: No UE additional processing time for OD-SSB deactivation

  - Option 2: Follow the same values as UE additional processing time for OD-SSB activation

For the UE additional processing time for OD-SSB deactivation, in general the UE processing procedure is same for OD-SSB activation and deactivation command hence it is ok to consider option 2. On the other hand if the additional processing time is not introduced, whether it causes anything harmful due to unnecessary measurement also depending on whether these unnecessary measurement will be reported or not. If the unnecessary measurements are not reported, the harm is also limited.

Proposal 12: For Additional Processing time for OD-SSB deactivation update, prefer option 2.

Issue 1-4-12: L1 measurement – collision with CSI-RS

- Proposals

  - Option 1: Huawei

    - RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB.

- Recommended WF

  - Further discussion.

For the OD-SSB collides with CSI-RS, the scenario is slightly different compared with the scenario where AO-SSB collides with CSI-RS since OD-SSB is UE specific and it may more important from UE perspective. Hence it is preferred that the OD-SSB is measured when OD-SSB collides with CSI-RS.

Proposal 13: The OD-SSB is measured when OD-SSB collides with CSI-RS.

Issue 1-4-14: OD-SSB based L3 reporting

- Proposals

  - Option 1: Huawei, vivo

    - RAN4 to send LS to RAN2 to support L3 report triggered by OD-SSB activation based on FMW.

- Recommended WF

  - Deprioritize the issue.

    - Note: From moderator’s understanding, this issue will be treated in RAN2.

Regarding the OD-SSB based L3 reporting, the intention to configure OD-SSB, besides NW energy saving perspective, another intention is to enable a quick report for NW through the FMW window hence supporting L3 report triggered by OD-SSB activation is reasonable.

Proposal 14: For OD-SSB based L3 reporting , support L3 report triggered by OD-SSB activation.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1: The collision could be avoided by NW configuration. If collision happens, UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion.

Proposal 2: When OD-SSB further activation/reconfiguration occurs within T2, it is preferred that the UE should perform ‘fast measurement’ immediately after T2.

Proposal 3: For requirement for RAN1 Alt Time-C2, option 1 is ok.

Proposal 4: For requirements for scenario 3, option 2, not define requirements for scenario 3, is still preferred.

Proposal 5: For neighbour cell measurement, whether a neighbour cell is treated as intra-frequency cell or inter-frequency cell follows its legacy status, i.e., the status when OD-SSB is not configured or activated.

Proposal 6: Whether a neighbour cell is treated as intra-frequency cell or inter-frequency cell is not impacted by whether AO-SSB or OD-SSB is measured.

Proposal 7: For the deactivated SCell measurement requirements for scenario 3, the requirements could be based on either AO-SSB or OD-SSB, instead of the combination of AO-SSB and OD-SSB.

Proposal 8: For the deactivated SCell measurement requirements, when OD-SSB is activated, the deactivated SCell measurement should be based on OD-SSB. The FMW and other conclusions for OD-SSB measurement introduced for case 1 can be reused.

Proposal 9: For the known cell conditions for scenario 3, when AO-SSB and OD-SSB are QCLed, the known cell conditions agreed for case 2 can be reused.

Proposal 10: For scenario 2A for scenario 3, SCell activation requirements is based on OD-SSB.

Proposal 11: For Additional Processing time for OD-SSB parameter update, support option 1.

Proposal 12: For Additional Processing time for OD-SSB deactivation update, prefer option 2.

Proposal 13: The OD-SSB is measured when OD-SSB collides with CSI-RS.

Proposal 14: For OD-SSB based L3 reporting , support L3 report triggered by OD-SSB activation.

- References

[1] R4-2508277, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4 115

[2] R4-2505570, Topic summary for [115][221] Netw_Energy_NR_enh_Part1, Ericsson, RAN4 115