3GPP TSG-RAN WG4 Meeting #116	R4-2510194

Bengaluru, India, 25 August – 29 August, 2025

Source: 	vivo

Title: 	Further discussion on remaining RRM issues for LP-WUR

Agenda Item:	7.24.5.2

Document for:	Discussion

Introduction

In RAN Plenary #102 meeting, WI for “Low-power wake-up signal and receiver for NR” was agreed [1]. The WI was further updated at RAN Plenary #103 meeting as [2] and RAN #105 [3].

The objectives of WI are duplicated as below:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

  - Specify necessary RRM requirements

The initial discussion for this topic was initially discussed at RAN4 110bis meeting. The WF on this WI of RAN4 114bis meeting and corresponding agreements can be found at [4]. The topic summary of RAN4 114bis can be found at [5].

Discussion

General aspects and working with other features

Issue 1-1-15 For RedCap UE

- Proposals

  - P1: RedCap UE shall meet LP-WUS related requirement based on UE’s capability (Ericsson)

Recommendations:

For the Redcap, except for eDRX related requirements, we did not find any other issues where the LP-WUR requirement in Rel-19 applying for 2 Rx Redcap. Logically, the major difference between Redcap UE and normal UE is bandwidth, number of antenna etc. and the LP-WUR idle mode requirements, like serving cell offloading and RRM relaxation, should not be impacted by these factors and can be used to 2 Rx Redcap UE. For 1Rx Redcap UE, the following special requirements are made for 1 Rx Redcap UE. Actually all difference for 1 Rx Redcap are requirements for MR and can be used directly.

4.2B.1	Introduction

The 1 Rx RedCap UE for performing the cell reselection procedure [1] applies:

-	Qrxlevmin as the signaled value of Qrxlevmin [2] -1 dB.

-	Qqualmin as the signaled value of Qqualmin [2] -1 dB.

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap

For idle mode cell re-selection purposes, and for UE supporting IdleInactiveMeasurements-r16 or idleInactiveEUTRA-MeasReport-r16, the UE shall be capable of monitoring at least:

-	Intra-frequency carrier, and

-	Depending on UE capability, 6 NR inter-frequency carriers, and

-	Depending on UE capability, 6 FDD E-UTRA inter-RAT carriers, and

-	Depending on UE capability, 6 TDD E-UTRA inter-RAT carriers.

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE

when rangeToBestCell is not configured:

-	the cell is at least 3 dB better ranked in FR1 or 4.5 dB better ranked in FR2 for 2 Rx RedCap.

-	the cell is at least 4 dB better ranked in FR1 for 1 Rx RedCap.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS 38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them.

-	the cell is at least 3 dB better ranked in FR1 or 4.5 dB better ranked in FR2 if the current serving cell is among them for 2 Rx RedCap.

-	the cell is at least 4 dB better ranked in FR1 if the current serving cell is among them for 1 Rx RedCap.

The 1 Rx RedCap UE applies absThreshSS-BlocksConsolidation as the signalled value of absThreshSS-BlocksConsolidation [2] + 1 dB.

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE

For 1 Rx RedCap by a margin of at least

6 dB in FR1 for reselections based on ranking or

7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

-	the condition when performing equal priority reselection and

-	when rangeToBestCell is not configured:

-	the cell is at least 5 dB better ranked in FR1 or 6.5 dB better ranked in FR2 for 2 Rx RedCap.

-	the cell is at least 6 dB better ranked in FR1 for 1 Rx RedCap.

-	when rangeToBestCell is configured:

-	the cell is at least 6 dB better ranked in FR1 if the current serving cell is among them, or 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities for 1 Rx RedCap.

The 1 Rx RedCap UE applies absThreshSS-BlocksConsolidation as the signalled value of absThreshSS-BlocksConsolidation [2] + 1 dB.

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE

The UE shall be able to evaluate whether a newly detectable inter-RAT E-UTRAN cell meets the reselection criteria defined in TS 38.304 [1] within (NEUTRA_carrier_RedCap) * Tdetect,EUTRAN_RedCap when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria are met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities for 2 Rx RedCap and at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities for 1 Rx RedCap.

For a cell that has been already detected, but has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified inter-RAT E-UTRA cell has met reselection criterion defined in TS 38.304 [1] within (NEUTRA_carrier_RedCap) * Tevaluate,EUTRAN_RedCap when Treselection = 0 provided that the reselection criteria are met by a margin of at least 6 dB for RSRP reselections based on absolute priorities or 4 dB for RSRQ reselections based on absolute priorities for 2 Rx RedCap and at least 6 dB for RSRP reselections based on absolute priorities or 4 dB for RSRQ reselections based on absolute priorities for 1 Rx RedCap.

The only technical issue we identified is related to MR wake up delay. At previous meeting we have the following agreement:

Issue 1-2-6-1: On the number of SSBs (for RAN1 LS R1-2501624)

Agreement:

For the 400ms and 800ms ramping up time: {5, 5} samples

For the 10ms ramping up time: 3 samples

For the wake up time for 1Rx Redcap UE, further study may need. Initially the same number of SSB can be reused for 1 Rx Redcap UE and further update is possible if any issue is identified.

- Specify serving cell and neighbor cell measurement for offloading and RRM relaxation for Redcap UE with 1 Rx and 2 Rx. Existing agreements for offloading and RRM relaxation will be used for Redcap UE.

- For the number of SSB for wake up delay for 1 Rx Redcap, the same number of SSB, i.e., { 3 5 5} SSBs for {10ms, 400ms, 800ms} ramping up time, can be reused initially.

Issue 1-5-1: LP-WUR works with EMR in IDLE/inactive mode

- Proposals

  - P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple Nokia)

    - P1-1: UE shall not enter relaxation / offloading if configured simultaneously with EMR. (Nokia)

  - P2: Postpone the discussion on the EMR issue (vivo)

  - P3: RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact (Ericsson Nokia)

For the relationship between EMR and LP-WUR operation, our understanding is the applicable scenario for these two features are unlikely to happen at the same time. When more measurements are preferred on carriers in order to have fast activation after entering the connect state, under this scenario the power saving may not be a crucial factor to be considered.

In Rel-16, for the not at the cell edge criteria, the followings are used to handle the operation of the EMR feature and UE power saving feature:

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion

The UE shall not relax measurements on NR inter-frequency carriers configured for idle mode CA/DC measurements (defined in clause 4.4) while T331 is running.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2.2.4 apply for this clause except that:

-	The UE shall be able to evaluate whether a newly detectable inter-frequency NR cell meets the reselection criteria defined in TS 38.304 [1] within Ncarrier_Relax * Tdetect,NR_Inter_Relax + Ncarrier_Non_relax  * Tdetect,NR_Inter. Cells which have been detected shall be measured at least every Ncarrier_Relax * Tmeasure,NR_Inter_Relax + Ncarrier_Non_relax  * Tmeasure,NR_Inter. The UE shall be able to evaluate that an already identified inter-frequency NR cell has met reselection criterion defined in TS 38.304 [1] within Ncarrier_Relax * Tevaluate,NR_Inter_Relax + Ncarrier_Non_relax  * Tevaluate,NR_Inter.

-	When T331 is running,

-	The parameter Ncarrier_Relax is the total number of NR inter-frequency carriers not configured for idle mode CA/DC measurements.

-	The parameter Ncarrier_Non_relax is the total number of NR inter-frequency carriers configured for idle mode CA/DC measurements.

-	When T331 is not running,

-	The parameter Ncarrier_Relax is the total number of inter-frequency carriers configured for mobility measurements only and the number of inter-frequency carriers configured for both mobility measurement and idle mode CA/DC measurements.

To our understanding differentiating carriers into CA/DC carriers (without any relaxation) and non CA/DC carriers (with relaxation) do not bring any power consumption gain or very little power consumption gain. In Rel-19 the procedure could be simplified as below:

- When both Rel-16 EMR and Rel-19 LP-WUR are configured, for case 1 (offloading) and case 3 (RRM relaxation) defined in Rel-19 LP-WUR:

  - when T331 is running, all carriers (carriers configured/not configured for CA/DC idle state measurement) follow (Rel-15) legacy measurements; when T331 is not running, all carriers follow RRM requirements defined in Rel-19 LP-WUR, i.e., relaxed with scaling factor 16 when case 3 (RRM relaxation criteria) are satisfied or serving cell offloading when case 1 criteria are satisfied;

For Rel-18 EMR, since it is not linked with T331, the following options could be considered:

- Option 1: When Rel-18 EMR is configured, all carriers follow (Rel-15) legacy measurement requirement

- Option 2: Rel-18 EMR and Rel-19 LP-WUR cannot be configured at the same time (need send LS to RAN2)

- When both Rel-16 EMR and Rel-19 LP-WUR are configured, for case 1 (offloading) and case 3 (RRM relaxation) defined in Rel-19 LP-WUR:

- when T331 is running, all carriers (carriers configured/not configured for CA/DC idle state measurement) follow (Rel-15) legacy measurements; when T331 is not running, all carriers follow RRM requirements defined in Rel-19 LP-WUR, i.e., relaxed with scaling factor 16 when case 3 (RRM relaxation criteria) are satisfied or serving cell offloading when case 1 criteria are satisfied;

- When both Rel-18 EMR and Rel-19 LP-WUR are configured, consider the following options:

Option 1: When Rel-18 EMR is configured, all carriers follow (Rel-15) legacy measurement requirement.

Option 2: Rel-18 EMR and Rel-19 LP-WUR cannot be configured at the same time (need send LS to RAN2)

Issue 1-5-2: eDRX related

- Proposals

  - P1: RAN4 to discuss followings LP-SS based RRM issue in IDLE/Inactive mode: how to enter and exit offloading status if eDRX is configured with PTW. (Apple)

  - P2: Discuss whether RAN4 needs to introduce RRM requirements for LP-WUR operation with eDRX. (Qualcomm)

Recommendations:

For both Redcap UE and normal UE, requirements are defined for RRM relaxation on top of eDRX operation, for example the following requirements are defined for NR UE with RRM relaxation in 4.2.2.10.3 Measurements for UE fulfilling not-at-cell edge criterion

Table 4.2.2.10.3-2: Tdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR1) for eDRX_IDLE cycle up to 10.24 s

| eDRX_IDLE cycle length [s] | Tdetect,NR_Inter_Relax [s] (number of eDRX IDLE cycles) | Tmeasure,NR_Inter_Relax [s] (number of eDRX IDLE cycles) | Tevaluate,NR_Inter_Relax [s] (number of eDRX IDLE cycles) |
| --- | --- | --- | --- |
| 2.56 | 58.88 x K1 (23 x K1) | 2.56 x K1 (1 x K1) | 7.68 x K1 (3 x K1) |
| 5.12 | 117.76 x K1 (23 x K1) | 5.12 x K1 (1 x K1) | 10.24 x K1 (2 x K1) |
| 10.24 | 235.52 x K1 (23 x K1) | 10.24 x K1 (1 x K1) | 20.48 x K1 (2 x K1) |
| NOTE 1: Void. NOTE 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |

Table 4.2.2.10.3-3: Tdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR2) for eDRX_IDLE cycle up to 10.24 s

| eDRX_IDLE cycle length [s] | Tdetect,NR_Inter_Relax [s] (number of eDRX IDLE cycles) | Tmeasure,NR_Inter_Relax [s] (number of eDRX IDLE cycles) | Tevaluate,NR_Inter_Relax [s] (number of eDRX IDLE cycles) |
| --- | --- | --- | --- |
| 2.56 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| 5.12 | 117.76 x N1 x K1 (23 x N1 x K1) | 5.12 x N1 x K1 (1 x N1 x K1) | 10.24 x N1 x K1 (2 x N1 x K1) |
| 10.24 | 235.52 x N1 x K1 (23 x N1 x K1) | 10.24 x N1 x K1 (1 x N1 x K1) | 20.48 x N1 x K1 (2 x N1 x K1) |
| NOTE 1: For UE supporting FR2-1 power class 2&3&4, N1=3. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. NOTE 2: Void. NOTE 3: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |

Table 4.2.2.10.3-4: Tdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax for UE configured with eDRX_IDLE cycle greater than 10.24 s(Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28 s periods) | Tdetect,NR_Inter_Relax [s] (number of DRX cycles Note 3) | Tmeasure,NR_Inter_Relax [s] (number of DRX cycles Note 3) | Tevaluate,NR_Inter_Relax [s] (number of DRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤[163.84] | 0.32 | ≥3.84 (3) | eDRX_cycle_length×23PTW/DRX_cycle_lengthx K1 (23 x K1) | 0.32 x M2 x K1 (1 x M2 x K1) | 0.64 x M2 x K1 (2 x M2 x K1) |
|  | 0.64 | ≥3.84 (3) |  | 0.64 x K1 (1 x K1) | 1.28 x K1 (2 x K1) |
|  | 1.28 | ≥7.68 (6) |  | 1.28 x K1 (1 x K1) | 2.56 x K1 (2 x K1) |
|  | 2.56 | ≥15.36 (12) |  | 2.56 x K1 (1 x K1) | 5.12 x K1 (2 x K1) |
| NOTE 1: The number of DRX cycles in this table correspond to the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in section 10.5.5.32 of TS 24.008 [42]. NOTE 3: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_Relax*DRX_cycle1.28*1.28. NOTE 4: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. NOTE 5: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |

Table 4.2.2.10.3-5: Tdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax for UE configured with eDRX_IDLE cycle greater than 10.24 s (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28 s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Inter_Relax [s] (number of DRX cycles Note 3) | Tmeasure,NR_Inter_Relax [s] (number of DRX cycles Note 3) |  | Tevaluate,NR_Inter_Relax [s] (number of DRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤[163.84] | 0.32 | ≥15.36 (12) | 8 | K1 x eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1 x K1) |  | 0.32 x N1 x K1 (1 x N1 x K1) | 0.64 x N1 x K1 (2 x N1 x K1) |
|  | 0.64 | ≥19.2 (15) | 5 |  |  | 0.64 x N1 x K1 (1 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) |
|  | 1.28 | ≥30.72 (24) | 4 |  |  | 1.28 x N1 x K1 (1 x N1 x K1) | 2.56 x N1 x K1 (2 x N1 x K1) |
|  | 2.56 | 40.96 (32) | 3 |  |  | 20.48 | 40.96 |
| NOTE 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. NOTE 2: The number of DRX cycles in this table correspond to the DRX cycles within PTWs. NOTE 3: The eDRX_IDLE cycle lengths are as specified in section 10.5.5.32 of TS 24.008 [42]. NOTE 4: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_Relax*DRX_cycle1.28*1.28. NOTE 5: The measurement shall not be performed across PTW’s. In this case the measurement is performed in the next available PTW. NOTE 6: The evaluation shall not be performed across PTW’s. NOTE 7: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the cellEdgeEvaluation criterion [2]. |  |  |  |  |  |  |  |

From the above requirements it can be learned that the requirement for RRM relaxation when eDRX (20.48 ≤  eDRX_IDLE cycle length ≤[163.84]) is configured simply based on legacy eDRX requirements with applying the relaxation factor K1. Although similar procedures can be used, we strongly prefer do not introduce RRM requirements for LR operation with eDRX, at least for Rel-19. To our understanding, the two features target the exact identical purpose – power saving and it is unlikely the two features operate simultaneously. In addition, since the relaxation factor is 16, to ensure one evaluation period is contained within a PTW window, extra specification work need to be done with very limited gain.

Hence it is suggested that no RAN4 RRM requirements for LP-WUR operation with eDRX (with length ≥20.48) in Rel-19. For the eDRX cycle length 2.56, 5.12 and 10.24, either no RAN4 requirement or same scaling factor 16 is used for 2.56, 5.12 and 10.24 eDRX cycle.

- No RAN4 RRM requirements for LP-WUR operation with eDRX or at least for eDRX with PTW window in Rel-19.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

- Proposals

  - P1: No LP-WUR measurement and evaluation requirement apply when LR is “on” at the legacy state; (vivo ZTE Huawei Ericsson Xiaomi)

Agreement at RAN4 114:

- At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

- At RAN4 114 meeting we have the agreement that “ when and how to turn on LR for serving cell measurement is up to UE implementation” and when LR is “on” at legacy state, whether the LR evaluation requirement apply or not still need discussion.

- Our view is at legacy whether the LR evaluation requirement apply or not apply has either cons or pros, however considering when and how to turn on LR is already up to UE implementation, it is preferred that LR requirements will not apply when LR is “on” at the legacy state.

- At legacy state, when LP-WUR measurement is “ON” for evaluating LR threshold, LR evaluation related requirements will not apply is preferred.

Issue 1-1-11: RRM requirements for FR2

- Proposals

  - P1: Prioritize defining RRM requirements for FR1 in this WI (Apple Nokia)

Observation 1: Regarding FR2 RRM requirements, there is not any concrete technical proposal on FR2 RRM requirements till RAN4 115 meeting

- Suggest RAN4 to consider to drop FR2 RRM requirement in Rel-19 if there is no consensus or still no concrete technical proposals for FR2 RRM requirements in RAN4 116 meeting.

Issue x1: when both Rel-16 relaxation and Rel-19 LP-WUR relaxation are safisfied; or when when both Rel-17 relaxation for Redcap and R19 LP-WUR relaxation are satisfied

Under this scenario the agreement used for Rel-17 Redcap can be reused, i.e., UE could follow the most relaxation requirements.

- When both Rel-16 relaxation and Rel-19 LP-WUR relaxation are satisfied; or when both Rel-17 relaxation for Redcap and R19 LP-WUR relaxation are satisfied, UE is allowed to follow the most relaxation requirements.

Issue x2: When UE leave fully offloading state due to serving cell quality

The issue is related to UE measurement behavior when a UE leave fully offloading state due to serving cell quality purpose. When that UE leave fully offloading mode, there is no measurement performed by MR hence the MR cannot know which state it should stay immediately. The following options are possible:

- Option 1: MR measurement follows RRM relaxation state (with 16 scaling factor), providing RRM relaxation criteria are configured

- Option 2: MR measurement follows legacy state

- Option 3: Speed up measurement for a short period for MR – MR quickly collect some SSBs samples (5) to determine which state (RRM relaxation state or legacy state) it will stay.

To our understanding option 3 is preferred. Since in the reply LS (R4-2508334) when the UE ramping up time is 400ms or 800ms and 5 SSBs are collected, the total period is 500ms and 900ms assuming SSB periodicity is 20ms. This implies that when the UE leaves the fully offloading mode due to paging reason, the MR will quickly collect some samples within a very short period. To align the UE behaviour after leaving fully offloading mode, it is suggested similar approach is used, i.e., option 3.

- When a UE leaves fully offloading state due to serving cell quality, the MR will consecutive measure a few (5) SSBs or consecutively measures SSBs within a transition period (100ms), then the MR will follow either relaxed measurement requirement or legacy measurement requirement.

Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements

Regarding the concrete number of accuracy, the following agreements are available at RAN4 114bis meeting:

Agreement:

For FR1:

    - [3.5] dB can be used for core requirements for LP-RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for LP-RSRP accuracy, under the side conditions Ês/Iot [x, x = -3] dB

    - [3.5] dB can be used for core requirements of SSB based RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for SSB based RSRP accuracy, under the side conditions Ês/Iot [y, y = -3] dB

    - Note: [2 or 2.5] dB RF impairment margin is assumed

Regarding the RF impairment margin, as we indicated in our previous contribution [R4-2503298], the RSRP accuracy requirement includes the impact of the RF impairment and a margin is added whose typical value is from 2-3 dB and 2 dB was suggested [R4-2503298]. Considering the performance of the LP-WUR, the RF impairment margin may need more hence it is ok to use 2.5 dB as suggested by other companies.

- Use 2.5 dB as the RF impairment margin for LP-RSRP and [SS-RSRP] accuracy requirements.

Issue 1-2-4-2-3: On how to define LR evaluation requirements

The working assumption from [4], [6] are:

Option 1: using x1=2*x and y1=2*y for the evaluation requirement.

Option 2: The value x1 for LP-SS based and y1 for SSB based LR evaluation requirement could be defined by adding extra [1 or 2 ] samples

It was determined that the number of samples used to derive the evaluation requirement x1 and y1 for LP-SS and SSB based LR, should be larger than the value x, y, which is the number of SSBs to satisfy the accuracy and obtained through simulation campaign. The difference between option 1 and 2 may imply some implementation issue such as whether a fixed sliding window is used. For example if the number of samples of final evaluation requirement x1 is multiple of x, multiple outputs using x samples can be obtained and used to compared with the threshold. On the other hand if the number of samples of final evaluation requirement x1 is not multiple of x, the output can be simple, for example, average of x1 samples which can also provide extra gain.

In addition we have the following working assumption for LP-SS based LP-WUR

Working assumption:

The number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRP at -3 dB SINR is [from 1 to 3];

The number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRQ for AWGN and TDL-C channel at -3 dB SINR is [2 or 3];

Our preference is the sample number for LP-SS based LP-WUR evaluation requirement is either 2 or 3 and 2 is preferred. The corresponding x1 is 4.

- Using x1=2*x and y1=2*y for the evaluation requirement. The value x1 for LP-SS based LR evaluation requirement could be 4 or [6] under the condition when x = 2 or [3]. The value y1 for SSB based LR evaluation requirement could be 4 under the condition when y = 2.

MR RRM relaxation

LP-WUR connected mode

Issue 1-4-1: LP-WUR at CONNECTED mode

- Proposals

  - P1: No RRM objectives is expected for connected mode in this WI. (oppo ZTE)

  - P2: Postponed until more progress in other groups (xiaomi vivo Ericsson)

  - P3: For LP-WUR at CONNECTED mode, discuss the interruption requirement for LP-WUS monitoring after the DRX related discussion is concluded in RAN1/2.(Apple)

  - P4: Prioritise idle-mode requirements over connected mode requirements. If needed, start the connected mode requirements from LR PSS/SSS and LP-WUS monitoring requirements. (Nokia)

  - P5: In the Connected mode, interruptions on the serving cell due to MGs as defined in Clause 9.1.2 also apply to WUS monitoring by the WUR. In the Connected mode, interruption requirements specified in Clause 8.2 also apply to WUS monitoring by WUR. (Qualcomm)

Regarding P5, in [7], it clearly indicates that,

Simultaneously LR and MR operations are not supported by the UE, where LR operations is the UE operations for LP-WUS monitoring, MR operation is the UE operations for all other NR signals/channels transmissions/receptions in RRC_CONNECTED mode.

Hence MR and LR work at different time and when MR performs measurement during a measurement gap, there will be no LR operation hence there is no need define interruption requirements on WUS monitoring by LR at connected state.

- MR and LR operate at different time, there is no need define interruption requirements on WUS monitoring by LR at connected state.

Conclusion

In this contribution, we provide our considerations on the LP-WUS/WUR WI and have the following observations and proposals.

Observation 1: Regarding FR2 RRM requirements, there is not any concrete technical proposal on FR2 RRM requirements till RAN4 115 meeting

- Specify serving cell and neighbor cell measurement for offloading and RRM relaxation for Redcap UE with 1 Rx and 2 Rx. Existing agreements for offloading and RRM relaxation will be used for Redcap UE.

- For the number of SSB for wake up delay for 1 Rx Redcap, the same number of SSB, i.e., { 3 5 5} SSBs for {10ms, 400ms, 800ms} ramping up time, can be reused initially.

- When both Rel-16 EMR and Rel-19 LP-WUR are configured, for case 1 (offloading) and case 3 (RRM relaxation) defined in Rel-19 LP-WUR:

- when T331 is running, all carriers (carriers configured/not configured for CA/DC idle state measurement) follow (Rel-15) legacy measurements; when T331 is not running, all carriers follow RRM requirements defined in Rel-19 LP-WUR, i.e., relaxed with scaling factor 16 when case 3 (RRM relaxation criteria) are satisfied or serving cell offloading when case 1 criteria are satisfied;

- When both Rel-18 EMR and Rel-19 LP-WUR are configured, consider the following options:

Option 1: When Rel-18 EMR is configured, all carriers follow (Rel-15) legacy measurement requirement.

Option 2: Rel-18 EMR and Rel-19 LP-WUR cannot be configured at the same time (need send LS to RAN2)

- No RAN4 RRM requirements for LP-WUR operation with eDRX or at least for eDRX with PTW window in Rel-19.

- At legacy state, when LP-WUR measurement is “ON” for evaluating LR threshold, LR evaluation related requirements will not apply is preferred.

- Suggest RAN4 to consider to drop FR2 RRM requirement in Rel-19 if there is no consensus or still no concrete technical proposals for FR2 RRM requirements in RAN4 116 meeting.

- When both Rel-16 relaxation and Rel-19 LP-WUR relaxation are satisfied; or when both Rel-17 relaxation for Redcap and R19 LP-WUR relaxation are satisfied, UE is allowed to follow the most relaxation requirements.

- When a UE leaves fully offloading state due to serving cell quality, the MR will consecutive measure a few (5) SSBs or consecutively measures SSBs within a transition period (100ms), then the MR will follow either relaxed measurement requirement or legacy measurement requirement.

- Use 2.5 dB as the RF impairment margin for LP-RSRP and [SS-RSRP] accuracy requirements.

- Using x1=2*x and y1=2*y for the evaluation requirement. The value x1 for LP-SS based LR evaluation requirement could be 4 or [6] under the condition when x = 2 or [3]. The value y1 for SSB based LR evaluation requirement could be 4 under the condition when y = 2.

- MR and LR operate at different time, there is no need define interruption requirements on WUS monitoring by LR at connected state.

- References

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 102

[2] RP-240801, Revised WID: Low-power wake-up signal and receiver for NR ((LP-WUS/WUR), RAN 103

[3] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 105, vivo, NTT DOCOMO

[4] R4-2508280, WF on RRM requirements for NR_LPWUS, RAN4 115, vivo

[5] R4-2505572,	Topic summary for [114][225] NR_LPWUS, RAN4 115, vivo

[6] R4-2508279, Ad hoc minutes for NR_LPWUS, RAN4 115, vivo

[7] TS38.300, running CR