3GPP TSG-RAN WG4 Meeting #116			R4-2510206

Bengaluru, India, 25th -29th Aug 2025

Agenda item:	5.8.1

Title:	Inconsistent RAN4 specification regarding doppler pre-compensations in NGSO

Source:	Nordic Semiconductor ASA

Document for:	Discussion

# Introduction

TS 36.102 states:

## 6.4B Transmit signal quality for category NB1 and NB2 ### 6.4B.1 Frequency error for UE category NB1 and NB2 For UE category NB1 and NB2, the UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift based on received ephemeris information of the SAN in IE EphemerisInfo (TS 36.331 [6]), its own location and UL carrier frequency signalled to the UE by the SAN (according to TS36.300 [8] clause 23.21.2.2). The UE pre-compensated modulated carrier frequency shall be accurate to within the limits in Table 6.4B.1-1, observed over a period of one time slot (0.5 ms for 15 kHz sub-carrier spacing and 2 ms excluding the 2304Ts gap for 3.75 kHz sub-carrier spacing) and averaged over 72/LCtone slots (where LCtone = {1, 3, 6, 12} is the number of sub-carriers used for the transmission), compared to the ideally pre-compensated reference uplink carrier frequency. When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift corresponding to the estimated Doppler frequency at the beginning of the transmission.] Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2 Carrier frequency [GHz]; Frequency error [ppm]. ≤1; ±0.2. >1; ±0.1.

Issue1: Based on current specification and since R17, a UE is not forbidden to pre-compensate doppler autonomously during UL transmission, e.g. on 2ms bases when repetition factor is R=1. For example, a single-tone resource unit is 8ms long, two resource units would be 16ms long, Based on current RAN4 specification, a UE may change doppler pre-compensation frequency between each two out of eight 2ms blocks autonomously when R=1. We believe this is not the specification intention. Therefore, in our opinion, the following condition should be removed. “When a repetition period is configured on the uplink for which repetition period (R ) >1” to ensure that one repetition period (in case of R=1) is also transmitted with the same doppler frequency pre-compensation, unless eNB configures UL segments.

Issue2: In addition, our understanding is that “segmentation is applied” corresponds to “when eNB configures UL segments”.  This editorial change is not essential if this is a general understanding.

36.306 4.3.38.6 ntn-SegmentedPrecompensationGaps-r17 This field indicates the supported gap length between segments for PUSCH and PUCCH required by a UE supporting ce-ModeA-r13 or for NPUSCH required by a UE supporting ue-category-NB, for TA pre-compensation. This feature is only applicable if the UE supports either ue-category-NB or ce-ModeA-r13 and also supports ntn-Connectivity-EPCr17. If a UE does not include this field but includes ntn-Connectivity-EPC-r17, in case of overlapped transmission between successive uplink segments, UE shall follow the procedure specified in TS 36.213 [22]. This field is not applicable for UEs indicating support of ue-Category-NB and ntn-ScenarioSupport-r17 with value GSO.

Proposed TP for 36.102: under [LTE_NBIOT_eMTC_NTN]

## 6.4B Transmit signal quality for category NB1 and NB2 ### 6.4B.1 Frequency error for UE category NB1 and NB2 For UE category NB1 and NB2, the UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift based on received ephemeris information of the SAN in IE EphemerisInfo (TS 36.331 [6]), its own location and UL carrier frequency signalled to the UE by the SAN (according to TS36.300 [8] clause 23.21.2.2). The UE pre-compensated modulated carrier frequency shall be accurate to within the limits in Table 6.4B.1-1, observed over a period of one time slot (0.5 ms for 15 kHz sub-carrier spacing and 2 ms excluding the 2304Ts gap for 3.75 kHz sub-carrier spacing) and averaged over 72/LCtone slots (where LCtone = {1, 3, 6, 12} is the number of sub-carriers used for the transmission), compared to the ideally pre-compensated reference uplink carrier frequency. When a repetition period is configured on the uplink for which repetition period (R ) >1, tThe UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied segments are configured by eNB, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift corresponding to the estimated Doppler frequency at the beginning of the transmission.] Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2 Carrier frequency [GHz]; Frequency error [ppm]. ≤1; ±0.2. >1; ±0.1.

# Conclusion

Intention of this contribution is to point out inconsistency in the specification, if any of two issues is found essential by 3GPP, consider CR on this topic in the future.