3GPP TSG-RAN WG4 Meeting #116			R4-2510207

Bengaluru, India, 25th – 29th Aug 2025

Agenda item:	7.28.2

Title:	On reply to LS on Harmonised Standard for NTN capable UE

Source:	Nordic Semiconductor ASA

Document for:	Discussion

1. Introduction

In RAN1#116 LS from ETSI on NTN IoT harmonized standard was received.

| R4-2509023 | LS on Harmonised Standard for NTN capable UE | Rel-17 | SES(25)000071 |  | Session Topic BDaT session ETSI TC SES has started the development of new harmonized standards for 3GPP-based IoT NTN and NR NTN capable UEs operating in respectively 256/255 and n256/n255 bands. With this LS, ETSI TC SES would like to provide further information to 3GPP RAN4 on the progress and agreements. ACTION: The “nominated bandwidth concept” used for the purpose of defining the Spectral Emission Mask requirement and defined in the relevant existing ETSI harmonised standards (e.g. EN 301 442 and EN 301 444), will be further discussed in ETSI TC SES. ETSI TC SES would welcome feedback from 3GPP on the usefulness of the concept before ETSI TS SESSCN#53e meeting (25th Sept 2025) where conclusion on this topic is expected. |
| --- | --- | --- | --- | --- | --- |

As noted by chair, LS requests a RAN4 reply to nominated bandwidth concept. Based on guidance, it should be discussed under  R19 NTN IoT.

2. Discussion

In our opinion, concept of nominated bandwidth is not only welcome, but it is essential for narrow band systems like NB-IoT.  NB-IoT RAT is low-cost, unlike LTE/LTE-M/NR, comprises only 10kHz guard band, one on each side of the channel. It is infeasible to comply with strict SEM requirements in ETSI EN 301 441 without considering significant A-MPR for edge tones, making the whole NB-IoT system unusable in NTN contect in practice.

Generally speaking, nominated bandwidth concept is equivalent to introducing an additional guard-band for NB-IoT SEM measurement.  For example, selected additional guard-band to be 100kHz on each side, corresponding nominated BW  would be 400kHz.  There is one-to-one mapping between these two concepts, when it comes to SEM requirements in ETSI and in 3GPP.

Important aspect is that nominated-BW/additional guard-band is not freely selectable by a UE. This was pointed out by multiple companies already in RAN1#115 when topic of nominated BW was discussed online under b249. Fixed nominated BW/guard-band will guide NTN MNO in their channel deployments.  3GPP should introduce at most two nominated-BW/additional guard-band per band per RAT.

For RATs other than NB-IoT, concept of nominated BW / additional guard-band maybe be introduced as well, once use-cases are identified.

3. Conclusion

Draft reply to ETSI:

RAN would like to thank SES for the LS. Regarding “nominated bandwidth concept”, RAN4 finds this ETSI concept to be essential for at least narrow-band RATs like 3GPP NB-IoT NTN, where it is not feasible to operate without significant A-MPRs with legacy 10kHz guard-bands while meeting band b249/b254 (1610-1626.5MHz) SEM requirements in ETSI EN 301 441.