3GPP TSG-RAN WG4 Meeting #116			R4-2500208

Bengaluru, India, 25th - 29th Aug 2025

Agenda item:	7.29.5

Title:	Discussion on UE RF requirements for NTN NB-IoT TDD mode

Source:	Nordic Semiconductor ASA

Document for:	Discussion

# 1. Introduction

In RAN1#115 in Malta the following was agreed for in-band SEM:

# 2. Discussion

## A-MPR

Using commercial module, we measured SEM with 30kHz measurement BW @1526.5 MHz, and measured the following values listed in the table. This being a single-device measurement. In practice some margin is needed for the UE to ensure that every device can pass the requirement.

| NS_05N | 12-tone | 6-tone edge | 3-tone edge | 1-tone edge 15kHz |
| --- | --- | --- | --- | --- |
| Legacy A-MPR | 2dB | 1dB | 0dB | 0dB |
| Guard band 0kHz + 10kHz (legacy) | 4.3dB | 9.88 | 12.14 | 16.59dB |
| Guard band 50kHz + 10kHz (legacy) | 0dB | 0dB | 0dB | [0]dB |

Generally speaking, it is not feasible to support A-MPR-less implementation with only 10kHz legacy guard-band. If NS-05N requirements are to be met, at least 50kHz of additional GB needs to be introduced for SEM measurements. Considering higher power classes, such as PC2 or higher, 100kHz of additional guard-band is more forward compatible solution.

Proposal-1:  If additional guard-band for SEM measurement is defined for b254 as part of R18 maintenance, adopt b254 NS_04N and NS_05N also for b249. Otherwise, introduce new NS values for which (ΔfOOB) is starting from the +- edge +- 100kHz of the assigned category NB1 or NB2 channel bandwidth.

## EIRP density within the operational band

According to our investigation, no aeronautical radionavigation systems are deployed in the frequency range of 1616-1626.5MHz. Therefore, the operational band’s mean EIRP density requirement in [1] of under -3 dB(W/4kHz) applies for the band 249. Therefore, agreed 23dBm transmit power is well within the requirements.

Observation-1: Current specification satisfies the operational band’s mean EIRP density requirement.

## NPDSCH demodulation performance

36.102 defined NPDSCH demodulation performance requirements.

In RAN4#114b, it has been agreed that target SNR for TDD band is -1dB. Therefore, the NPDSCH Minimum performance tables shall be updated accordingly in 36.102.  We suggest assuming 4 repetitions for NPDSCH and set R_max=4 and G=16. The MCS (iTBS, iSF) to be selected is FFS.

Proposal-2: Revisit the minimum performance requirements for NPDSCH for R17 NTN

- assume 4 NPDSCH repetitions, -1dB target SNR, R_max=4 and G=16, FFS: MCS (iTBS, iSF)

# 3. Conclusion

Proposal-1:  If additional guard-band for SEM measurement is defined for b254 as part of R18 maintenance, adopt b254 NS_04N and NS_05N also for b249. Otherwise, introduce new NS values for which (ΔfOOB) is starting from the +- edge +- 100kHz of the assigned category NB1 or NB2 channel bandwidth.

Observation-1: Current specification satisfies the operational band’s mean EIRP density requirement.

Proposal-2: Revisit the minimum performance requirements for NPDSCH for R17 NTN

- assume 4 NPDSCH repetitions, -1dB target SNR, R_max=4 and G=16, FFS: MCS (iTBS, iSF)

# 4. References

[1] Radio regulations, ITU, 2024

[2] ETSI EN 301 441, V2.1.1, 2016