3GPP TSG-RAN WG4 Meeting #116			R4-2510209

Bengaluru, India, 25th -29th Aug 2025

Agenda item:	7.29.6

Title:	On reply to RAN1 LS on frequency and time pre-compensations

Source:	Nordic Semiconductor ASA

Document for:	Discussion

1. Introduction

In RAN1#115 in Malta the following was agreed for b249 timing and frequency error pre-compensation. Timing error aspect are in square brackets, because they are to be still confirmed by RRM session.

2. Discussion

We visualised the following 800km orbit  in Figure 1.  Satellite passes above UE at 90deg elevation.  At 1.6GHz center frequency. Orbit has 90 deg inclication and UE is at (0,0,0) x,y,z coordinate. Orbit details are the following:

"semi-major axis" : 7178137.0,

"eccentricity" : 0.0,

"inclination" : 1.5707963267948966,

"anomaly" : 0,

"longitude" : 0,

}

Figure 1 shows the Doppler, RTT of service link and elevation and azimuth of the satellite.

Figure 1 Visualization of 800km orbit

In Figure 1, the largest drift of doppler is at 90 deg elevation and is about 300 Hz/s -> 2.4Hz per 8ms

In Figure 1, the largest drift of RTT is at 0 elevation and is about 46us/s -> 0.368us/8ms -> 11.28Ts

Even if this number of RTT drift would be doubled due to presence of feeder link in the system, 22.56Ts is still less than 25% of requirement which is 97Ts. This is why frequency nor TA pre-compensations within 8ms UL burst are necessary for the NTN TDD operation based on existing satellites operating in the target TDD band.

3. Conclusion

Reply to RAN1 that also from RAN4 point of view further segmentation within 8ms burst for NTN TDD mode in b249 is not necessary/essential.