3GPP TSG-RAN WG4 Meeting #116	R4-2510217

# Bengaluru, India, August 25th – 29th, 2025

Title:	Discussion on FR2 CA MPR related feature list

Source:	Samsung

Agenda item:	7.1.2.2.3

Document for:	Discussion

1.	Introduction

In last meeting two new UE capabilities for FR2 CA enhancement were agreed in the approved feature list [1, R4-2507576], with details as below

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-4 | FR2 MPR-Improvement Downlink Independent | Indicates whether UE supports reduced MPR by removing dependence on DL CA configuration. |  | Yes | N/A | The FR2 UE only supports MPR requirement with dependence of DL CA configuration. | Per BC | Yes | FR2 only | N/A |  | Optional with capability signaling |
|  | 46-5 | FR2 MPR Improvement Activation Dependent | Indicates whether UE supports MPR based on activation status of its configured CCs |  | Yes | N/A | The FR2 UE shall support MPR requirement based on UL CA configuration. | Per BC | Yes | FR2 only | N/A |  | Optional with capability signaling |

A tiny error was identified later in the column “Consequence if the feature is not supported by the UE“ for “46-5 FR2 MPR Improvement Activation Dependent”. In this contribution we propose to correct this error.

2. 	Discussion

For the Rel-19 NR_ENDC_RF_Ph4 work item, RAN4 has agreed to introduce two new UE capabilities for MPR enhancement, for FR2 intra-band contiguous CA. One capability is to indicate that the UE supports reduced MPR by removing dependence on DL CA configuration, the other is to indicate that UE supports MPR based on activation status of its configured CCs. The two new UE capabilities are both optional and independent to each other. The possible UE supporting status has been summarized in the LS to RAN2 [2, R4-2505144], reproduced as below

|  | [fr2-MPR-ImprovementDownlinkIndependent-r19] supported? | [fr2-MPR-ImprovementActivationDependent-r19] supported? | MPR dependency |
| --- | --- | --- | --- |
| Legacy UE | No | No | Downlink+Uplink configured CCs （CABW） |
| Enhanced R19 UE | Yes | No | Uplink configured CCs |
| Enhanced R19 UE | No | Yes | Downlink+Uplink activated CCs (within CABW) |
| Enhanced R19 UE | Yes | Yes | Uplink activated CCs |

Consequently, when [fr2-MPR-ImprovementActivationDependent-r19] is not supported, the MPR dependency could be either CABW (dominated by DL configured CCs) or UL configured CCs.

Accordingly, the feature list description should be updated by removing the ‘UL’ wording in the column “Consequence if the feature is not supported by the UE” for “FG 46-5 FR2 MPR Improvement Activation Dependent”.

Proposal 1:	Update the feature list by removing the ‘UL’ wording in the column “Consequence if the feature is not supported by the UE” for “FG 46-5 FR2 MPR Improvement Activation Dependent”.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-5 | FR2 MPR Improvement Activation Dependent | Indicates whether UE supports MPR based on activation status of its configured CCs |  | Yes | N/A | The FR2 UE shall support MPR requirement based on UL CA configuration. | Per BC | Yes | FR2 only | N/A |  | Optional with capability signaling |

3. 	Conclusion

Proposal 1:	Update the feature list by removing the ‘UL’ wording in the column “Consequence if the feature is not supported by the UE” for “FG 46-5 FR2 MPR Improvement Activation Dependent”.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-5 | FR2 MPR Improvement Activation Dependent | Indicates whether UE supports MPR based on activation status of its configured CCs |  | Yes | N/A | The FR2 UE shall support MPR requirement based on UL CA configuration. | Per BC | Yes | FR2 only | N/A |  | Optional with capability signaling |

4. 	References

- R4-2507576     “Topic summary for [115][137] NR_feature_list”, Moderator (CMCC)

- R4-2505144     “LS on introduction of new UE capabilities for FR2 MPR improvement”, Samsung, Qualcomm