3GPP TSG-RAN WG4 Meeting #116	R4-2510218

# Bengaluru, India, August 25th – 29th, 2025

Title:	Discussion on FR2 UE RF requirements for LP-WUR

Source:	Samsung

Agenda item:	7.24.3.2

Document for:	Discussion

1.	Introduction

According to the WF of last meeting [1, R4-2507936], the remaining issues for FR2 LP-WUR are as following:

Issue 2-2-2: Diversity gain value for FR2 LP-WUR Agreements: - -3dB for diversity gain. - FFS: 1% MDR REFSENS requirement should be satisfied for each orthogonal link polarization respectively. Issue 2-2-4: NF+IM for FR2 LP-WUR Agreements: - For FR2, NF+IM is [12] dB for at least type-2 LP-WUR. - FFS two values for two types. Issue 2-2-6: other Rx requirements for FR2 LP-WUR WF: - FFS whether skip out-of-band blocking and spurious response requirements for FR2 LP-WUR.

In this contribution, we share views focus on above remaining issues targeted at core requirement completion in this meeting.

2. 	Discussion

2.1 LP-WUR type for FR2

It is agreed to define two types of LP-WUR for FR1 targeted for different NF+IM values. However, for FR2 all discussion has been focusing on OFDM receiver only. In addition, it was also assumed that LR and MR is probably to share the same chain for FR2. Given this is the final meeting for core requirement, the practical way may be to define only one type of LP-WUR for FR2 based on OFDM receiver.

Proposal 1:	Define only one type of LP-WUR for FR2 based on ODFM receiver for timely completion of WI.

2.2 REFSENS and spherical coverage for FR2 LP-WUR

REFSENS and other RX requirements of LP-WUR are agreed to use MDR criteria, so the EIS metric corresponding to a certain throughput level is not suitable any more. The metric for MR is actually an “averaged EIS” metric, because the legacy EIS metric only requires the averaged performance between different link polarization testing but the new MDR metric requires the worst case between the two polarizations testing has to pass. For EIS, there is a sensitivity search procedure for each link polarization, so the EIS at each test point can be averaged:

averaged EIS = 2*[1/EIS(PolMeas= PolLink= +1/EIS(PolMeas= PolLink=]-1

For MDR, it was already agreed that it is pass/fail testing without searching the exact power level yielding 1% MDR. An average approach between the two orthogonal link polarization is not applicable. Given -3dB diversity gain was already agreed in last meeting, it is reasonable to apply the same REFSENS requirement to each orthogonal link polarization respectively.

Proposal 2:	Do not apply EIS metric to LP-WUR REFSENS.

Proposal 3:	MDR testing in REFSENS is one-shot pass/fail testing without searching the exact power level yielding 1% MDR

Proposal 4:	1% MDR REFSENS requirement applies for each orthogonal link polarization respectively

Regarding spherical coverage requirements, EIS CDF approach is adopted for MR. For LP-WUR, given there is no EIS testing but one-shot pass/fail testing for each direction, CDF approach is not applicable either. The spherical coverage requirement for LP-WUR can be defined as the minimum fraction of the sphere over which the MDR criterion is met. The minimum fraction here is corresponding to the N-th%-tile for each power class, i.e. (1-N%)

Proposal 5:	Do not apply CDF approach for LP-WUR spherical coverage

Proposal 6	The spherical coverage requirement for LP-WUR can be defined as the minimum fraction of the sphere over which the MDR criterion is met, e.g., the minimum fraction >= (1-N%) for each power class.

2.3 blocking and spurious response

In RAN4#114, whether to specify out-of-band blocking and spurious response requirements is still an open issue [1]

Issue 2-2-6: other Rx requirements for FR2 LP-WUR WF: - FFS whether skip out-of-band blocking and spurious response requirements for FR2 LP-WUR.

It is common understanding that out-of-band blocking and spurious response are two correlated requirements. Given there is no out-of-band blocking for MR, so no spurious response requirements are specified accordingly. The same principle can be used for LR.

Proposal 7:	No need to specify out-of-band blocking and spurious response requirements for FR2 LP-WUR.

3. 	Conclusion

Proposal 1:	Define only one type of LP-WUR for FR2 based on ODFM receiver for timely completion of WI.

Proposal 2:	Do not apply EIS metric to LP-WUR REFSENS.

Proposal 3:	MDR testing in REFSENS is one-shot pass/fail testing without searching the exact power level yielding 1% MDR

Proposal 4:	1% MDR REFSENS requirement applies for each orthogonal link polarization respectively

Proposal 5:	Do not apply CDF approach for LP-WUR spherical coverage

Proposal 6	The spherical coverage requirement for LP-WUR can be defined as the minimum fraction of the sphere over which the MDR criterion is met, e.g., the minimum fraction >= (1-N%) for each power class.

Proposal 7:	No need to specify out-of-band blocking and spurious response requirements for FR2 LP-WUR.

4. 	References

- R4-2507936    “WF for [115][136] NR_LPWUS_UERF”, vivo