3GPP TSG-RAN WG4 Meeting #116	R4-2510220

# Bengaluru, India, August 25th – 29th, 2025

Title:	Discussion on testability for LP-WUR

Source:	Samsung

Agenda item:	7.24.3.4

Document for:	Discussion

1.	Introduction

Testability is one of the scopes for this work item. Up to now, the main outcome is the general framework as agreed in [1, R4-2508090]:

It has been agreed that same LP-WUS RF requirements will apply to all RRC states, but only one RRC state needs to be tested. It is agreed to verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods). The ultimate test method including the potential test mode will be decided by RAN5 based on RAN4 input. For demodulation requirements FAR will also be used.

There are still many other testability issues to be addressed by RAN4. Specially, enormous configuration permutations for both LR and MR, various UE capabilities, and so on, lead to dramatically high test burden.

In this contribution, we share our views on LP-WUR testability focusing on reducing test burden.

2. 	Discussion

RAN4 requirements apply to all possible cases but it is not practical and not necessary to test all of those. LP-WUR as a simple receiver but bonding with NR’s various configurations leading to complicated test cases. LP-WUR UE can support various different capabilities leading to numerous verification scenarios. UE can support OOK, OFDM, or both; UE can support RRC_Idle/Inactive, RRC_Connected, or both; UE can support Type 1 or Type 2 requirements; … Actually the RF performance can be considered the same in many different scenarios. It is kind of waste of time to verify all possible scenarios for a simple receiver which is not part of the main radio.

2.1 NR configurations

Though LP-WUS bandwidth is fixed as 11 RBs, the LR UE RF requirements for all NR channel BW. E.g.,

Table 7.3M.2-1a: One antenna port reference sensitivity for FDD and TDD bands

| RX parameter | SCS kHz | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 3, 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70,80, 90, 100 MHz (dBm) |
| REFSENS for UEs supporting [Type1 LR] | 15 | TBD |
|  | 30 | TBD |
| REFSENS for UEs supporting [Type2 LR] | 15 | TBD |
|  | 30 | TBD |

The NR configurations are actually the assisting role in the LR RF performance verification. It would be enough to verify with single channel BW, e.g., the middle CBW or the worst case CBW.

Proposal 1:	LP-WUR’s UE RF requirement can be verified with single set of MR configuration, e.g., single channel BW.

2.2 RRC states and waveform

There kinds of RRC states are involved for LP-WUS feature, i.e., RRC_Idle, RRC_inactive and RRC_Connected, and two kinds of waveform are involved, i.e., OOK and OFDM.

| Features | Index | Feature group |
| --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM [overlaid] sequence |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence |

It has been agreed that same LP-WUS RF requirements will apply to all RRC states, but only one RRC state needs to be tested. For the case when LR supporting both states, the test skipping rule is still open [2, R4-2507936]

Issue 4-1-3: skipping rule for LP-WUR different mode Agreements: - For idle-mode only or connected-mode only LP-WUR, the test should be done accordingly. - FFS For LP-WUR support both.

Compared with RRC_Connected state, Supporting LP-WUS in RRC_Idle state showed most obvious power saving benefits. So the LP-WUS performance in RRC_Idle state is more important for commercialization. In this sense, it is more meaningful to test with RRC_Idle state for UE supporting both idle and connected modes.

Proposal 2:	For the case when LR supporting both RRC_Idle/Inactive and RRC_Connected states, only test with RRC_Idle state.

Similar as RRC states, one UE may also support both OOK and OFDM. Usually, the requirements for OFDM are more stringent than OOK. In this sense it is reasonable to verify with OFDM waveform only.

Proposal 3:	For the case when LR supporting both OOK and OFDM waveforms, only test with OFDM waveform.

2.3 LP-WUR type

According to previous agreement, “A UE can only support either Type 1 or Type 2 but not both for LP-WUR” [3, R4-2505234], there is no overlapping case and hence no test skipping rule needed.

2.4 FR2 spherical coverage

It has been proposed to skip the FR2 spherical coverage for LP-WUR in previous meetings as long as REFSENS of LP-WUR is met. In last meeting it was agreed that the peak direction for LP-WUR is the same as legacy RX beam peak direction for MR. This is from the assumption that the LR and MR share the same chain, but beam forming is appliable for MR with antenna array, but no beamforming assumption for LR with single antenna element. Base on this background, it is not necessary to test FR2 LP-WUR spherical coverage as long as REFSENS is pass.

Proposal 4:	it is proposed to skip the FR2 LP-WUR spherical coverage test

2.5 MDR testing time

MDR testing is time consuming if the LR wakes up MR for each LP-WUS signal, especially for Idle mode. To save test time, actually it is not necessary to wake up MR for each detected LP-WUS signal. The UE can count the detection rate without waking up the MR for each detected LP-WUS signal but just wake up the MR for last one.

Proposal 5:	To verify LP-WUS RF performance with MDR testing, it is enough to counter the detection rate without waking up the MR for each LP-WUS signal.

3. 	Conclusion

Proposal 1:	LP-WUR’s UE RF requirement can be verified with single set of MR configuration, e.g., single channel BW.

Proposal 2:	For the case when LR supporting both RRC_Idle/Inactive and RRC_Connected states, only test with RRC_Idle state.

Proposal 3:	For the case when LR supporting both OOK and OFDM waveforms, only test with OFDM waveform.

Proposal 4:	it is proposed to skip the FR2 LP-WUR spherical coverage test

Proposal 5:	To verify LP-WUS RF performance with MDR testing, it is enough to counter the detection rate without waking up the MR for each LP-WUS signal.

4. 	References

- R4-2508090   “TP to TR 38.774 on testability of LP-WUR”, Samsung

- R4-2507936   “WF for [115][136] NR_LPWUS_UERF”, vivo

- R4-2505234    “WF for [114bis][131] NR_LPWUS_UERF”, vivo