3GPP TSG-RAN WG4 Meeting #116	R4-2510222

# Bengaluru, India, August 25th – 29th, 2025

Title:	Discussion on partial sphere metric for NTN OTA

Source:	Samsung

Agenda item:	7.10.3.2

Document for:	Discussion

1.	Introduction

In RAN4#114bis meeting, there usage scenarios NTN handheld UE were agreed and legacy full sphere TRP TRS metric still applies for scenario 1 and 3 [1, R4-2504763]:

Issue 2-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2 Agreements: o For IoT-NTN and NR-NTN, typical usage scenarios are: § Scenario 1: both head+hand mode and hand only mode. § Scenario 2: hand only mode targeted for specific UE orientation usage § Scenario 3: hand only mode targeted for arbitrary UE orientation usage

For Scenario 2, partial sphere metric will be adopted and in RAN4#115 Malta meeting much progress was made, as captured in the approved WF [2, R4-2508782]

Issue 2-1-1: Details on NTN usage scenario declaration Agreements: - RAN4 to adopt the following rules for scenario declaration in NTN OTA testing: - DUT shall declare to support only one scenario - The declaration of scenario applicability shall be consistent with product manual usage scenario - The declaration of scenario shall be clearly stated in the test report Issue 2-1-2: alignment of understanding on UE positioning guideline for Scenario 2 Agreements: - For Scenario 2 UEs: - The DUT shall be mounted in a suitable hand phantom - The elevation tilt of the default position of the DUT can be: - 0 degree (similar to Figure 6.1-1 of TR 38.870, but with hand phantom) or as shown below - [±15, ±30, ±45] degrees; Other tilt angles are not precluded - NOTE 1: the granularity of the elevation tilt to be further discussed next meeting - The elevation tilt of the hand phantom & DUT is determined based on OEM declaration Issue 2-1-4: Whether the partial sphere is using the =0° (top pole) as a center Agreements: the partial sphere metric is using the  =0° (top pole) as a center Issue 2-1-5: What is the angle range of the partial sphere-after concluding center of the sphere Agreements: - RAN4 further discuss the theta angle range of partial sphere metric, from ±30°, ±60°, ±90°. - Option 1: single range is selected and captured in TR - Option 2: multi-range in TR but based on UE declaration, details are FFS - Note1: the phi range should be 360 degrees - Note2: if multiple ranges are captured in TR, the metric applicability rule should also be defined. The target is selecting one for NTN scenario 2. Issue 2-1-6: Whether Using integration of EIRP/EIS or CDF? Agreements: - Integration is baseline. - If different range is declared in issue 2-1-5, how to ensure the metric comparable should be discussed and resolved.

In this contribution, we further discuss the open issues as indicated in above agreements, mainly focusing on the elevation tilt in UE positioning guideline and the angle range of the partial sphere.

2. 	Discussion

2.1 UE positioning guideline for Scenario 2

It was agreed that the default orientation for Scenario 2 is the same as that for free space, while elevation tilt is allowed based on vendor declaration:

Agreements: - For Scenario 2 UEs: - The DUT shall be mounted in a suitable hand phantom - The elevation tilt of the default position of the DUT can be: - 0 degree (similar to Figure 6.1-1 of TR 38.870, but with hand phantom) or as shown below - [±15, ±30, ±45] degrees; Other tilt angles are not precluded - NOTE 1: the granularity of the elevation tilt to be further discussed next meeting - The elevation tilt of the hand phantom & DUT is determined based on OEM declaration

Then the remaining issues are all about the elevation tilt. Though several elevation tilt angles are captured in square bracket as examples i.e. [±15, ±30, ±45] degrees, it was also agreed that other tilt angles are not precluded. In our understanding, the elevation tilt angle should be not limited up to +/-45degree. To well reflect all potential NTN antenna design, it is suggested to allow the elevation tilt angle up to +/-90degree.

According to the definition of NTN scenario 2:

§  Scenario 2: hand only mode targeted for specific UE orientation usage

This Scenario 2 is targeted for specific UE orientation usage which is arbitrary. There is no premise that the specific UE orientation usage is within a too limited range.

Observation 1:	The definition of NTN scenario 2 allows any arbitrary UE orientation. There is no premise that the specific UE orientation usage is within a too limited range

Due to testability issue, i.e., only elevation tilt is supported and roll tilt is not supported after the DUT is mounted in the hand phantom, that is actually already a limitation to the arbitrary UE orientation. It is not reasonable to further restrict the elevation tilt angle ranges.

Observation 2:	only allowing elevation tilt is already a limitation to the Scenario 2 targeted arbitrary UE orientation because roll tilt is not supported due to testability issue. It is not reasonable to further restrict the elevation tilt angles.

In last meeting when RAN4 discussed the declaration rule of scenario applicability, it was agreed that “The declaration of scenario applicability shall be consistent with product manual usage scenario”. Given the elevation tilt is also a declaration parameter which is expected to be consistent with product manual, it should be up to vendor declaration from any possible elevation tilt angles in granularity.

Observation 3:	elevation tilt is a vendor declaration parameter and it should be up to vendor declaration from any possible elevation tilt angles in granularity.

Based on above discussions, we propose that the elevation tilt angle should be declared from {-90º, -75º, -60º, -45º, -30º, -15 º, 0º, 15º, 30º, 45º, 60º, 75º, 90º}

Proposal 1:	the elevation tilt angle shall be declared from {-90º, -75º, -60º, -45º, -30º, -15 º, 0º, 15º, 30º, 45º, 60º, 75º, 90º}

2.2 angle range of the partial sphere

Besides elevation tilt angles, another controversial issue is about the angle range of the partial sphere. The agreement of last meeting shows 3 theta angle options (±30°, ±60°, ±90°) and it is also open if single range or multi-range could be captured in the TR.

Issue 2-1-5: What is the angle range of the partial sphere-after concluding center of the sphere Agreements: - RAN4 further discuss the theta angle range of partial sphere metric, from ±30°, ±60°, ±90°. - Option 1: single range is selected and captured in TR - Option 2: multi-range in TR but based on UE declaration, details are FFS

Regarding the 3 theta angle options (±30°, ±60°, ±90°), it should be noted that the corresponding solid angle is not in a linear relationship with theta angle.

| θ angle(degree) | Surface ratio of Spherical-cap = (1-cos(θ))/2 |
| --- | --- |
| 15 | 1.7% |
| 30 | 6.7% |
| 45 | 14.64% |
| 60 | 25% |
| 75 | 37% |
| 90 (half sphere) | 50% |
| 105 | 62.94% |
| 120 | 75% |

It can be noticed that when the theta angle is small, the surface ratio of the corresponding spherical cap out of the whole sphere is way smaller than expected. E.g., when theta angle is 30degree, the spherical cap only occupies 6.7% of the whole sphere; when theta angle is 60degree, the spherical cap only occupies 25% of the whole sphere.

Observation 4:	The theta angle and its corresponding solid angle is not in a linear relationship. When theta angle is 30degree, the spherical cap only occupies 6.7% of the whole sphere; When theta angle is 60degree, the spherical cap only occupies 25% of the whole sphere.

Even when theta angle is 60degree, a 25% of whole sphere can not cover all satellites in the sky. In this sense, both 30degree and 60degree theta angles are questionable to be adopted as NTN OTA evaluated range.

Observation 5:	when theta angle is 60degree, a 25% coverage of whole sphere can not cover all satellites in the sky, and thus not applicable to be adopted as NTN OTA evaluated range.

Based on above discussion, it is meaningful to only consider theta angle as +/-90degree, i.e., upper hemisphere as the angle range for partial sphere NTN OTA.

Regarding if single range or multi-range should be adopted, if multi-range was adopted, e.g., both 60degree and 90degree theta angle range were adopted depending on vendor declaration, it is still challenging to find method on how to ensure the metric comparable.

Observation 6:	if multi-range was adopted, e.g., both 60degree and 90degree theta angle range were adopted depending on vendor declaration, it is still challenging to find method on how to ensure the metric comparable.

Based on above discussion, it is proposed to adopt single range and 90degree theta range (i.e. upper hemisphere) as NTN OTA partial sphere.

Proposal 2:	it is proposed to adopt single range and 90degree theta range (i.e. upper hemisphere) as NTN OTA partial sphere.

2.3 integration of EIRP/EIS

In last meeting, RAN4 discussed whether integration of EIRP/EIS or CDF based approach should be adopted, majority view is to adopt integration than CDF. It was agreed in last meeting that integration of EIRP/EIS as baseline. For completion of core part in this meeting, it is proposed to confirm the legacy integration approach.

Proposal 3:	Integration of EIRP/EIS in upper hemisphere as the performance metric for NTN Scenario 2.

3. 	Conclusion

Observation 1:	The definition of NTN scenario 2 allows any arbitrary UE orientation. There is no premise that the specific UE orientation usage is within a too limited range

Observation 2:	only allowing elevation tilt is already a limitation to the Scenario 2 targeted arbitrary UE orientation because roll tilt is not supported due to testability issue. It is not reasonable to further restrict the elevation tilt angles.

Observation 3:	elevation tilt is a vendor declaration parameter and it should be up to vendor declaration from any possible elevation tilt angles in granularity.

Proposal 1:	the elevation tilt angle shall be declared from {-90º, -75º, -60º, -45º, -30º, -15 º, 0º, 15º, 30º, 45º, 60º, 75º, 90º}

Observation 4:	The theta angle and its corresponding solid angle is not in a linear relationship. When theta angle is 30degree, the spherical cap only occupies 6.7% of the whole sphere; When theta angle is 60degree, the spherical cap only occupies 25% of the whole sphere.

Observation 5:	when theta angle is 60degree, a 25% coverage of whole sphere can not cover all satellites in the sky, and thus not applicable to be adopted as NTN OTA evaluated range.

Observation 6:	if multi-range was adopted, e.g., both 60degree and 90degree theta angle range were adopted depending on vendor declaration, it is still challenging to find method on how to ensure the metric comparable.

Proposal 2:	it is proposed to adopt single range and 90degree theta range (i.e. upper hemisphere) as NTN OTA partial sphere.

Proposal 3:	Integration of EIRP/EIS in upper hemisphere as the performance metric for NTN Scenario 2.

4. 	References

- R4-2504763    “WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3”, vivo, RAN4#114bis

- R4-2508782    “Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3”, vivo, RAN4#115