TSG-RAN Working Group 4 (Radio) meeting #116	R4-2510229

Bangalore, India, 25th to 29th August 2025

Source:	Ericsson

Title:	Discussion on BS RF evolution general aspects

Agenda item:	7.9.2

Document for:	Approval

# Introduction

Since the start of Rel-19 a working item with the intention to evolve BS RF specifications is ongoing. The work item was created with large set of different work tasks, as listed below:

- Evolving co-location requirement and re-solving test issues related to bands above 3 GHz.

- Optimizing test coverage for some OTA requirements.

- Addition of new requirement for protection of FSS UL for band n104.

- Improvements of TRP test methods in TS 38.141-2, Annex I.

- Re-evaluation of transmitter spurious emission requirement levels.

The work schedule included activities to be executed in serial as well as in parallel including both RF core aspects and conformance aspects. For some activities the partitioning between RF core and conformance is not very clear.

RAN4 is now coming to a point where Rel-19 RF core work should be finalized moving focus to conformance test aspects for the remaining part of RAN4 Rel-19. A substantial part of the planned work has been done, while some aspects to e.g. evolvement of co-location requirements didn’t manage to reach the goal to introduce updates within the scope of Rel-19. The work related to co-location requirement improvements have more been aimed to study the background for fundamental assumptions needed for requirement derivation and test method development. It can now be concluded that the work related to improving the co-location requirement needs to be extended beyond current release into Rel-20.

In this contribution we summarize the status of the work done in BS RF evolution WI in Rel-19 and suggest a continuation of the BS RF evolution work in Rel-20.

# Discussion

In Rel-19, RAN4 have worked to improve the BS RF specification within the following areas:

- Evolving co-location requirements: This work has aimed to re-solve identified technical test issues related to bands above 3 GHz. Several technical issues related to RF core requirement derivation and conformance testing have been identified. In Rel-19 work related to better understand the coupling loss assumption used for requirement derivation have been studied via measurement results and simulation results. It’s clear that the BS-to-BS coupling loss relevant for bands in upper region of FR1 deviate significantly from the original assumptions used in Rel-15. This would directly impact the requirement definition and corresponding requirement levels. In addition, technical issues related to the current requirement concept of using a co-location reference antenna and corresponding Co-Location Test Antenna (CLTA) have been discussed. Measurements have been conducted to understand the coupling loss if another type of antenna is used (e.g. Horn antennas). It can be concluded that the type of antenna and how the antenna is located will have large impact on the coupling loss. It’s difficult to mimic a situation providing the same coupling loss as the CLTA achieve. The outcome from the Rel-19 finding will be captured in TR 37.941 to facilitate future normative work in Rel-20 to evolve co-location requirements in TS 38.104, TS 38.141-1 and TS 38.141-2 and corresponding MSR specifications if needed.

- Optimizing OTA test coverage: For some requirements (Operating Band Unwanted Emission, Transmitter intermodulation and Receiver out-of-band blocking) the test scope in the originally defined in Rel-15 was extensive. Based on experiences the test scope is proposed to be minimized maintaining relevant testing. This would conserve test time in OTA test laboratories. The plan is to implement agreements in remaining part of Rel-19 conformance work phase.

- Addition of new requirement for protection of Fixed Satellite Service (FSS) UL for band n104: A new condition was defined by ITU-R for the frequency range 6425 to 7125 MHz to protect FSS UL. As a consequence, RAN4 has developed a new requirement for spatial emission towards other services. The requirement has now been included in BS RF core specification and a corresponding OTA test method have been developed for the conformance specification. The work is complete, and improvements can be handled as maintenance.

- Improvements of TRP test methods in TS 38.141-2, Annex I: The TRP test method descriptions can be split up in two parts. The test up and measurement phase is described within the section of each individual requirement in the conformance specification, while how TRP is calculated based on a collection of EIRP measurement samples is described in a separate annex (Annex I). In the annex, several integration methods are described on how to calculate the TRP for different situations. The background related to what approach to use for a specific situation is not very clear, some of the calculation methods are overlapping and some may require additional descriptions. It is expected that RAN4 can add missing information and if needed consolidate some methods to simplify the description with the intention to improve the technical quality of Annex I within the time frame of Rel-19.

- Re-evaluation of transmitter spurious emission requirement levels: The assumptions used to derive requirement levels can be traced back to Rel-99 and the development of UTRA. The evolution of 3GPP have impacted on network deployments and capability of BS and UE electronics. Therefore, RAN4 need to update some of the simulation assumptions to reflect aspects fundamental for NR. With NR RAN4 has evolved and new assumptions for models and RF characteristics is used for requirements derivation. This will have direct impact on bands in the upper region of FR1. Therefore, a request from ETSI TFES initiated work to re-evaluate requirement levels for transmitter spurious emission for some bands in upper region of FR1. The work is progressing, new assumptions are adopted, and collection of simulation results is in progress. It is expected that RAN4 will conclude the re-evaluation work within the time frame of Rel-19 RF core. The impact to Rel-19 conformance is very small.

Since the studies related to improve co-location requirements are coming close to finalization within Rel-19 it is time to start to look at the work needed to improve specifications in Rel-20. some further information related to fundamental aspect to consider in the coming Rel-20 work is presented in section 2.1.

In addition, some other work related to other areas needs to be considered to Rel-20 BS RF evolution WI. To prepare the specifications for large AAS BS array antennas (foreseen to be used in the upper region of FR1), there is an evident need to improve the specification to better support BS adopting large arrays with hybrid beamforming. There are two areas with high priority to consider:

- Introduction of new AAS BS type: The background for the new BS type is to allow for smoother transition for the industry to full OTA using BS type 1-O. More technical information will be presented in section 2.2.

- Re-evaluation of receiver blocking requirement: In parallel with the work related to the request to re-evaluate transmitter spurious emission requirement levels we started to look at in-band blocking. We noticed that the blocking level originally determined in Rel-99 may need to be re-checked using for NR more recent assumptions. This may result in new requirement levels for bands in upper region of FR1. More background is captured in section 2.3.

  - Evolution of BS-to-BS co-location requirements

In Rel-19 BS RF evolution work started to study possibilities to evolve the co-location concept originally introduced in specification during Rel-15. Now it is clear that the Rel-15 concept is suitable for NR BS type 1-O AAS BS (and corresponding MSR AAS BS) operating below 3 GHz where the Co-Location Test Antenna (CLTA) is selected according to the description in the conformance test specification. The concept was derived from the idea that the CLTA is selected as a commercially available single column passive BS antenna. Now, with support of bands up to 7125 MHz and AAS BS are using much larger antenna arrays, the requirement concept needs to be improved accordingly to guarantee site multi-vendor BS co-location and test repeatability.

In a previous contribution [3] submitted to RAN4#114-bis technical issues related to the current requirement concept for co-location requirements was listed. Based on the identified technical issues with current concept work have been initiated in Rel-19 BS RF evolution WI with the intension to initiate work to evolve the co-location requirements.

Observation 1: The current concept used for BS type 1-O co-location requirements captured in TS 38.104, Clause 4.9 is not practical for conformance testing of product operating above 3.5 GHz since the current description of the co-location reference antenna and corresponding CLTA don’t match real deployment situations. Hence, BS type 1-O is not a feasible option for AAS BS implementations above 3.5 GHz.

It can also be noticed that when BS with large frequency offset is considered, like for the case where a 900 MHz passive antenna is co-located with a 3.5 GHz AAS BS as showed in Figure 2.1-1, there will be a significant different is antenna aperture length.

# Figure 2.1-1: Example situations, co-location between low-band (non-AAS) and mid-band (AAS)

Observation 2: The height difference between to co-located base stations at different bands (900 MHz and 3.5 GHz) is far larger than what is captured in the CTLA description in current specification.

In addition to previous issues presented in [4] additional technical issues related to the relevance of current requirement concept have been identified. These issues come with the background that during last meeting a discussion emerged on the option to let each BS manufacturer design its own set of CLTA(s) per product, to resolve the issue that CLTA(s) according to the specification are not commercially available.

Today the guidance for selection of CLTA is described in the conformance test specification as listed in Table 2.1-1. If now that information is used to design a CLTA some additional technical issues will emerge.

Table 2.1-1: CLTA characteristics (TS 38.141-2)

| Parameter | In-band CLTA | Out-of-band CLTA |
| --- | --- | --- |
| Vertical radiating dimension (h) | Test object vertical radiating length ±30% | Test object vertical radiating length ±30% (Note 2) |
| Horizontal beam width | 65° ± 10° | 65° ± 10° |
| Vertical beam width | N/A | The half-power vertical beam width of the CLTA equals the narrowest declared (D.3) vertical beamwidth ±3° (Note 2) |
| Polarization (Note 3) | Match (Note 4) | Match to in-band (Note 4) |
| Conducted interface return loss | > 10 dB | > 10 dB |
| Note 1: If a multi-column or multi-band antenna is used the column closest to the NR BS shall be selected while other columns are terminated during testing. Note 2: The vertical radiating dimension definition shall be used instead of the vertical beam width definition when the test chamber dimensions limit the use of vertical beam width definition. Otherwise, the vertical beam width definition shall be used. Note 3: For BS type 1-O with dual polarization the CLTA has two conducted interfaces each representing one polarization. Note 4: Matched to the polarization of EUT antenna. |  |  |

Observation 3: It can be noticed that information provided to describe the CLTA in the specification is overlapping and contradicting. The description provides information on both vertical radiating dimension and vertical beamwidth for the out-of-band case. Both conditions cannot be met at the same time.

Observation 4: The CLTA antenna gain is indirectly specified via the horizontal beam width and the vertical radiating dimension, while the antenna efficiency is not specified. This means that large isolation between the test object and CLTA can be created by having a CLTA with poor antenna efficiency.

In the case where the CLTA following the description in TS 38.141-2, Clause 4.12 is not commercially available the remaining way-forward is to let the BS manufacturer to build a CLTA antenna according to the technical specification. For this situation where the CLTA needs to be designed we have observed the following issues:

- The technical description of the CLTA provides information on how the beamforming characteristics (3-sector coverage in azimuth and height similar as test object gives beamwidth along vertical axis). However, there is no guidance on the antenna efficiency. Hence sufficient isolation can be created between the test antenna and CLTA by poor antenna efficiency.

- If the BS manufacturer is responsible to design the CLTA, special means to increase the isolation can be used. There will differ among different BS manufacturers. This means that there is no guarantee that different BS manufacturer will use the same CLTA for conformance testing. As a result, the co-location requirement will not guarantee co-location of multiple BS from different vendors at the same site as the co-location performance will be arbitrary and not repeatable, which was the original intention with the co-location requirements.

- Defining a calibration methodology to secure that expected coupling loss (today assumed to be 30 dB) is achieved.

Observation 5: The CLTA antenna efficiency is not captured by the description in current specification.

Observation 6: Different types of CLTA from different vendors with very different characteristics would risk making the co-location requirements use-less. To guarantee robust BS-to-BS co-location at the same site, the CLTA used should be of similar type and have the same characteristics or the requirements should be based on regular OTA parameters.

Proposal 1: For BS RF in Rel-20 define a WI with clear goal to resolve open issues identified for BS type 1-O co-location requirements using outcome from Rel-19 coupling loss investigations for bands in upper region of FR1.

To be able to improve co-location requirements, RAN4 need to consider following work to be included in Rel-20:

- Establish simulation methodology to be able to evaluate near-filed coupling loss characteristics. Simulations would guarantee more relevant values and reduce extensive measurement campaign required to cover all situations.

- Decide on co-location situation, either re-use current side-by-side co-location scenario as baseline or adopt another scenario as baseline. Unlike Rel-15 (where AAS to non-AAS is considered), consider AAS to AAS co-location for some bands.

- Establish a reduced set of intra-band and inter-band cases and establish coupling loss assumption per co-location requirement.

- Conduct simulation champaign for given situations (including intra-band and inter-band).

- Conclude on coupling values based on statistical results (CDF curves). Pick relevant point on CDF as baseline for coupling loss assumption (e.g. 95%, 99% or 99.9%).

- Establish requirement background (relation to relevant requirement parameter) and per each individual requirement derive requirement levels.

Proposal 2: For co-location requirement establish requirement derivation framework based on simulations aiming towards the goal to define requirements based on regular OTA parameters testable in traditional OTA test environments.

  - Introduction of new AAS BS type

This BS type is intended for AAS BS with a large array antenna required to provide coverage operating within bands in the upper region of FR1. The new BS type will include more OTA requirements than BS type 1-H, which would allow for a smooth transition to full OTA and BS type 1-O.

In Rel-15, the concept of NR BS types was established to allow the specification to support different types of base stations for FR1 (non-AAS and AAS) and FR2 (AAS). The requirement applicability for the different base station types is different since requirements are defined at different requirement interfaces referred to as TAB and RIB. The interfaces are visualised in Figure 2.2-1.

# Figure 2.2-1: AAS BS architecture

The currently supported BS types and a potential new additional BS type (here referred to BS type 1-H-enh) are listed in Table 2.2-1.

Table 2.2-1: BS types

| BS type | Requirement interfaces | Note |
| --- | --- | --- |
| 1-C | TAB | FR1 non-AAS BS with few RF ports and passive antenna |
| 1-H | TAB, RIB | FR1 AAS BS with access to RF ports at TAB |
| 1-H-enh | TAB2, RIB | New BS type for FR1 AAS BS with access to RF ports at TAB2 |
| 1-O | RIB | FR1 AAS BS without access to RF ports |
| 2-O | RIB | FR2 AAS BS without access to RF ports |

Considering the coverage capability for AAS BS to be the same for bands in the upper region of FR1 as for mid FR1 bands would lead to larger number of antenna elements needed for AAS BS products in the upper region of FR1 compared with mid FR1 bands. Since the number of antenna elements needed will be significantly larger than the supported number of layer 1 logical ports (also referred to as baseband ports or CSI-RS ports), different types of port expansion techniques will be required to map signals from logical ports to physical antenna elements. Traditionally the port expansion is done by combining elements as vertical sub-arrays. This combination is done using passive combination networks in the Radio Distribution Network (RDN) and often in conjunction with digital port expansion between baseband and radio. For large arrays, the vertical sub-array tends be very large which would restrict the vertical steering range. Therefore, hybrid beamforming schemes are often considered to add steering capabilities in the RDN by adding analogue RF components, such as phase shifters, switched delay lines, etc.

The current BS architecture described in TS 38.104, clause 4.3 do not explicitly describe type of beamforming schemes supported. However, for electrically large arrays it is essential to define the requirement interface such that all emission generated by the electronics is captured. In the case of having active analogue components in the RDN, a requirement interface should be defined after the RDN. With this interface TAB2 all emission generated by the radio electronics will be captured at TAB2 see Figure 2.2-1.

Another aspect to consider is that with very many AAS BS transceiver branches it is essential to define a relevant requirement level per branch. For BS type 1-H, the legacy requirement for the receiver is re-used with the consequence that the receiver for a AAS BS with a large array antenna the system will be over specified, which would increase cost and power consumption without providing benefit for the operator. Therefore, it would be natural to move requirements to the OTA domain for in-band receiver requirements and in-band transmitter requirements. Similar to the transmitter, additional considerations might be needed to ensure that receiver requirements are properly defined, and relevant test framework is in place as we move towards large electrically antennas including the architecture which cover such receiver beam forming and corresponding requirement interface.

In addition, the OTA test lab capabilities in the industry are not mature for AAS BS type 1-O all OTA conformance testing. Therefore, we suggest introducing a new BS type with a requirement set consisting of in-band requirements to be specified as OTA requirement while out of band requirements are defined as conducted requirements.

The general principles for establishing a complete requirement set for the new BS type are listed below:

- All carrier centric transmitter requirements are defined OTA at RIB.

- All receiver requirements are defined OTA at RIB.

- All out-of-band transmitter unwanted emission requirements and receiver spurious emission requirements are defined conducted at TAB2.

- TDD ON/OFF power is defined conducted at TAB2.

Based on the general principles above, a complete requirement set for TS 38.104 have been created in Table 2.2-2.

Table 2.2-2: New BS type requirement applicability mapping

| Requirement | BS type |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1-C | 1-H | 1-H-enh | 1-O | 2-O |
| BS output power | 6.2 | 6.2 | NA |  |  |
| Output power dynamics | 6.3 | 6.3 | NA |  |  |
| Transmit ON/OFF power | 6.4 | 6.4 | 6.4 |  |  |
| Transmitted signal quality | 6.5 | 6.5 | NA |  |  |
| Occupied bandwidth | 6.6.2 | 6.6.2 | NA |  |  |
| ACLR | 6.6.3 | 6.6.3 | 6.6.3 |  |  |
| Operating band unwanted emissions | 6.6.4 | 6.6.4 | 6.6.4 |  |  |
| Transmitter spurious emissions | 6.6.5 | 6.6.5 | 6.6.5 |  |  |
| Transmitter intermodulation | 6.7 | 6.7 | 6.7 | NA | NA |
| Reference sensitivity level | 7.2 | 7.2 | NA |  |  |
| Dynamic range | 7.3 | 7.3 | NA |  |  |
| In-band selectivity and blocking | 7.4 | 7.4 | NA |  |  |
| Out-of-band blocking | 7.5 | 7.5 | NA |  |  |
| Receiver spurious emissions | 7.6 | 7.6 | 7.6 |  |  |
| Receiver intermodulation | 7.7 | 7.7 | NA |  |  |
| In-channel selectivity | 7.8 | 7.8 | NA |  |  |
| Performance requirements | 8 | 8 | NA |  |  |
| Radiated transmit power |  | 9.2 | 9.2 | 9.2 | 9.2 |
| OTA base station output power |  |  | 9.3 | 9.3 | 9.3 |
| OTA output power dynamics |  |  | 9.4 | 9.4 | 9.4 |
| OTA transmit ON/OFF power |  |  | NA | 9.5 | 9.5 |
| OTA transmitted signal quality |  |  | 9.6 | 9.6 | 9.6 |
| OTA occupied bandwidth |  |  | 9.7.2 | 9.7.2 | 9.7.2 |
| OTA ACLR |  | NA | NA | 9.7.3 | 9.7.3 |
| OTA Operating band unwanted emissions |  |  | NA | 9.7.4 | 9.7.4 |
| OTA transmitter spurious emission |  |  | NA | 9.7.5 | 9.7.5 |
| OTA transmitter intermodulation |  |  | NA | 9.8 | NA |
| OTA spatial emission |  | 9.9 | 9.9 | 9.9 | NA |
| OTA sensitivity |  | 10.2 | 10.2 | 10.2 | NA |
| OTA reference sensitivity level | NA |  | 10.3 | 10.3 | 10.3 |
| OTA dynamic range |  |  | 10.4 | 10.4 | NA |
| OTA in-band selectivity and blocking |  |  | 10.5 | 10.5 | 10.5 |
| OTA out-of-band blocking |  | NA | 10.6 | 10.6 | 10.6 |
| OTA receiver spurious emission |  |  | NA | 10.7 | 10.7 |
| OTA receiver intermodulation |  |  | 10.8 | 10.8 | 10.8 |
| OTA in-channel selectivity |  |  | 10.9 | 10.9 | 10.9 |
| Radiated performance requirements |  |  | 11 | 11 | 11 |

With the given general principles and example requirement set applicability, further details related to the exact mapping required for the applicability table in TS 38.104, Clause 4.6 can be worked out.

Observation 7: Based on high level principles required to define a new BS type for AAS BS there is a direct link to work package related to improvements of co-location requirements. For the new BS type to be practically relevant, work to establish relevant and practically feasible co-location requirements will be needed.

Observation 8: The new BS type (BS type 1-H enh) would require a new requirement interface (TAB2 after the RDN) to be defined.

Proposal 3: For BS RF in Rel-20, include WI to define new BS type with relevant requirement applicability and corresponding requirement interface relevant for AAS BS with large array antenna.

If a new BS type for FR1 AAS BS is defined in addition to BS type 1-H and BS type 1-O, RAN4 need to work out general principles on requirement applicability and for when different BS types are relevant, e.g. defining caps on BS types based on number of ports, number of transceivers and supported beamforming schemes for each AAS BS type, to avoid having a specification with multiple alternatives for a given design.

For the OTA receiver requirement for BS type 1-O, declarations related to the element/sub-array is needed to translate requirement levels from legacy conducted requirement levels to OTA requirements levels. With the introduction of more advanced RDN structures with fast analogue phase shifters combining several sub-arrays in more complex sub-array structures it’s not clear how the declaration defined in current specification should be done. More work is needed to clarify interfaces in relating to the RDN and the antenna array and clearly describe what type of functionality is included in the RDN.

Observation 9: OTA receiver requirements rely on declarations of the element/sub-array characteristics. The description of the RDN needs to be enhanced to support more complex beamforming schemes (e.g. hybrid beamforming including a mix of analog beamforming and digital beamforming) to provide guidance for required declarations.

  - Re-evaluation of receiver blocking requirement

The current requirement level of -43 dBm for in-band blocking was originally derived based on simulation models and assumptions originates from 1999. The technical background related to the derivation of in-band receiver blocking requirement can be traced back to UTRA and is captured in TR 25.942.

Now in Rel-20 networks are typically build in different manner as originally assumed for UTRA. For NR, the network grid is denser, UEs are using high output power and AAS BS antennas are significantly larger with dynamic beamforming. It is therefore time to re-evaluate requirement levels for the in-band blocking requirement current specified in the BS RF specification with focus on bands in the upper region of FR1. For this work RAN4 need to establish a new set of simulation assumptions reflecting for NR relevant network deployment situations and BS and UE parameters. This work resembles with the work done in Rel-19 where transmitter spurious emission requirement levels was re-evaluated based on request from ETSI TFES.

Another aspect to consider is wider bands, where multiple UEs operating within several networks will transmits at the same time within the BS receiver bandwidth. This would change the blocking statistics.

As an example, for the case where the in-band blocking power level seen per receiver branch modelled for AAS BS with the same physical antenna area for 2.1 GHz, 3.5 GHz and 7 GHz (different number of elements) the statistical outcome is shifted, as indicated in Figure 2.3-1.

# Figure 2.3-1: Indicative simulation results

The trend that the CDFs are shifted as function on AAS BS array size indicates that having one requirement level for all bands and AAS BS implementations may not be suitable for cost and power optimized implementations.

For an AAS BS, the power received per each receiver branch will depend on the number antenna size and configuration. Since the sub-array is smaller than the legacy passive antenna, less power needs to be handled by a single receiver for AAS BS. The power received by a single element/sub-array is expected to be less than the power received by the complete system.

From an OTA requirement perspective, it would also be beneficial to define the requirement based on a statistical analysis in such way that the requirement can be defined based on a EIRL level (aka. EIS level or field strength level). Then the requirement would be independent on antenna array configuration and manufacturer declarations of sub-array gain.

Observation 10: Comparing the reference (2 GHz) used for deriving the original in-band receiver blocking requirement level to an AAS BS with large array antenna (operating at 3.5 GHz or 7 GHz), shows that the CDF of interference is shifted.  This would indicate that in-band blocking requirement levels needs some further re-evaluation for bands in upper region of FR1.

Proposal 4: For BS RF in Rel-20, include a WI to re-evaluate receiver in-band blocking requirement levels with the focus on bands in the upper region of FR1. Establish relevant simulation assumptions for AAS BS operating in the upper region of FR1 and conduct simulation campaign and derive relevant requirement levels.

Regarding the receiver out-of-band blocking requirement, it was originally derived based on an interferer signal level at -15 dBm which was converted to a fixed field strength level for NR BS type 1-O. Considering bands in the upper region of FR1 it would be worth to also consider the relevance of the receiver out-of-band blocking requirement level.

If the TU budget for RAN4 BS RF evolution within Rel-20 5G-Advanced don’t allow for evolution of receiver blocking requirements it is essential to include this work package in the RAN4 6G SI.

# Conclusion

In this contribution we summarize the status of the work in the Rel-19 BS RF evolution WI (NR_BS_RF_req_evo). The work has resulted in improvements and new requirements. However, more work is needed to re-solve some remaining issues.

In 5G several new bands above 3 GHz have been added within FR1. For these bands AAS base station supporting massive MIMO is often used. It is therefore important to evolve the specification to improve AAS BS requirements for bands in the upper region of FR1, enabling usage of BS type 1-O requirements and correspond conformance testing.

We have identified 3 different areas to focus on during Rel-20 with the intention to improve BS RF requirements:

- Evolution of BS-to-BS co-location requirements to guarantee robust BS-to-BS co-location and reproducible tests.

- Introduction of new AAS BS type to provide soft transition to full OTA with BS type 1-O.

- Re-evaluation of receiver blocking requirement to guarantee relevant requirements for bands in upper region of FR1.

In this contribution we have observed following:

Observation 1: The current concept used for BS type 1-O co-location requirements captured in TS 38.104, Clause 4.9 is not practical for conformance testing of product operating above 3.5 GHz since the current description of the co-location reference antenna and corresponding CLTA don’t match real deployment situations. Hence, BS type 1-O is not a feasible option for AAS BS implementations above 3.5 GHz.

Observation 2: The height difference between to co-located base stations at different bands (900 MHz and 3.5 GHz) is far larger than what is captured in the CTLA description in current specification.

Observation 3: It can be noticed that information provided to describe the CLTA in the specification is overlapping and contradicting. The description provides information on both vertical radiating dimension and vertical beamwidth for the out-of-band case. Both conditions cannot be met at the same time.

Observation 4: The CLTA antenna gain is indirectly specified via the horizontal beam width and the vertical radiating dimension, while the antenna efficiency is not specified. This means that large isolation between the test object and CLTA can be created by having a CLTA with poor antenna efficiency.

Observation 5: The CLTA antenna efficiency is not captured by the description in current specification.

Observation 6: Different types of CLTA from different vendors with very different characteristics would risk making the co-location requirements use-less. To guarantee robust BS-to-BS co-location at the same site, the CLTA used should be of similar type and have the same characteristics or the requirements should be based on regular OTA parameters.

Observation 7: Based on high level principles required to define a new BS type for AAS BS there is a direct link to work package related to improvements of co-location requirements. For the new BS type to be practically relevant, work to establish relevant and practically feasible co-location requirements will be needed.

Observation 8: The new BS type (BS type 1-H enh) would require a new requirement interface (TAB2 after the RDN) to be defined.

Observation 9: OTA receiver requirements rely on declarations of the element/sub-array characteristics. The description of the RDN needs to be enhanced to support more complex beamforming schemes (e.g. hybrid beamforming including a mix of analog beamforming and digital beamforming) to provide guidance for required declarations.

Observation 10: Comparing the reference (2 GHz) used for deriving the original in-band receiver blocking requirement level to an AAS BS with large array antenna (operating at 3.5 GHz or 7 GHz), shows that the CDF of interference is shifted.  This would indicate that in-band blocking requirement levels needs some further re-evaluation for bands in upper region of FR1.

To progress the work to defining BS RF package for Rel-20 and considering above observations we propose following:

Proposal 1: For BS RF in Rel-20 define a WI with clear goal to resolve open issues identified for BS type 1-O co-location requirements using outcome from Rel-19 coupling loss investigations for bands in upper region of FR1.

Proposal 2: For co-location requirement establish requirement derivation framework based on simulations aiming towards the goal to define requirements based on regular OTA parameters testable in traditional OTA test environments.

Proposal 3: For BS RF in Rel-20, include WI to define new BS type with relevant requirement applicability and corresponding requirement interface relevant for AAS BS with large array antenna.

Proposal 4: For BS RF in Rel-20, include a WI to re-evaluate receiver in-band blocking requirement levels with the focus on bands in the upper region of FR1. Establish relevant simulation assumptions for AAS BS operating in the upper region of FR1 and conduct simulation campaign and derive relevant requirement levels.

# References

[1]	RP-251879, “WF for RAN4 led REL-20 topics”, RAN4 chairman (Apple)

[2] 	RP-251285, “Discussion on UE RF and BS RF proposals for Rel-20 5G-Advanced”, Ericsson

[3]	R4-2503725, “Considerations on Rel-20 BS RF package objectives”, Ericsson

[4]	R4-2402475, “NR BS RF enhancements in Rel-19”, Ericsson