# 3GPP TSG-RAN WG4 Meeting #116	R4-2510236

# Bengaluru, IN, 25-29 Aug, 2025

Agenda Item:	7.10.3.2

Source: 		Qualcomm Incorporated

Title: 	Discussion on FR1 NTN OTA test methdology

Document for:	Approval

# Introduction

In RAN4#115, the agreements and open issues for FR1 NTN OTA test method are captured in WF [1]. In this paper, we provide our views on OTA test method and performance metric for FR1 NTN devices.

# 2. Discussion

RAN4 has agreed to define 3 typical usage scenarios for IoT-NTN and NR-NTN in OTA testing discussions.

- Scenario 1: both head+hand mode and hand only mode.

- Scenario 2: hand only mode targeted for specific UE orientation usage

- Scenario 3: hand only mode targeted for arbitrary UE orientation usage

For both Scenario 1 and Scenario 3, the performance metrics are full sphere TRP and TRS. For Scenario 2, the performance metric is to be partial sphere TRP/TRS, but the details of how to define the angle range is FFS.

In [1], the following agreements were reached regarding the details of angle range.

Issue 2-1-4: Whether the partial sphere is using the =0° (top pole) as a center Agreements: the partial sphere metric is using the  =0° (top pole) as a center Issue 2-1-5: What is the angle range of the partial sphere-after concluding center of the sphere Agreements: - RAN4 further discuss the theta angle range of partial sphere metric, from ±30°, ±60°, ±90°. - Option 1: single range is selected and captured in TR - Option 2: multi-range in TR but based on UE declaration, details are FFS - Note1: the phi range should be 360 degrees - Note2: if multiple ranges are captured in TR, the metric applicability rule should also be defined. The target is selecting one for NTN scenario 2.

RAN4 needs to decide the theta angle range of partial sphere metric, from ±30°, ±60°, ±90° with the following two options:

- Option 1: single range is selected and captured in TR

- Option 2:  multi-range in TR but based on UE declaration, details are FFS

Option 2 is to specify and verify NTN UE partial TRP/TRS based on UE declared angle ranges including ±30°, ±60°, ±90°. Firstly, it would be very difficult to define multiple limits based on the candidate angle ranges (±30°, ±60°, ±90°). In RAN4, OTA requirements are derived based on the measurement campaign, if the angle range is based on the UE declaration, RAN4 will have to define OTA limits for all 3 candidate angle ranges. It is not possible to have enough commercial devices and lab resources for measurement campaigns. Therefore, we propose to go with Option 1.

Regarding the specific angle range for Option 1, since we cannot alwarys guarantee the assistant pointing software works well in real deployment, especially when UE is at the low elevation angle. In addition, RAN4 has agreed NOT to verify the UE’s performance for the assistant pointing software. In this case, it would be safer to define a relatively wider angle, i.e., ±90° in the first release of NTN OTA requirements. Therefore, we have the following proposal:

Proposal 1: RAN4 agrees to define upper hemisphere TRP and TRS (theta angle range of ±90°) as the performance metric for Scenario 2 UE in Rel-19.

In [1], the UE positioning guideline for Scenario 2 was agreed as follows.

Issue 2-1-2: alignment of understanding on UE positioning guideline for Scenario 2 Agreements: - For Scenario 2 UEs: - The DUT shall be mounted in a suitable hand phantom - The elevation tilt of the default position of the DUT can be: - 0 degree (similar to Figure 6.1-1 of TR 38.870, but with hand phantom) or as shown below - [±15, ±30, ±45] degrees; Other tilt angles are not precluded - NOTE 1: the granularity of the elevation tilt to be further discussed next meeting - The elevation tilt of the hand phantom & DUT is determined based on OEM declaration

It is not suggested to define too many candidate elevations which will make the test procedure complicated. It is recommended to define 0 degree as the default elevation tilt and UE can declare other title from [±15, ±30, ±45] degrees depending on the feasibility of hand phantom.

Proposal 2: It is recommended to define 0 degree as the default elevation tilt and UE can declare other title from {±15, ±30, ±45} degrees depending on the feasibility of hand phantom.

The current NTN devices usually use assistant pointing software to point toward the satellite to ensure successful data transmission. In the TRP/TRS testing, we need to figure out how to configure the assistant pointing software. We should first clarify whether the NTN antenna pattern is related to the orientation of assistant pointing software.

- If the antenna pattern and directivity are not related to the orientation of the assistant pointing software, then the assistant pointing software could be turned off during the test, and the UE orientation can be configured according to the UE positioning guideline.

- If the antenna pattern and directivity are related to the orientation of the assistant pointing software, then we should consider an approach similar to FR2 TRP testing, i.e., locking the antenna direction at the θ=0° (top pole) to ensure that the antenna pattern is locked during the testing.

- A test mode might be needed to disable/lock the assistant pointing software if the prerequisite for an NTN device to transmit/receive is successfully pointing to the satellite

Proposal 3: RAN4 to configure the assistant pointing software with the following considerations:

- If the antenna pattern and directivity are not related to the orientation of the assistant pointing software, then the assistant pointing software could be turned off during the test, and the UE orientation can be configured according to the UE positioning guideline.

- If the antenna pattern and directivity are related to the orientation of the assistant pointing software, then we should consider an approach similar to FR2 TRP testing, i.e., locking the antenna direction at the θ=0° (top pole) to ensure that the antenna pattern is locked during the testing.

- A test mode might be needed to disable/lock the assistant pointing software if the prerequisite for an NTN device to transmit/receive is successfully pointing to the satellite

# 3.	Conclusion

In this paper, we provide our views on OTA test method for FR1 NTN devices. The following proposals are made:

Proposal 1: RAN4 agrees to define upper hemisphere TRP and TRS (theta angle range of ±90°) as the performance metric for Scenario 2 UE in Rel-19.

Proposal 2: It is recommended to define 0 degree as the default elevation tilt and UE can declare other title from {±15, ±30, ±45} degrees depending on the feasibility of hand phantom.

Proposal 3: RAN4 to configure the assistant pointing software with the following considerations:

- If the antenna pattern and directivity are not related to the orientation of the assistant pointing software, then the assistant pointing software could be turned off during the test, and the UE orientation can be configured according to the UE positioning guideline.

- If the antenna pattern and directivity are related to the orientation of the assistant pointing software, then we should consider an approach similar to FR2 TRP testing, i.e., locking the antenna direction at the θ=0° (top pole) to ensure that the antenna pattern is locked during the testing.

- A test mode might be needed to disable/lock the assistant pointing software if the prerequisite for an NTN device to transmit/receive is successfully pointing to the satellite

# 4.	References

- R4-2508782, Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3, vivo