3GPP TSG-RAN WG4 Meeting # 116	R4-2510240

Bengaluru, MT, 25th – 29th August 2025

Agenda Item:	7.7.3

Source: 		Qualcomm Incorporated

Title: 	Coexistence simulation results between TN and NTN for Ku band

Document for:	Approval

# Introduction

In RAN4#115, NTN-TN co-existence simulation assumptions for Ku band were discussed and RAN4 agreed to consider the simulation assumptions in [1]. In this paper, we provide our coexistence simulation results between NTN and TN and vice versa for Ku band.

# 2. Discussion

In this document we provide co-existence simulation results between TN and NTN and vice versa for Ku band for the scenarios highlighted in Table 1 and as agreed in [1].

Table 1: Simulated coexistence scenarios

| No. | Combination | Aggressor | Victim | Frequency band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| Scenario 1 | TN with NTN | NTN UL | TN UL | 14 GHz | ACLR NTN UE to be varied/defined. ACS TN gNB fixed |
| Scenario 6 | TN with NTN | NTN DL | TN DL | 11 GHz | ACLR NTN SAN to be varied/defined. ACS TN UE fixed |
| Scenario 5 | TN with NTN | TN DL | NTN DL | 11 GHz | ACLR TN gNB fixed. ACS NTN UE to be varied/defined |
| Scenario 4 | TN with NTN | TN DL | NTN UL | 14 GHz | ACLR TN gNB fixed. ACS NTN SAN to be varied/defined |

In the coexistence results below we have used fixed VSAT at 1.5 m height for all the cases which can be considered as worst-case scenario. Satellite elevation angles of 35 and 90 degrees for the two satellites in Table 2 are used. Two types of VSAT antennas are considered as highlighted in Table 2.

FRF 2 is used for all the cases as agreed in [1]. Furthermore, flat ACIR model is considered for all DL and UL simulations. The number of TN UEs is 1 UE in both DL and UL. The number of NTN UEs is 1 UE in DL and 10 UEs in UL.

Table 2: Simulation Matrix

| VSAT model SAN | 45 x 45 Phased Array (1st step for coexistence) | 60 cm Parabolic (1st step for coexistence) | 78 x 78 Phased Array |
| --- | --- | --- | --- |
| GEO | X | X | X |
| LEO 600km | X | X |  |

In the following, the simulation results will show the average and cell edge throughput loss as a function of the ACIR.

- Scenario 1 (NTN UL to TN UL)

The simulation results for Scenario 1 for both elevation angles 35 and 90 degrees are shown in Table 3 considering the VSAT at 1.5 m. From the results we can see that the effect from the VSAT UEs to the TN BS is not very high and the ACIR requirements are about 20 dB under worst case assumptions which is 35 degrees elevation angle. For elevation angle 90 degrees, the VSAT is directed towards the sky and the sidelobes towards the BS are very low so lower ACIR is observed with about 5 dB less. It can be seen also that the throughput loss is decreasing with lower altitudes because the VSAT UE will use less TX power.

Table 3: Required ACIR for Scenario 1 considering VSAT Parabolic antenna.

|  |  | ACIR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 5 | 10 | 15 | 20 | 25 |
| GEO @90 deg | Average THP-Loss [%] | 2.4 | 1.5 | 0.9 | 0.5 | 0.33 | 0.18 |
|  | Cell edge THP-Loss [%] | 16.9 | 12.1 | 6.3 | 3.1 | 1.1 | 0.7 |
| GEO @35 deg | Average THP-Loss [%] | 5.1 | 3.4 | 2.2 | 1.4 | 0.9 | 0.5 |
|  | Cell edge THP-Loss [%] | 38 | 24.3 | 15.7 | 9.2 | 5.5 | 3 |
| LEO 600 @90 deg | Average THP-Loss [%] | 0.19 | 0.09 | 0.03 | 0.01 | 0.004 | 0.001 |
|  | Cell edge THP-Loss [%] | 0.88 | 0.57 | 0.08 | 0 | 0 | 0 |
| LEO 600 @35 deg | Average THP-Loss [%] | 3.2 | 2.2 | 1.5 | 0.97 | 0.6 | 0.35 |
|  | Cell edge THP-Loss [%] | 22.6 | 15.7 | 11.4 | 6.6 | 3.1 | 1.1 |

Observation 1: The required ACIR values for Scenario 1 for VSAT under worst case assumption with 35 degrees of elevation can be 20 dB and the TN BS ACS of 30 dB can be respected. Hence the ACLR of the NTN UE VSAT can be defined with 20 dB for the Parabolic antenna case.

The simulation results for Scenario 1 for both elevation angles 35 and 90 degrees are shown in Table 4 considering the VSAT at 1.5 m and phased array antenna with 45x45. From the results we can see that the effect from the VSAT using phased array antenna to the TN BS is worst compared to VSAT using parabolic antenna and stringent ACIR requirements are needed which is about 25 dB under worst case assumptions which is 35 degrees elevation angle. The effect of the sidelobes from the phased array VSAT antenna towards the TN BS is much higher compared to VSAT parabolic case, due to higher sidelobes as can be seen in Annex1.

Table 4: Required ACIR for Scenario 1 considering VSAT Phased array antenna (45x45).

|  |  | ACIR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 5 | 10 | 15 | 20 | 25 |
| GEO @90 deg | Average THP-Loss [%] | 6.13 | 4.1 | 2.7 | 1.7 | 1.1 | 0.6 |
|  | Cell edge THP-Loss [%] | 42 | 28.2 | 19.1 | 12.9 | 7.3 | 3.4 |
| GEO @35 deg | Average THP-Loss [%] | 7.6 | 5.4 | 3.7 | 2.5 | 1.6 | 1.1 |
|  | Cell edge THP-Loss [%] | 62.7 | 42.9 | 26.9 | 16.8 | 11.6 | 6.6 |
| LEO 600 @90 deg | Average THP-Loss [%] | 0.65 | 0.38 | 0.2 | 0.09 | 0.04 | 0.01 |
|  | Cell edge THP-Loss [%] | 3.1 | 2 | 0.95 | 0.57 | 0.08 | 0 |
| LEO 600 @35 deg | Average THP-Loss [%] | 5.1 | 3.6 | 2.5 | 1.7 | 1.1 | 0.7 |
|  | Cell edge THP-Loss [%] | 41.3 | 26.8 | 17.7 | 12.3 | 7.2 | 4.4 |

Observation 2: The required ACIR values for Scenario 1 for VSAT with phased array antenna under worst case assumption with 35 degrees of elevation can be 25 dB and the TN BS ACS of 30 dB can still be respected. The ACLR of the NTN UE VSAT can be defined with 25 dB for the phased array antenna case.

The simulation results for scenario 1 for both elevation angles 35 and 90 degrees are shown in Table 5 considering the VSAT at 1.5 m and phased array antenna with 78x78 for the GEO case only. The results are slightly better in the 90-degree case compared to the 45x45 case, due to smaller side lobes. However, it is worst in the 35-degree case compared to the 45x45 because of the increased EIRP.

Table 5: Required ACIR for Scenario 1 considering VSAT Phased array antenna (78x78).

|  |  | ACIR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 5 | 10 | 15 | 20 | 25 | 30 |
| GEO @90 deg | Average THP-Loss [%] | 3.6 | 2.4 | 1.6 | 1 | 0.6 | 0.4 | - |
|  | Cell edge THP-Loss [%] | 26.2 | 16.8 | 11.3 | 6 | 3.3 | 2.2 | - |
| GEO @35 deg | Average THP-Loss [%] | 11.6 | 8.7 | 6.3 | 4.4 | 3 | 2 | 1.3 |
|  | Cell edge THP-Loss [%] | 88.2 | 71.6 | 49.5 | 33.5 | 20.1 | 13.9 | 8 |

Observation 3: The ACIR values for Scenario 1 for VSAT with phased array antenna 78x78 for GEO under worst case assumption with 35 degrees of elevation is 30dB. With 90degree of elevation, the ACIR is 15dB. In the real development, the interference with 78*78 from NTN UL to TN UL would be smaller than that with 45*45 case, due to smaller side lobes.

Proposal 1: Based on the observations 1, and 2, the phased array antenna case should consider stringent requirements compared to parabolic antenna case.

Proposal 2: Based on the observations 1, and 2, the ACLR for NTN UE VSAT is 20 dB and 25 dB for parabolic and phased array antenna respectively.

Proposal 3: Based on observation 2, and 3, the GEO and LEO VSAT types can be defined a unified ACLR requirements.

- Scenario 6 (NTN DL to TN DL)

The simulation results for Scenario 6 for both elevation 35 and 90 degrees are shown in Table 6 considering VSAT phased array and parabolic antennas. From the results we can see that the effect from the satellite to the TN UE is negligible because of the very good signal level from the BS compared to the satellite interference.

Table 6: Required ACIR for Scenario 6 considering VSAT phased array and parabolic antennas.

| For elevation 35 and 90 degrees |  | ACIR |
| --- | --- | --- |
|  |  | 0 |
| GEO | Average THP-Loss [%] | Less than 0.1 |
|  | Cell edge THP-Loss [%] | Less than 0.1 |
| LEO 600 | Average THP-Loss [%] | Less than 0.1 |
|  | Cell edge THP-Loss [%] | Less than 0.1 |

Observation 4: The interference from NTN DL to TN DL for Scenario 6 is negligible for all the satellite altitudes and at different elevation angles. The TN UE ACS of 24 dB can be respected. Hence the ACLR of the NTN Satellite can be defined with even less value than 24 dB or based on other scenarios.

Proposal 4: Based on observation 4, the ACLR of the satellite can be defined with value less than 24 dB.  It is proposed to define ACLR of GEO/LEO as 14dBc comparing with Ka-band ACLR,

- Scenario 5 (TN DL to NTN DL)

The simulation results for scenario 5 for both elevation angles 35 and 90 degrees are shown in Table 7. From the results we can see that the adjacent interference from the TN BS to the VSAT UE is very high. Furthermore, decreasing the elevation angle to 35 degree show worse results due to bad coverage with low elevation. The “NaN” in the cell edge throughput for 35-degree elevation angle represents out of coverage and that even without TN interference, there is no link and 50% or more of the NTN UEs fails to meet the DL like budget.

Table 7: Required ACIR for Scenario 5 considering VSAT Parabolic antenna.

|  |  | ACIR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 | 20 | 25 | 30 | 35 | 40 |
| GEO @90 deg | Average THP-Loss [%] | 28.2 | 18.1 | 11.1 | 6.4 | 3.3 | 1.5 |
|  | Cell edge THP-Loss [%] | 95.6 | 82.8 | 62.4 | 38.8 | 5.7 | 3.9 |
| GEO @35 deg | Average THP-Loss [%] | 46.3 | 33.5 | 23 | 14.7 | 8.5 | 4.4 |
|  | Cell edge THP-Loss [%] | NAN | NAN | NAN | NAN | NAN | NAN |
| LEO 600 @90 deg | Average THP-Loss [%] | 29.2 | 18.8 | 11.4 | 6.6 | 3.4 | 1.5 |
|  | Cell edge THP-Loss [%] | 95.7 | 83.9 | 66.3 | 42.6 | 9.2 | 6 |
| LEO 600 @35 deg | Average THP-Loss [%] | 50 | 36.8 | 25.9 | 17.3 | 10.5 | 5.8 |
|  | Cell edge THP-Loss [%] | NAN | NAN | NAN | NAN | NAN | NAN |

Based on the results in Table 7, a high ACIR (e.g., 40 dB) is required to meet the target criteria. However, this specific scenario warrants careful consideration. The main reason for the high ACIR requirement is the very weak NTN link relative to the adjacent channel interference from the TN. Under the current simulation assumptions, even in the absence of TN interference, the NTN performance is already poor, making the system highly sensitive to additional interference. At high elevation angles (e.g., 90 degrees), NTN performance improves but remains vulnerable to strong TN interference, especially in co-located urban deployments. In such cases, imposing a stringent adjacent channel coexistence requirement may not be effective in ensuring the desired overall performance. In summary, when evaluating the results for Scenario 5, several factors must be considered — including the likelihood of encountering similar geometrical conditions (i.e., a very weak NTN link and a very strong adjacent TN link) in realistic VSAT operations.

Observation 5: The required ACIR value for scenario 5 is 40 dB considering different elevation angles for VSAT using parabolic antenna which is exceeding the TN BS ACLR of 37 dB. The main reason for the high ACIR requirement is the very weak NTN link relative to the adjacent channel interference from the TN. Under the current simulation assumptions, even in the absence of TN interference, the NTN performance is already poor, making the system highly sensitive to additional interference. Imposing a stringent adjacent channel coexistence requirement may not be effective in ensuring the desired overall performance.

Proposal 5: Based on observation 5, in case that cell edge throughput is “NaN” due to the out of coverage, RAN4 can conclude based on average throughput loss.

The simulation results for Scenario 5 for both elevation angles 35 and 90 degrees are shown in Table 8 considering the VSAT phased array antenna. From the results we can see that the effect from the TN BS to the VSAT with phased array antenna is worse than the VSAT with parabolic antenna case with about 5 dB more stringent ACIR.

Table 8: Required ACIR for Scenario 5 considering VSAT Phased array antenna (45x45).

|  |  | ACIR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| GEO @90 deg | Average THP-Loss [%] | 41.8 | 27.9 | 17.6 | 10.6 | 6 | 3.1 | 1.3 |
|  | Cell edge THP-Loss [%] | 100 | 100 | 82.9 | 56.6 | 28 | 5.5 | 0.6 |
| GEO @35 deg | Average THP-Loss [%] | 50.9 | 37.9 | 27.4 | 19.1 | 12.6 | 8 | 4.8 |
|  | Cell edge THP-Loss [%] | NAN | NAN | NAN | NAN | NAN | NAN | NAN |
| LEO 600 @90 deg | Average THP-Loss [%] | 43.5 | 29.2 | 18.4 | 11.2 | 6.4 | 3.3 | 1.4 |
|  | Cell edge THP-Loss [%] | 100 | 100 | 85.2 | 63.7 | 30.6 | 10 | 2.8 |
| LEO 600 @35 deg | Average THP-Loss [%] | 54 | 40.5 | 29.1 | 20.2 | 13.1 | 8.5 | 5.2 |
|  | Cell edge THP-Loss [%] | NAN | NAN | NAN | NAN | NAN | NAN | NAN |

Observation 6: The ACIR value for scenario 5 is 45 dB considering different elevation angles for VSAT using phased array antenna which is exceeding the TN BS ACLR of 37 dB. Similarly, the main reason for the high ACIR requirement is the very weak NTN link relative to the adjacent channel interference from the TN. Under the current simulation assumptions, even in the absence of TN interference, the NTN performance is already poor, making the system highly sensitive to additional interference. Imposing a stringent adjacent channel coexistence requirement may not be effective in ensuring the desired overall performance.

The simulation results for Scenario 5 for both elevation angles 35 and 90 degrees are shown in Table 9 considering the VSAT phased array antenna with 78x78 GEO case. The results of 45x45 case are worst compared to 78x78 due to increased EIRP in 78x78 case.

Table 9: Required ACIR for Scenario 5 considering VSAT Phased array antenna (78x78).

|  |  | ACIR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| GEO @90 deg | Average THP-Loss [%] | 25.9 | 18.2 | 12.3 | 7.9 | 4.8 | 2.5 | 1.2 |
|  | Cell edge THP-Loss [%] | 100 | 93.7 | 82.7 | 59.6 | 35 | 8 | 3.5 |
| GEO @35 deg | Average THP-Loss [%] | 49.5 | 37 | 26 | 17.1 | 10.7 | 6.2 | 3.4 |
|  | Cell edge THP-Loss [%] | NAN | NAN | NAN | NAN | NAN | NAN | NAN |

Observation 7: For scenario 5, 78x78 case shows similar results as 45x45 case with up to 45 dB ACIR.

Proposal 6: When deriving VSAT ACS, several factors should be considered including the likelihood of encountering similar geometrical conditions (i.e., a very weak NTN link and a very strong adjacent TN link) in realistic VSAT operations. The VSAT ACS can be defined as [26-30]dB, and the justification of defining a more relaxed value compared with coexistence simulations should be captured in the TR.

- Scenario 4 (TN DL to NTN UL)

The simulation results for Scenario 4 for both elevation angles 35 and 90 degrees are shown in Table 10 and Table 11 considering VSAT with parabolic antenna and phased array antenna respectively. From the results we can see that the effect from the TN BS to the satellite is not very high for the elevation angle of 90 degrees for all the satellite altitudes. However, the effect increases with decreasing the elevation angle and stringent ACIR values are required up to 30 dB.

Table 10: Required ACIR for Scenario 4 considering VSAT Parabolic antenna.

|  |  | ACIR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 |
| GEO @90 deg | Average THP-Loss [%] | 6.2 | 2.5 | 0.9 | 0.3 | 0.09 | 0.03 |
|  | Cell edge THP-Loss [%] | 14 | 6.4 | 3.1 | 0.7 | 0.5 | 0.16 |
| GEO @35 deg | Average THP-Loss [%] | 20 | 11.6 | 6.3 | 3 | 1.3 | 0.4 |
|  | Cell edge THP-Loss [%] | NaN | NaN | NaN | NaN | NaN | NaN |
| LEO 600 @90 deg | Average THP-Loss [%] | 18.9 | 9.5 | 4.2 | 1.6 | 0.5 | 0.18 |
|  | Cell edge THP-Loss [%] | 66.9 | 44.3 | 23.4 | 10.3 | 3.7 | 1.2 |
| LEO 600 @35 deg | Average THP-Loss [%] | 35.1 | 22.1 | 13 | 7.1 | 3.5 | 1.5 |
|  | Cell edge THP-Loss [%] | 100 | 100 | 52.2 | 23.2 | 15.9 | 4.2 |

Table 11: Required ACIR for Scenario 4 considering VSAT Phased array antenna (45x45).

|  |  | ACIR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 |
| GEO @90 deg | Average THP-Loss [%] | 6.3 | 2.5 | 0.9 | 0.3 | 0.09 | 0.03 |
|  | Cell edge THP-Loss [%] | 14 | 6.4 | 3.1 | 0.7 | 0.5 | 0.16 |
| GEO @35 deg | Average THP-Loss [%] | 22.7 | 13.3 | 7.4 | 3.7 | 1.6 | 0.6 |
|  | Cell edge THP-Loss [%] | NaN | NaN | NaN | NaN | NaN | NaN |
| LEO 600 @90 deg | Average THP-Loss [%] | 18.9 | 9.5 | 4.2 | 1.6 | 0.5 | 0.18 |
|  | Cell edge THP-Loss [%] | 66.9 | 44.3 | 23.4 | 10.3 | 3.7 | 1.2 |
| LEO 600 @35 deg | Average THP-Loss [%] | 34.6 | 21.8 | 12.8 | 7 | 3.4 | 1.5 |
|  | Cell edge THP-Loss [%] | NaN | NaN | NaN | NaN | NaN | NaN |

Observation 8: The ACIR values for Scenario 4 are between 15 to 30 dB for different satellite altitudes and different elevation angles. The TN BS ACLR of 30 dB can be respected. Hence the ACS of the satellite can be defined with 30 dB. Furthermore, the system parameters for the elevation angle at 35 degrees need to be reconsidered to avoid the out of coverage for UL with GEO.

The simulation results for scenario 4 for both elevation angles 35 and 90 degrees for the 78x78 GEO case is shown in Table 12. From the results we can see that the results are better than the 45x45 case due to higher EIRP from the NTN VSAT terminal.

Table 12: Required ACIR for Scenario 4 considering VSAT Phased array antenna (78x78).

|  |  | ACIR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 |
| GEO @90 deg | Average THP-Loss [%] | 5.4 | 2.1 | 0.7 | 0.26 | 0.08 | 0.02 |
|  | Cell edge THP-Loss [%] | 27.8 | 12.3 | 4.5 | 2 | 0.6 | 0.2 |
| GEO @35 deg | Average THP-Loss [%] | 18 | 10.3 | 5.5 | 2.6 | 1 | 0.4 |
|  | Cell edge THP-Loss [%] | NaN | NaN | NaN | NaN | NaN | NaN |

Observation 9: For scenario 4, 78x78 case shows better results compared to 45x45 case.

Proposal 7: Based on observation 8 and 9, the ACS of the satellite can be defined as 30dB for LEO, and 20dB for GEO.

Proposal 8: Summary of proposed ACLR and ACS for Ku-band is given as below table:

|  | SAN |  | VSAT |  |
| --- | --- | --- | --- | --- |
|  | GEO | LEO | Parabolic | Phased array |
| ACLR (dBc) | 14 | 14 | 20 | 25 |
| ACS (dBc) | 20 | 30 | [26-30] | [26-30] |

# 3.	Conclusion

The summary of the required ACIR for every scenario is summarized in Table 13.

Table 13: Required ACIR values for all the scenarios

| Scenario | Satellite | Antenna Type | ACIR Elevation 90 deg | ACIR Elevation 35 deg |
| --- | --- | --- | --- | --- |
| Scenario 1 NTN UL aggressor TN UL victim VSAT ACLR to be defined | GEO | Phase array 45x45 | 25 | 25 |
|  |  | Phase array 78x78 | 15 | 30 |
|  |  | Parabolic | 15 | 20 |
|  | LEO | Phase array | 0 | 25 |
|  |  | Parabolic | 0 | 20 |
| Scenario 6 TN DL aggressor NTN UL victim SAN ACS to be defined | GEO | Phase array 45x45 | 15 | 20 |
|  |  | Phase array 78x78 | 15 | 15 |
|  |  | Parabolic | 15 | 20 |
|  | LEO | Phase array | 25 | 25 |
|  |  | Parabolic | 25 | 30 |
| Scenario 5 TN DL aggressor NTN DL victim VSAT ACS to be defined | GEO | Phase array | 40 | 45 |
|  |  | Parabolic | 40 | 40 |
|  | LEO | Phase array | 45 | 45 |
|  |  | Parabolic | 40 | 40 |
| Scenario 4 NTN DL aggressor TN DL victim SAN ACLR to be defined | GEO | Phase array | 0 | 0 |
|  |  | Parabolic | 0 | 0 |
|  | LEO | Phase array | 0 | 0 |
|  |  | Parabolic | 0 | 0 |

In this paper, we provided a coexistence simulation result between TN and NTN and vice versa for both elevation angles of 35 and 90 degrees considering both NTN VSAT parabolic and phases array antennas. We have made the following observations as shown below:

Observation 1: The required ACIR values for Scenario 1 for VSAT under worst case assumption with 35 degrees of elevation can be 20 dB and the TN BS ACS of 30 dB can be respected. Hence the ACLR of the NTN UE VSAT can be defined with 20 dB for the Parabolic antenna case.

Observation 2: The required ACIR values for Scenario 1 for VSAT with phased array antenna under worst case assumption with 35 degrees of elevation can be 25 dB and the TN BS ACS of 30 dB can still be respected. The ACLR of the NTN UE VSAT can be defined with 25 dB for the phased array antenna case.

Observation 3: The ACIR values for Scenario 1 for VSAT with phased array antenna 78x78 for GEO under worst case assumption with 35 degrees of elevation is 30dB. With 90degree of elevation, the ACIR is 15dB. In the real development, the interference with 78*78 from NTN UL to TN UL would be smaller than that with 45*45 case, due to smaller side lobes.

Observation 4: The interference from NTN DL to TN DL for Scenario 6 is negligible for all the satellite altitudes and at different elevation angles. The TN UE ACS of 24 dB can be respected. Hence the ACLR of the NTN Satellite can be defined with even less value than 24 dB or based on other scenarios.

Observation 5: The required ACIR value for scenario 5 is 40 dB considering different elevation angles for VSAT using parabolic antenna which is exceeding the TN BS ACLR of 37 dB. The main reason for the high ACIR requirement is the very weak NTN link relative to the adjacent channel interference from the TN. Under the current simulation assumptions, even in the absence of TN interference, the NTN performance is already poor, making the system highly sensitive to additional interference. Imposing a stringent adjacent channel coexistence requirement may not be effective in ensuring the desired overall performance.

Observation 6: The ACIR value for scenario 5 is 45 dB considering different elevation angles for VSAT using phased array antenna which is exceeding the TN BS ACLR of 37 dB. Similarly, the main reason for the high ACIR requirement is the very weak NTN link relative to the adjacent channel interference from the TN. Under the current simulation assumptions, even in the absence of TN interference, the NTN performance is already poor, making the system highly sensitive to additional interference. Imposing a stringent adjacent channel coexistence requirement may not be effective in ensuring the desired overall performance.

Observation 7: For scenario 5, 78x78 case shows similar results as 45x45 case with up to 45 dB ACIR.

Observation 8: The ACIR values for Scenario 4 are between 15 to 30 dB for different satellite altitudes and different elevation angles. The TN BS ACLR of 30 dB can be respected. Hence the ACS of the satellite can be defined with 30 dB. Furthermore, the system parameters for the elevation angle at 35 degrees need to be reconsidered to avoid the out of coverage for UL with GEO.

Observation 9: For scenario 4, 78x78 case shows better results compared to 45x45 case.

Based on the above observations we have the following proposals:

Proposal 1: Based on the observations 1, and 2, the phased array antenna case should consider stringent requirements compared to parabolic antenna case.

Proposal 2: Based on the observations 1, and 2, the ACLR for NTN UE VSAT is 20 dB and 25 dB for parabolic and phased array antenna respectively.

Proposal 3: Based on observation 2, and 3, the GEO and LEO VSAT types can be defined a unified ACLR requirements.

Proposal 4: Based on observation 4, the ACLR of the satellite can be defined with value less than 24 dB.  It is proposed to define ACLR of GEO/LEO as 14dBc comparing with Ka-band ACLR,

Proposal 5: Based on observation 5, in case that cell edge throughput is “NaN” due to the out of coverage, RAN4 can conclude based on average throughput loss.

Proposal 6: When deriving VSAT ACS, several factors should be considered including the likelihood of encountering similar geometrical conditions (i.e., a very weak NTN link and a very strong adjacent TN link) in realistic VSAT operations. The VSAT ACS can be defined as [26-30]dB, and the justification of defining a more relaxed value compared with coexistence simulations should be captured in the TR.

Proposal 7: Based on observation 8 and 9, the ACS of the satellite can be defined as 30dB for LEO, and 20dB for GEO.

Proposal 8: Summary of proposed ACLR and ACS for Ku-band is given as below table:

|  | SAN |  | VSAT |  |
| --- | --- | --- | --- | --- |
|  | GEO | LEO | Parabolic | Phased array |
| ACLR (dBc) | 14 | 14 | 20 | 25 |
| ACS (dBc) | 20 | 30 | [26-30] | [26-30] |

# 4.	References

- R4-2508773, Way Forward for [115][313] NR_NTN_Ku_Band_Coexistene , Moderator Intelsat

# 5. Annex 1

In Figure 1 below, the gain is plotted as a function of the elevation angle for both the phased array antenna and the parabolic antenna. For the phased array antenna, the 45x45 configuration is used with 0 deg direction/pointing to the sky and compared with the parabolic antenna with 60 cm diameter. For both cases, the maximum EIRP is 76.6 dBm however the sidelobes in the phased array case is much stronger. That explains why with phased array case, the results are more stringent in some cases as highlighted in the results section above.

Figure 1: VSAT Parabolic antenna vs Phased array antenna pattern