3GPP TSG-RAN WG4 Meeting # 116 	R4-2510248

Bengaluru, India, August 25th – 29th, 2025

Source: 	vivo

Title: 	Discussion on the RF requirement of AIoT device

Agenda Item:	7.22.3.2

Document for:	Approval

- Introduction

In [1], good progress was made on RF requirement of AIoT device and we further provide our views on the remaining issues.

- Discussion

2.1 Transmit output power

- Performance metric: EIRP

  - FFS on how to determine device’s peak antenna gain direction

  - FFS on whether to measure multiple directions

- Take [10] dB backscatter loss for OOK as starting point, based on the assumption that the incoming CW signal and the backscatter signal are aligned with the peak gain direction of the device antenna.

  - FFS for BPSK

- The power of the backscattered signal shall include only the 1st lower sideband and the 1st upper sideband, excluding the carrier itself.

In [1], we agreed that:

There are mainly two methods to get the peak antenna gain direction: peak search and device declaration. Considering the AIoT device is cheap and sensitive to the cost, the declaration could be a better choice.

Proposal 1: The peak antenna gain direction is declared by device.

Another issue here is whether multiple direction is needed to be verified. In [2], we agreed that the backscatter loss is defined as:

Minimum Backscattering power at device antenna= Input CW Tx power at device antenna-Backscattering loss

The backscatter loss can be divided into two parts. The first part is from the circuit inside, for example, RF component insertion loss, reflection coefficient due to the impedance mismatch, but all these factors should be insensitive to the direction.

Another part is the antenna gain. Both CW power and D2R power are described as power at device antenna, but from the test perspective the D2R power include the device antenna gain while the CW power doesn’t. It means the backscatter loss would benefit from antenna gain twice based on the equation above, and the backscatter loss from the direction that has better antenna gain will be smaller. So, the backscatter loss test results from different direction mainly reflect the antenna pattern of device.

Observation 1: The backscatter loss test results from different direction mainly reflect the antenna pattern of device.

Similar discussion also happens in Rx requirement, e.g., device sensitivity, but from device perspective, the antenna pattern is same for both Rx and Tx and it would burden device if we decide verify the antenna pattern in both Tx requirement and Rx requirement.

Proposal 2: It is unnecessary to evaluate the device antenna pattern in both Tx requirement (backscatter loss from multiple direction) and Rx requirement (reference sensitivity from multiple direction).

In the last meeting, two step backscatter loss is proposed by some companies since the loss become larger when incident CW power is increased. We suggest only define two backscatter loss with certain incident CW power as side condition in the spec to avoid burden the device in the test.

Proposal 3: Only two backscatter loss with corresponding certain maximum incident CW power level as side condition will be defined in the spec.

2.2 SFO requirement

As we discussed in the last meeting, the SFO can be verified implicitly in other RF requirements, e.g., emission requirements. Since the guard band of D2R will count the SFO and if the SFO is larger than 10%, the device cannot pass the emission requirements. However, it could be clearer if we list this requirement explicitly in the spec.

Proposal 4: The 10% SFO can be explicitly captured in the spec.

2.3 SEM requirements

- Use EIRP as performance metric

  - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing

- The applicable frequency range for SEM: from the CBW edge to [OOB boundary]

- Define flat requirements for SEM as starting point, [9dBc~14dBc] based on achievable harmonic performance

- Define requirements applicable to all CBWs, FFS on how to reduce the test burden.

- Use the [maximum] CW input power level, FFS on the value.

For SEM, we agreed that [1]:

The out of band emission is described as the emission due to the modulation process but exclude spurious emission. For OOK, the main emission due to the modulation process is its harmonic. The harmonic can be analyzed based on the Fourier series expansion:

For 3rd harmonic, the amplitude will be 1/3 of main sidelobe (i.e., D2R power defined in previous part), and the power will be around 9.5 dBc lower than D2R power. It is noted that the measurement bandwidth should be same as D2R transmission bandwidth for this value. Considering the SEM is commonly defined as single side, both requirement and measurement bandwidth should be half of analysis above.

Proposal 5: The flat requirement for SEM is set to 12 dBc with the measurement bandwidth equal to 0.5*D2R transmission bandwidth.

2.4 Spurious emission requirements

- Use EIRP as performance metric

  - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing

- OOB boundary

  - Option 1: D2R channel bandwidth plus minimum D2R channel bandwidth

  - Option 2: max of 500kHz and 10 times NB where NB equals to D2R CBW

  - Other options are not precluded

- Define requirements applicable to all CBWs, FFS on how to reduce the test burden.

- Use the [maximum] CW input power level, FFS on the value.

For spurious, we agreed that:

In ITU-R SM.329, the boundary of spurious is defined as below:

“According to the principles stated in Appendix 3 to the RR, the spurious domain generally consists of frequencies separated from the centre frequency of the emission by 250% or more of the necessary bandwidth of the emission”

However, for narrow bandwidth, ITU provide a relaxed alternative:

“Narrow-band analogue modulated systems of the land mobile service, with output power higher than 1 W and operated above 30 MHz, and digitally modulated systems of the land mobile service, although generally providing a good spectrum efficiency, are unable to comply with the Category B limits for close-in frequencies due to the wideband noise generated by such systems. It is therefore necessary to provide generic steps of reference bandwidth in order to produce suitable transition area for the spectral density.”

Figure 1 Illustration in ITU-R

The necessary bandwidth is defined as:

“For a given class of emission, the width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions.”

Obviously, the necessary bandwidth in AIoT is the D2R CBW, and we prefer use 10*NB which could be easier for device and also align with ITU-R.

Proposal 6: The OOB boundary is set at max of 500kHz and 10 times NB where NB equals to D2R CBW

The out-of-band emission requirement is defined as power difference so the CW actually does not matter, but the spurious is the absolute power limit and the CW power need to be set carefully. As we analyzed in previous part, the power relationship of the D2R signal harmonic can be summarized as:

Relative power of Nthharmonic=10*log10(1N2)

For device, all D2R CBWs are listed in table below:

| D2R channel bandwidth (kHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D2R transmission Bandwidth (kHz) | Small frequency shift (kHz) |  |  |  |  |  |  |  |  |
|  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |
| 15 | 16.5 | 24.75 | 41.25 | 74.25 | 140.25 | 272.25 | 536.25 | 1064.25 |  |
| 30 |  | 33 | 49.5 | 82.5 | 148.5 | 280.5 | 544.5 | 1072.5 |  |
| 60 |  |  | 66 | 99 | 165 | 297 | 561 | 1089 |  |
| 120 |  |  |  | 132 | 198 | 330 | 594 | 1122 |  |
| 240 |  |  |  |  | 264 | 396 | 660 | 1188 |  |
| 480 |  |  |  |  |  | 528 | 792 | 1320 |  |
| 960 |  |  |  |  |  |  | 1056 | 1584 |  |
| 2880 |  |  |  |  |  |  |  |  | 3168 |

When the OOB boundary is set at max of 500kHz and 10 times D2R CBW, the boundary of red D2R CBW is 500kHz and the boundary of blue D2R CBW is 10*D2R CBW. The following table list the maximum D2R power for each D2R CBW. It is noted that for simplicity, we covert the first harmonic that fall out of the OOB boundary to 100kHz measurement bandwidth.

| D2R channel bandwidth (kHz) and corresponding D2R power (dBm) that comply to -36 dBm/100kHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D2R transmission Bandwidth (kHz) | Small frequency shift (kHz) |  |  |  |  |  |  |  |  |
|  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |
| 15 | 16.5(-4.7) | 24.75(-10.7) | 41.25(-16.4) | 74.25 (-20) | 140.25(-20.8) | 272.25(-20.8) | 536.25(-20.8) | 1064.25(-20.8) |  |
| 30 |  | 33(-7.7) | 49.5(-13.4) | 82.5 (-16.3) | 148.5(-17) | 280.5(-17.8) | 544.5(-17.8) | 1072.5(-17.8) |  |
| 60 |  |  | 66 (-8.1) | 99 (-11.4) | 165(-13.3) | 297(-14) | 561(-14.8) | 1089(-14.8) |  |
| 120 |  |  |  | 132 (-5.9) | 198(-8.4) | 330(-10.3) | 594(-11) | 1122(-11.8) |  |
| 240 |  |  |  |  | 264(-3) | 396(-5.4) | 660(-7.2) | 1188(-8) |  |
| 480 |  |  |  |  |  | 528(0) | 792(-2.4) | 1320(-4.3) |  |
| 960 |  |  |  |  |  |  | 1056(3) | 1584(0.6) |  |
| 2880 |  |  |  |  |  |  |  |  | 3168(7.8) |

Observation 2: To comply with ITU-R regulation, the maximum D2R power should be -20 dBm, and the worst case is the small frequency shift = 480kHz with 15kHz 2SB transmission bandwidth.

Use 10 dB backscatter loss as baseline, and CW input power should be -10 dBm and only test worst case should be enough.

Proposal 7: The CW power for spurious and SEM is set to -10 dBm, and only SFS = 480kHz with 15kHz 2SB transmission bandwidth need to be tested.

  - Reference sensitivity

- Using [-34dBm] as starting point to define REFSENS requirements, based on the peak gain direction of device antenna.

- Consider to define orientation requirements with following options:

  - Option 1: min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Xth percentage

  - Option 2: spatial averaging EIS

  - Option 3: EIS in the peak direction

- Testing metric: 10% miss detection rate, FFS on whether to consider false alarm

For sensitivity, we agreed that [1]:

The false alarm means that when reader transmit nothing but the device sends the correct response. From device perspective, this probability should be very low since the device need transmit D2R with correct information at correct timing rather than just response whether the R2D is received or not, so we think it is unnecessary to consider false alarm as performance metric.

Proposal 8: The false alarm is not considered as the performance metric.

As for the three options for sensitivity here, option 1 need to test all grid over the whole sphere which is quite time consuming and would heavily burden the device cost. Option 2 is similar as option1, and it is lack of physical meaning to do such average under current assumption, since the CW source actually changed for each test point.

Observation 3: Both option 1 and option 2 need to perform whole sphere scan which is time consuming and will burden on device.

The option 3 is the easiest way for device, but we also understand the intention of the option 1&option 2 from companies since in practice the direction of device best performance couldn’t always toward the reader. In RFID performance test [3], a test for similar intention is named as orientation tolerance, as shown in below:

Figure II Free space read range vs. azimuth and elevation angles [3]

Device typically use dipole antenna and the gain degradation should be slow when the direction is deviated from gain peak. As our simulation results shown in [4], the degradation at 50% CDF is around 1.5 dB, so it could be not necessary to test too much points. To address the companies’ concern on performance stability and also try to alleviate the burden on device, we propose use minimum sensitivity over partial sphere as the additional performance metric for Rx.

Figure III Illustration of partial sphere

Figure IV shows the antenna gain degradation for example dipole antenna

Figure IV antenna gain degradation of example dipole antenna

Under 45° offset, the maximum degradation within partial sphere is around 4.5 dB and under 60° offset, the degradation is around 7 ~8dB. When the partial sphere close to the concave point of dipole antenna pattern, the antenna gain will be increase rapidly and will require more accurate measurement which may put additional burden on the device, so we suggest use 45° offset with 5.5 dB degradation as the requirement.

Proposal 9: Use minimum sensitivity over partial sphere as the additional requirement, 45° sphere with 5.5 dB degradation compared to the peak EIS.

- Conclusion

In this contribution, we provide our analysis for the remaining issue of ambient IoT device.

Proposal 1: The peak antenna gain direction is declared by device.

Observation 1: The backscatter loss test results from different direction mainly reflect the antenna pattern of device.

Proposal 2: It is unnecessary to evaluate the device antenna pattern in both Tx requirement (backscatter loss from multiple direction) and Rx requirement (reference sensitivity from multiple direction).

Proposal 3: Only two backscatter loss with corresponding certain maximum incident CW power level as side condition will be defined in the spec.

Proposal 4: The 10% SFO can be explicitly captured in the spec.

Proposal 5: The flat requirement for SEM is set to 12 dBc with the measurement bandwidth equal to 0.5*D2R transmission bandwidth.

Proposal 6: The OOB boundary is set at max of 500kHz and 10 times NB where NB equals to D2R CBW

Observation 2: To comply with ITU-R regulation, the maximum D2R power should be -20 dBm, and the worst case is the small frequency shift = 480kHz with 15kHz 2SB transmission bandwidth.

Proposal 7: The CW power for spurious and SEM is set to -10 dBm, and only SFS = 480kHz with 15kHz 2SB transmission bandwidth need to be tested.

Proposal 8: The false alarm is not considered as the performance metric.

Observation 3: Both option 1 and option 2 need to perform whole sphere scan which is time consuming and will burden on device.

Proposal 9: Use minimum sensitivity over partial sphere as the additional requirement, 45° sphere with 5.5 dB degradation compared to the peak EIS.

- References

- R4-2508116, WF on A-IOT device requirements, CMCC

- R4-2505230, WF on A-IOT device requirements, CMCC

- Tag Performance Parameters and Test Methods Version 1.1.3

- R4-2506842, Discussion on the RF requirement for device 1, vivo