3GPP TSG-RAN WG4 Meeting #115 	R4-2510249

Bengaluru, India, August 25th – 29th, 2025

Source: 	vivo

Title: 	Discussion on the CW phase noise

Agenda Item:	7.26.2.3

Document for:	Approval

- Introduction

In this contribution, we provide our views on the CW node phase noise requirement.

- Discussion

The phase noise requirement is discussed for several meeting but it is still unclear whether and how to define such requirement for CW. The impact of CW is also discussed in reader Rx sensitivity, but it is identified that the phase noise cannot be reflected in the D2R reception performance since the phase noise of the CW from signal generator is much better than a practical CW node.

Observation 1: The impact of phase noise cannot be reflected in reader D2R reception, but can only be restricted in CW Tx requirement.

In the last meeting, RAN1 finished the Table for the physical layer parameter of R/Tb/Tc, and corresponding D2R transmission bandwidth and small frequency offset are summarized as table below:

| D2R channel bandwidth (kHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D2R transmission Bandwidth (kHz) | Small frequency shift (kHz) |  |  |  |  |  |  |  |  |
|  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |
| 15 | 16.5 | 24.75 | 41.25 | 74.25 | 140.25 | 272.25 | 536.25 | 1064.25 |  |
| 30 |  | 33 | 49.5 | 82.5 | 148.5 | 280.5 | 544.5 | 1072.5 |  |
| 60 |  |  | 66 | 99 | 165 | 297 | 561 | 1089 |  |
| 120 |  |  |  | 132 | 198 | 330 | 594 | 1122 |  |
| 240 |  |  |  |  | 264 | 396 | 660 | 1188 |  |
| 480 |  |  |  |  |  | 528 | 792 | 1320 |  |
| 960 |  |  |  |  |  |  | 1056 | 1584 |  |
| 2880 |  |  |  |  |  |  |  |  | 3168 |

Companies still have concern on setting the phase noise at the frequency close to CW due to the difficulty of CW implementation. In our understanding, for a CW is not capable to perform phase noise cancellation, it is hard to ensure that there is no desens for D2R reception when D2R signal is close to CW center frequency, but such CW still should ensure when SFS is applied, the desens is at an acceptable level.

Observation 2: For CW node without phase noise cancellation capability, the phase noise requirement should ensure that the impact of phase noise is controllable when SFS is applied.

In [2], -34 dBm sensitivity of device is taken as the starting point, and the coupling loss in DL link budget is only 38-(-34) = 72 dB. The maximum coverage design target of device is 15m, so we assume the maximum distance between CW and device is also not larger than 15m. The path loss of 15m is around 59 dB, so the minimum D2R backscatter power is 33 (CW output power) – 59 (pathloss) – 10(backscatter loss) = -36 dBm. Considering same coupling loss for UL and DL, the D2R power at reader is -36 -72 = -108 dBm.

Based on the SI assumption, considering 20m NLOS pathloss (around 61 dB) between CW and reader, 4dB shadow fading, 10dB spatial isolation due to the reader Rx antenna, the residual power of 33 dBm CW at reader could be – 42 dBm. The desens with 7.5kHz 1SB transmission bandwidth due to phase noise can be calculated as below:

| D2R 1SB transmission bandwidth | Wanted signal | PN level | Noise power within D2R 1SB transmission channel bandwidth | SNR | D2R SNR@ 10% BLER | Desens/dB |
| --- | --- | --- | --- | --- | --- | --- |
| 7.5 kHz | -108 dBm | -60 dBc/7.5kHz | -105 dBm | -3 | 0 dB (-6 dB as baseline and consider 6 dB desens due to the CW) | 3 |
|  |  | -65 dBc/7.5kHz | -110 dBm | 2 |  | 0 |
|  |  | -70 dBc/7.5kHz | -115 dBm | 7 |  | 0 |
|  |  | -75 dBc/7.5kHz | -120 dBm | 12 |  | 0 |
|  |  | -80 dBc/7.5kHz | -125 dBm | 17 |  | 0 |

Under -65 dBc/7.5kHz, the desens of CW phase noise can be eliminated based on the SI assumption.

Observation 3: The impact of phase noise can be avoided when the PN level is less than -63 dBc/7.5kHz.

The figure below is a practical phase noise profile of GSM for reference, and it is noted that when the offset larger than 100kHz, the roll-off of phase noise is become slower, so SFS = 120 kHz can be considered as the verification point from implementation perspective.

Figure x Practical GSM phase noise in [x]

The -63 dBc/7.5kHz equals to -100 dBc/Hz which is 10 dBc larger than GSM phase noise above at 120kHz offset, but considering the CW is specifically designed for AIoT system, necessary optimization should be required.

Proposal: The limit of phase noise can be defined as -30 dBm/7.5kHz at 120kHz offset from the center frequency of CW.

- Conclusion

In this contribution, we provide our views on CW phase noise requirement.

Observation 1: The impact of phase noise cannot be reflected in reader D2R reception, but can only be restricted in CW Tx requirement.

Observation 2: For CW node without phase noise cancellation capability, the phase noise requirement should ensure that the impact of phase noise is controllable when SFS is applied.

Observation 3: The impact of phase noise can be avoided when the PN level is less than -63 dBc/7.5kHz.

Proposal: The limit of phase noise can be -30 dBm/7.5kHz at 120kHz offset from the center frequency of CW.

- References

- R4-2508101 WF on requirements for A-IoT BS and CW	Huawei

- R4-2508116, WF on A-IOT device requirements, CMCC