3GPP TSG-RAN WG4 Meeting #116											R4-2510251

Bengaluru, India, August 25th – 29th, 2025

Source: 	vivo

Title: 	draft TP to TS 38.191 on device unwanted emission

Agenda Item:	7.22.3.2

Document for:	Approval

- Introduction

In this contribution, we update the TS based on the agreements in the meeting.

- Text proposal

## <<Start of Change>>

## 6.4	Output RF spectrum emissions

#### 6.4.1 	Out of band emissions

6.4.1.1 	General

The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions.

6.4.1.2	Spectrum emission mask

The spectrum emission mask of the device applies to frequencies (ΔfOOB) starting from the  edge of the assigned D2R channel bandwidth. For frequencies offset greater than ΔfOOB as specified in Table 6.4.1.2-1 the spurious requirements in clause 6.4.2 are applicable.

The power difference between transmitter output power and emission of any device 1 shall not exceed the levels specified in Table 6.4.1.2-1 for all specified D2R channel bandwidth. The requirement is verified with the test metric of EIRP under the incident CW power level at device is set to [-10] dBm. The test direction is same as the transmitter output power as specified in clause 6.2.

Table 6.4.1.2-1: General NR spectrum emission mask for device 1.

| ΔfOOB (MHz) | Spectrum emission limit (dBc) | Measurement bandwidth |
| --- | --- | --- |
| ± 0- max(500kHz, 10*D2R CBW)-0.5* D2R CBW | 12 | 0.5* D2R transmission bandwidth |

#### 6.4.2 	Spurious emissions

6.4.1.1 	General

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements in line with SM.329 [x].

6.4.1.2 	General spurious emission

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.4.1.2-1 from the edge of the D2R channel bandwidth. The spurious emission limits in Table 6.4.1.2-2 apply for all D2R channel bandwidths. The requirement is verified with the test metric of EIRP under the incident CW power level at device is set to [-10] dBm. The test direction is same as the transmitter output power as specified in clause 6.2.

Table 6.4.1.2-1: Boundary between NR out of band and general spurious emission domain

| D2R Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| All D2R CBW specified in clause 5.3.2 | max(500kHz, 10*D2R CBW)-0.5* D2R CBW |

Table 6.4.1.2-2: Requirement for general spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz ≤ f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 dBm | 1 MHz |  |
|  | -25 dBm | 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 2.55 GHz and less than or equal to 5.2 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz |  |  |  |

## <<End of Change>>

- Reference