# 3GPP TSG-RAN WG4 Meeting #116	R4-2510257

Bengaluru, India, August 25th – 29th, 2025

Source: 	vivo

Title: 	Discussion on general aspects of 6Rx requirements

Agenda Item:	7.1.2.3.1

Document for:	Approval

Introduction

The feature for 6Rx is one of important aspects for UE RF enhancements. In the last meeting, we discussed the general aspects of 6Rx requirements. The WFs are captured in [1]. In this contribution, we will further discuss the remaining issues for 6Rx.

Discussion

For FWA terminology, the WF was captured as follows:

Agreement: - Remove “form factor” from the notes in the corresponding tables for 4Rx, 6Rx and 8Rx, and use “FWA CPE” or “FWA” for ΔRIB,6R requirement in the note of the table. - Consider adding the definition of FWA CPE or FWA, and industrial UE in the specifications - Focus on modifying the notes of the receiver requirements for 4Rx, 6Rx and 8Rx for Rel-19 specifications in RAN4#115 - FFS on whether and how to align the existing notes between 4Rx, 6Rx and 8Rx since the notes were specified according to the separate WIDs. - Further modifications will be done in the future meetings - Address the similar issues for previous releases under the maintenance agenda - Address the similar issue for transmission requirements under the maintenance agenda.

Currently, there is misalignment for the notes of the receiver requirements for 4Rx, 6Rx and 8Rx.

- NOTE 1:	When 4 Rx operation is supported by FWA form factor

- NOTE 2:	When 6 Rx operation is supported by [FWA CPE].

- NOTE 1:	8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor.

These notes are confusing for the reader. One simple way we can think of is to use a more general term ‘non-handheld UE’ to replace FWA form factor and for the alignment between the notes of the receiver requirements for 4Rx, 6Rx and 8Rx. We can avoid to add more UE definitions for FWA/CPE/Vehicle/Industrial devices. We can further clarify this ‘non-handheld UE’ has larger form factor than handheld UE, e.g., FWA/CPE/Vehicle/Industrial devices, for the receiver requirements for 4Rx, 6Rx and 8Rx.

Proposal 1: It is suggested to use a general term ‘Non-handheld UE’ for 4Rx, 6Rx, 8Rx instead of ‘FWA/CPE/Vehicle/Industrial devices form factor’ for alignment.

The WF [1] for general aspects of 6Rx were captured as follows:

<Sub-topic 2-1> Release independency for 6Rx UE Agreement: - Option 1: - Support 6Rx with up to 4 MIMO layers for both handheld UE and FWA from Rel-18 - Support 6Rx with up to 6 MIMO layers for FWA from Rel-19 - Option 2: - Support 6Rx from Rel-19 - Option 3: - Support 6Rx from Rel-18

We prefer 6Rx RF requirements release independent from Rel-19.

6Rx RF requirements mainly include 6Rx REFSENS and SRS-AS IL ΔTRxSRS. The ΔTRxSRS requirement involves many new UE capabilities, e.g. t3r6 SRS antenna switching, which are introduced from R19. If release independence from earlier releases is defined, there will be forward compatibility issues; this is one reason why release independence is not recommended. However, if there are special industry requests to support 6Rx from earlier releases, we can further discuss it.

Proposal 1：It is suggested 6Rx RF requirements to be release independent from Rel-19.

Option 2 and 3 consider 6Rx as a whole feature for the release independence issue.

Option 1 can be a good compromise. However, 6Rx is a big feature, and we not only need to consider the release independence for the 4 and 6 MIMO layers, but also some sub-features such as SRS antenna switching t3r6. From our perspective, we can accept 6Rx with 4-layer release independent from Rel-18. For other subfeatures, such as 6Rx with 6layer, SRS antenna switching t3r6 can be release independent from Rel-19 because there are many new UE capabilities involved which may cause forward compatibility issues.

Conclusion

This contribution discusses general aspects for 6Rx. The following proposals are made:

Proposal 1: It is suggested to use a general term ‘Non-handheld UE’ for 4Rx, 6Rx, 8Rx instead of ‘FWA/CPE/Vehicle/Industrial devices form factor’ for alignment.

Proposal 2：It is suggested 6Rx RF requirements to be release independent from Rel-19.

- References

[1] R4-2508086, WF on 6Rx and SRS IL imbalance, Huawei, HiSilicon, RAN4#115