3GPP TSG-RAN WG4 Meeting # 116	R4-2510265

Bengaluru, India, August 25th – 29th, 2025

Source: 	vivo

Title: 	Iot NTN HPUE Tx Requirements

Agenda Item:	7.6.3.3.1

Document for:	Approval

Introduction

From RAN4#112 to #114bis agreements, summary, way forward and evaluation results were included in [1] to [17].

In RAN4#115, an evaluation of NR PA for NB-Iot Iot-NTN was submitted in [18], and WF and summary is provided in [19][20]. A draft big CR is also endorsed in [21].

In this paper, more evaluation of NB-Iot based Iot-NTN based on NR PA has been submitted for evaluation of MPR requirements. An accompanying draft CR [22] was also submitted to incorporate some of the proposals in this document.

Tx Requirements for HPUE

General

The current status is for requirements applicability is as following which was referenced from last meeting’s WF [19].

Table 4: Current status for requirements applicability for Iot-NTN

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | Both |
|  | PC2 | 1Tx | Both |
| eMTC based with half duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |
| eMTC based with full duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |

NB-Iot Based

- 2.2.1 MPR

MPR General

In last meeting, the following general principle of MPR has been agreed in the WF.

Issue 1-1-1: MPR requirements definitions for NB-Iot based Iot-NTN

- Updated information and tentative conclusions in previous offline discussion

  - Modulation and SCS restriction in physical spec

    - Single Tone allocation

      - Supported Modulation: pi/2 BPSK, pi/4 QPSK

      - Supported SCS: 15KHz, 3.75KHz

    - Multi-Tone allocation

      - Supported Modulation: QPSK

      - Supported SCS: 15KHz

  - An allowed configuration without requirement is likely deemed as 0dB for MPR

Agreement:

  - MPR requirements need to cover all supported modulation and SCS

MPR for PC2

For PC2, some agreements have already been made, and we would discuss by single tone and multi-tone respectively.

For single tone, the previous WF is as following:

Issue 1-1-2: MPR requirements for NB-Iot based Iot-NTN PC2

- Agreement:

  - Unified requirements for pi/2 bpsk and pi/4 QPSK for single Tone

  - The following structure and requirements considered as starting point. Further refinements on tables and wording can be made later

- Table 6.2B.2-2: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK | π/2 BPSK, π/4 QPSK |
| --- | --- | --- |
| SCS | 15KHz | 3.75kHz |
| Tone positions for 1 Tone allocation | [0-11] | [0-47] |
| MPR | [0] dB | TBD dB |
| Note 1: For single-tone allocation, the listed modulation and SCS are supported. |  |  |

- Table 6.2B.2-3: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ TBD dB | TBD dB |  | ≤ TBD dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ TBD dB |  | ≤ TBD dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [3] dB |  |  |  |
| Note 1: For multi-tone allocation, only QPSK and 15KHz SCS can be supported |  |  |  |  |

For PC2, although the current structure assumes unified requirements for 3.75kHz for all the allocation, the edge allocation is assumed to be more difficult. So still some relaxation can be considered for edge cases for single tone.

For multiple tone case, based on the evaluation and proper implementation margin, the following proposal are also provided.

Proposal 1: The following proposal for PC2 NB-Iot based MPR are listed:

- Table 6.2B.2-2: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz | 3.75kHz |  |  |
| Tone positions for 1 Tone allocation | [0-11] | [0-7] | [8-39] | [40-47] |
| MPR | [0] dB | [0.5] dB | [0] dB | [0.5] dB |
| Note 1: For single-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

- Table 6.2B.2-3: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [1] dB | [0] dB |  | ≤ [1] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [1.5] dB |  | ≤ [1.5] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [3] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

MPR for PC1

In last meeting, the following tentative MPR requirements have been reached for PC1:

- Table 6.2B.2-4: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  | 3.75kHz |  |  |
| Tone positions for 1 Tone allocation | [0-2] | [3-8] | [8-11] | [0-TBD] | TBD | [TBD-47] |
| MPR | ≤[1.5~2] dB | [0.5] dB | ≤[1.5~2] dB | ≤ TBD dB | [0.5] dB | ≤ TBD dB |
| Note 1: For single-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |  |  |

- Table 6.2B.2-5: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤TBD dB | TBD dB |  | ≤TBD dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤TBD dB |  | ≤ TBD dB |  |
| [Tone positions for 12 Tones allocation] | 0-11 |  |  |  |
| [MPR] | ≤ [5-5.5] dB |  |  |  |
| Note 1: For multi-tone allocation, only QPSK and 15KHz SCS can be supported |  |  |  |  |

Based on the analysis and previous results, the following proposal for single tone and multi-tones are provided:

Proposal 2: The following proposal for PC1 NB-Iot based MPR are listed:

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  | 3.75kHz |  |  |
| Tone positions for 1 Tone allocation | [0-2] | [3-8] | [8-11] | [0-7] | [8-39] | [40-47] |
| MPR | ≤[1.5~2] dB | [0.5] dB | ≤[1.5~2] dB | ≤ [3.5-4] dB | [0.5] dB | ≤ [3.5-4] dB |
| Note 1: For single-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |  |  |

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [2] dB | [1.5] dB |  | ≤ [2] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [2.5] dB |  | ≤ [2.5] dB |  |
| [Tone positions for 12 Tones allocation] | 0-11 |  |  |  |
| [MPR] | ≤ [5-5.5] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

- 2.2.2 A-MPR

A-MPR General

The current PC3 A-MPR requirement for NB-Iot based Iot-NTN is as following:

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5B.4.4.5 | 254 | TBD |  |
| NS_05N | 6.5B.4.4.6 | 254 | TBD |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |

In last meeting, the following WF has been agreed:

Issue 1-1-5: A-MPR for NB-Iot based Iot-NTN PC2

- Agreement:

  - Wait for PC3 A-MPR baseline before concluding PC2 A-MPR for NS_04N and NS_05N (band 254)

  - No A-MPR is needed for [NS_03N] and NS_02N for NB-IoT PC2 in square brackets

  - Define [2] dB A-MPR increase for NS_24 to support NB-IoT PC2 compared to PC3

    - The exact value can be revisited once MPR is agreed.

Although the WF was focused on PC2, actually for many signalling, different power class shared similar situations. so analysis would be done by different signalling respectively.

NS_02N and NS_03N

For NS_02N and NS_03N, the FCC SEM is relative to in-band power, so even in current addition SEM requirements, the emission limit is scaled according to the power class as following:

Table 6.5B.3.3.1-1: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -12 for PC3 -15 for PC5 | 4 kHz |
| ± >0.85 | -13 for PC3 and PC5 | 4 kHz |

Table 6.5B.3.3.2-1: Additional requirements for "NS_03N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -12 for PC3 -15 for PC5 | 4 kHz |
| ± 0.85–1.7 | -13 for PC3 and PC5 | 4 kHz |

After some check, this emission limit is actually more relaxed than general SEM. In order to make the comparison more clear, a general comparison was done:

Observation 1: The addition SEM for NS_02N and NS_03N is actually more relaxed than general SEM for NB-Iot, and the scalable to in-band power.

In this sense, even for higher power class like PC2 and PC1, the addition SEM seems would not be a bottle neck.

Of course, for additional spurious emissions, there are still requirements to be met as following:

It might be assumed that band 254 might be more challenging compared to band 255. At least for band 255, it seems that sufficient margin should be there at least for band 255 even for PC1.

Observation 2: The addition spurious emission limit should have sufficient margin, at least for band 255.

Proposal 3a: For band 255, no additional A-MPR needed for NS_02N for PC2 and PC1 NB-Iot based NTN.

Proposal 3b: For band 254, define PC2 requirements and more discussion can be considered for NS_03N for PC1 NB-Iot based NTN.

In addition, the previous draft big CR seems omitted the revision of the power class dependent of spectrum emission limit, so this is also need to be supplemented.

Proposal 4: The additional SEM also needs to be supplemented, e.g.

Table 6.5B.3.3.1-1: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | -2 for PC3 -5 for PC5 1 for PC2 6 for PC1 | 4 kHz |
| ± 0.28–0.85 | -12 for PC3 -15 for PC5 1 for PC2 6 for PC1 | 4 kHz |
| ± >0.85 | -13 for PC1, PC2, PC3 and PC5 | 4 kHz |

NS_04N and NS_05N

As indicated, the PC3 requirements is still not ready for NS_04N and NS_05N, thus there is no need to discuss PC2/1 requirements.

Proposal 5: Do not define requirements for NS_04N and NS_05N for PC2/1 as long as PC3 requirements are not ready for NB-Iot based Iot-NTN.

NS_24

The additional requirement for NS_24 if for protection of band 34 which is as following:

The current NS_24 requirements currently use a unified A-MPR for all the allocations:

As in the previous WF, [2] dB A-MPR increase for NS_24 to support NB-IoT PC2 compared to PC3 was suggested and agreed. It was noted that MPR would be added to A-MPR in LTE and NB-Iot, so there is suggestion to also consider MPR. However, there is one difficulty: for PC2 and PC1, the differentiation of MPR become much larger depending on different modulation and resource allocation.

It should be noted that currently, the A-MPR is single value based, which do not differentiate different resource allocations. If this principle is kept, it means that the worst case has to be considered. For spurious emission, which is not that close to operation band, the emission may not that sensitive to resource allocation.

In this sense, if more secure value need to be considered without further evaluation, an addition of [5]dB can be considered for PC1 based on PC2. This might be more relax than needed for some resource allocation since the total MPR+A-MPR.

Based on the previous analysis, the following table was provided:

| Power Class | MPR range (dB) | NS_24 A-MPR (dB) | MPR+A-MPR |
| --- | --- | --- | --- |
| PC3 | 0 ~ 2 | 3.5 | 3.5 ~ 5.5 |
| PC5 | 0 ~ 2 | 0.5 | 0.5 ~ 2.5 |
| PC2 | [0] ~ [3] | 3.5 + [2] | [5.5] ~ [8.5] |
| PC1 | [0.5] ~ [5.5] | Option 1: 3.5 + [2] + [4.5] Option 2: 3.5 + [2] + [2.5] | Option 1: [10.5] ~ [15.5] Option 2: [8.5] ~ [13.5] |

For PC1, two options were proposed for different A-MPR. One is a general [4.5]dB increase compared to PC2. This is a “safe” option considering the worst case, but the total back off would seems large for multi-tone case. This corresponds to 5dB increase for the lower limit. Another option is adding a more moderate [2.5]dB offset compared to PC2. This would still have a 5dB increase for the higher limit of total back off which corresponds to multi-tone cases. If multi-tone case is deemed as main bottle neck, this option can also be considered.

In all, a 2.5~4.5dB increase can be considered for PC1 A-MPR compared to PC2 A-MPR, after considering MPR.

Proposal 6a: For PC2 NB-Iot based NS_24, confirm 2dB A-MPR increase compared to PC3, after considering MPR.

Proposal 6b: For PC1 NB-Iot based NS_24, [2.5~4.5] dB A-MPR increase can be considered for A-MPR compared to PC2, after considering MPR.

- 2.2.3 Others

Power Boosting

In last meeting’s WF, the agreement for power boosting was agreed as following:

Issue 1-1-8: On relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation.

- Background (Last meeting’s WF):

  - Restrict the boosting condition to PC2 NB-Iot based Iot-NTN

  - To down-select in next meeting between:

    - Option 1: Increase [1] dB MOP upper tolerance

    - Option 2: Increase [1] dB MOP nominal power (optional with capability signalling)

- WF(NEW):

  - Restrict the boosting condition to PC2 NB-Iot based Iot-NTN

  - Increase [0.8] dB MOP or add a power boosting item in Pcmax equation (optional with capability signalling)

It should be noted that this power boosting is totally different from power boosting defined for NR.

One of the remaining issue for this whether only higher limit or both higher and lower limit of Pcmax would be impacted. And this need to be further discussed.

Proposal 7: Discuss and confirm whether lower limit would be changed or not by this boosting feature.

16QAM for NB2

It should be noted that for the UE type NB2, 16QAM is supported by NB-Iot in physical layer spec in multi-tone cases. Since no other specific clarification, it seems that NB-Iot based Iot-NTN can also support 16QAM.

However, 16QAM is never considered in the context of Iot-NTN. Even the RMC currently defined only considered NB1 support and never consider 16QAM. But the current requirements are marked to be applicable to both NB1 and NB2 in both NB-Iot and NB-Iot based Iot-NTN.

Observation 3: Though NB-Iot NB2 can support 16QAM, 16QAM has never been considered in 36.101 and 36.102.

Though this may seems not that complete, actually multi-tone 16QAM may not popular. We may not to consider the 16QAM. However, the note in the multi-tone MPR tables may have to make some further refinements to consider this situation.

Proposal 7a: Keep the current status that do not consider 16QAM in NB-Iot based Iot-NTN requirements.

eMTC-Based

- 2.3.1 MPR

For eMTC based Iot-NTN, PC1 MPR still not confirm yet.

Issue 1-1-7: MPR requirement for eMTC based Iot-NTN for PC1

- Agreement:

  - Reuse eMTC TN PC2/PC3 MPR requirements for eMTC based Iot-NTN for PC1 as starting point

  - Edge situation need to be checked

The eMTC is it is based on LTE, which is quite different from NR in terms of MPR reduction. Take the LTE MPR and current eMTC category M1 NTN as following:

By comparison, NR has more detailed MPR requirements, which separately listed edge cases and have dedicated relaxations for PC1. This may reflect more detailed characteristics and thus more accurate. So one way is to add similar edge requirements in eMTC and LTE as in NR. However, this may take more time, and also bring alignment problems between eMTC and LTE. Given that the

Proposal 8: For eMTC PC1, keep the MPR requirements same to PC2/3 as aligned with LTE if introduced.

- 2.3.2 A-MPR

General

The current A-MPR requirements for eMTC based Iot-NTN is as following:

The basic condition is actually pretty similar to NB-Iot.

NS_02N and NS_03N

Similar to NB-Iot case, the NS_02N and NS_03N also have additional SEM which is relative to the in-band power, making hpue more relaxed, thus no need to apply higher A-MPR. At least for band 255 and NS_02N, no more A-MPR seems needed.

The spurious emission is also likely not be bottle neck.

Proposal 9a: For band 255, no additional A-MPR needed for NS_02N for PC2 and PC1 eMTC based NTN.

Proposal 9b: For band 254, define PC2 requirements and more discussion can be considered for NS_03N for PC1 eMTC based NTN.

NS_04N and NS_05N

Similar to NB-Iot based, the PC3 requirements is still not ready for NS_04N and NS_05N, thus there is no need to discuss PC2/1 requirements.

Proposal 10: Do not define requirements for NS_04N and NS_05N for PC2/1 as long as PC3 requirements are not ready for eMTC based NTN.

NS_24

The discussion in NB-Iot based can also be considered here.

For PC2, an offset of [3]dB can be applied to compared to PC3. Since the MPR for PC2 already use PC3 requirements, here [3]dB is a safer choice.

For PC1, since the MPR characteristics for PC1 can be more problematic and lack sufficient study, the requirements may need further discussion.

Proposal 11a: For PC2 eMTC based NS_24, use 3dB A-MPR increase compared to PC3.

Proposal 11b: For PC1 NB-Iot based NS_24, discuss more on A-MPR compared to PC2.

- 2.2.3 Others

Overall speaking, the PC1 eMTC-based Iot-NTN seems have less mature implementation reference. If companies prefer, it can be further discussed whether this power class can be skipped in the final specification.

SAR and Duty cycle

In last meeting WF, all the previous schemes were summarized and some options were listed. For completeness, they were copied below:

Issue 2-1-1: Whether and how to define SAR scheme other than already agreed P-MPR (e.g. duty cycle related scheme):

- Background (for information):

  - P-MPR was already agreed to be the baseline SAR solution for all NTN HPUE cases.

  - The current status for duty-cycle based SAR scheme in TN

    - NR:

      - Defined in MOP section;

      - Applied in ΔPPowerClass in configured transmitted power clause;

      - whether can apply to FDD band has some different understanding.

    - eMTC:

      - Not defined in MOP section;

      - Reference general LTE configurated transmitted power clause, no separate definition

      - For general LTE, the duty cycle related power backoff explicitly defined only for TDD band whose frame configuration is 0 or 6

    - NB-Iot:

      - Not defined in MOP section;

      - Not defined in configured transmitted power clause which use dedicated equations (Not reference LTE general equation)

- Options for Selection:

  - Option 1: Keep the same status as TN for respective base RAT

    - NR-NTN:	Reuse NR TN,

      - Including ΔPPowerClass in configured transmitted power clause

    - eMTC based Iot-NTN:		Reuse eMTC

      - Not precluding ΔPPowerClass in configurated transmitted power clause

    - NB-Iot based Iot-NTN: 	Reusing NB-Iot

      - No duty cycle related scheme defined

  - Option 2: Introduce further revision / new method

    - NR-NTN sub-options:

      - A. Introduce clarification for FDD band support; (Nokia, Qualcomm)

      - B. changing the ΔPPowerClass from mandatory to optional;(Qualcomm)

      - C. UE voluntarily dropping uplink transmissions by self-assessment to control the SAR (vivo)

      - D. differentiate UE working status in terms of whether SAR is required in a time duration. (OPPO)

      - E. To have the network to notify/signal its upper bound of uplink ratio for a certain time duration (Samsung)

      - F. Energy headroom reporting (Qualcomm)

      - G. ΔPPowerClass and P-MPR reporting (Ericsson, SONY, Nokia)

      - H. Send information LS to RAN2 support PC1 NTN HPUE for FDD use cases. (Nokia)

      - I. inform the RAN that it is close to the SAR limit (ViaSet, Inmarsat)

    - Iot-NTN (including eMTC & NB-Iot based):

      - Introduce NR like scheme

      - Introduce other revision / new method as proposed for NR-NTN

  - Option3: Do not introduce for NTN

    - NR NTN	:	Explicitly preclude ΔPPowerClass and no other duty cycle related reference

    - eMTC-based Iot-NTN:		Explicitly preclude ΔPPowerClass and no other duty cycle related reference

    - NB-Iot based Iot-NTN:		No duty cycle or other new scheme defined

Among those options, option 1 is most likely compromise which kept the basic concept while also kept the ambiguities; option 2 is actually not an option, but a summary of all the different proposals from different companies; option 3 means explicitly exclude this duty cycle related delta_powerclass.

Observation 4: After long online and offline discussion, it is apparent in current stage that option 1 is the only possible compromise other than the introduce nothing option (option 3).

Unfortunately, no compromise solution was made last meeting on the options. It is apparent it is difficult to reach any further compromise solution. If the situation continues, then it means option 3 is chosen. There is anyway P-MPR solution onboard.

Proposal 12: If option 1 cannot be agreed, then option 3 (do not introduce delta_powerclass for NTN) would be the final option for this duty cycle related SAR scheme.

Conclusion

NB-Iot Based Iot-NTN

MPR

Proposal 1: The following proposal for PC2 NB-Iot based MPR are listed:

- Table 6.2B.2-2: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz | 3.75kHz |  |  |
| Tone positions for 1 Tone allocation | [0-11] | [0-7] | [8-39] | [40-47] |
| MPR | [0] dB | [0.5] dB | [0] dB | [0.5] dB |
| Note 1: For single-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

- Table 6.2B.2-3: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [1] dB | [0] dB |  | ≤ [1] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [1.5] dB |  | ≤ [1.5] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [3] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Proposal 2: The following proposal for PC1 NB-Iot based MPR are listed:

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  | 3.75kHz |  |  |
| Tone positions for 1 Tone allocation | [0-2] | [3-8] | [8-11] | [0-7] | [8-39] | [40-47] |
| MPR | ≤[1.5~2] dB | [0.5] dB | ≤[1.5~2] dB | ≤ [3.5-4] dB | [0.5] dB | ≤ [3.5-4] dB |
| Note 1: For single-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |  |  |

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [2] dB | [1.5] dB |  | ≤ [2] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [2.5] dB |  | ≤ [2.5] dB |  |
| [Tone positions for 12 Tones allocation] | 0-11 |  |  |  |
| [MPR] | ≤ [5-5.5] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

A-MPR

Observation 1: The addition SEM for NS_02N and NS_03N is actually more relaxed than general SEM for NB-Iot, and the scalable to in-band power.

Observation 2: The addition spurious emission limit should have sufficient margin, at least for band 255.

Proposal 3a: For band 255, no additional A-MPR needed for NS_02N for PC2 and PC1 NB-Iot based NTN.

Proposal 3b: For band 254, define PC2 requirements and more discussion can be considered for NS_03N for PC1 NB-Iot based NTN.

Proposal 4: The additional SEM also needs to be supplemented, e.g.

Table 6.5B.3.3.1-1: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | -2 for PC3 -5 for PC5 1 for PC2 6 for PC1 | 4 kHz |
| ± 0.28–0.85 | -12 for PC3 -15 for PC5 1 for PC2 6 for PC1 | 4 kHz |
| ± >0.85 | -13 for PC1, PC2, PC3 and PC5 | 4 kHz |

Proposal 5: Do not define requirements for NS_04N and NS_05N for PC2/1 as long as PC3 requirements are not ready for NB-Iot based Iot-NTN.

Proposal 6a: For PC2 NB-Iot based NS_24, confirm 2dB A-MPR increase compared to PC3, after considering MPR.

Proposal 6b: For PC1 NB-Iot based NS_24, [2.5~4.5] A-MPR increase can be considered for A-MPR compared to PC2, after considering MPR.

Others

Proposal 7: Discuss and confirm whether lower limit would be changed or not by this boosting feature.

Observation 3: Though NB-Iot NB2 can support 16QAM, 16QAM has never been considered in 36.101 and 36.102.

Proposal 7a: Keep the current status that do not consider 16QAM in NB-Iot based Iot-NTN requirements.

eMTC Based Iot-NTN

MPR

Proposal 8: For eMTC PC1, keep the MPR requirements same to PC2/3 as aligned with LTE if introduced.

A-MPR

Proposal 9a: For band 255, no additional A-MPR needed for NS_02N for PC2 and PC1 eMTC based NTN.

Proposal 9b: For band 254, define PC2 requirements and more discussion can be considered for NS_03N for PC1 eMTC based NTN.

Proposal 10: Do not define requirements for NS_04N and NS_05N for PC2/1 as long as PC3 requirements are not ready for eMTC based NTN.

Proposal 11a: For PC2 eMTC based NS_24, use 3dB A-MPR increase compared to PC3.

Proposal 11b: For PC1 NB-Iot based NS_24, discuss more on A-MPR compared to PC2.

Others

SAR and Duty Cycle

Observation 4: After long online and offline discussion, it is apparent in current stage that option 1 is the only possible compromise other than the introduce nothing option (option 3).

Proposal 12: If option 1 cannot be agreed, then option 3 (do not introduce delta_powerclass for NTN) would be the final option for this duty cycle related SAR scheme.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

[2] R4-2412829, Topic summary for [112][127] NR_IoT_NTN_HPUE , Samsung, RAN4#112

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112

[4] R4-2412099, Feasibility study and Initial analysis for NTN HPUE RF Tx Requirements, vivo, RAN4#112

[5] R4-2415790, Feasibility study and analysis for Iot NTN HPUE Tx Requirements, vivo, RAN4#112bis

[6] R4-2415229, Topic summary for [112bis][125] NR_IoT_NTN_HPUE_part2,vivo, RAN4#112bis

[7] R4-2417205, WF on HPUE for IoT NTN, vivo, RAN4#112bis

[8] R4-2418202, Iot NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#113

[9] R4-2418149, Topic summary for [113][127] NR_IoT_NTN_HPUE_part2, Moderator (vivo), RAN4#113

[10] R4-2420404, WF on IoT NTN HPUE, vivo, RAN4#113

[11] R4- 2500766, CR on Remove GSM_ACLR for NB-Iot based Iot-NTN, vivo, Huawei, MediaTek, RAN4#114

[12] R4-2500764, Iot NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#114

[13] R4-2500681, Topic summary for [114][129] NR_IoT_NTN_HPUE_part2, Moderator(Vivo), RAN4#114

[14] R4-2503006, WF on HPUE UE requirement for IoT-NTN, vivo, RAN4#114

[15] R4-2503842, Iot NTN HPUE Tx Requirements and feasibility analysis, vivo, SmarterMicro, RAN4#114bis

[16] R4-2505114, WF on HPUE RF requirements for IoT-NTN, vivo, RAN4#114bis

[17] R4-2504682, Topic summary for [114bis][124] NR_IoT_NTN_HPUE_part2, Moderator(Vivo), RAN4#114bis

[18] R4-2506858, Iot NTN HPUE Tx Requirements and feasibility analysis, vivo, SmarterMicro, RAN4#115

[19] R4-2508008, WF on NR_IoT_NTN_HPUE_part2, vivo, RAN4#115

[20] R4-2507568, Topic summary for [115][129] NR_IoT_NTN_HPUE_part2, Moderator(vivo), RAN4#115

[21] R4-2508105, Draft big CR for 36.102 for Iot-NTN HPUE, vivo, RAN4#115

[22] R4-2510267, draft CR for Iot-NTN MOP related requirements, vivo, RAN4#115