3GPP TSG-RAN4 Meeting #116	R4-2510267

# Bengaluru, India, 25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR for Iot-NTN MOP related requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2025-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The requirements for Iot-NTN HPUE are still absent. According to the arrangement of R4-2505113, several draft CRs were splitted and a draft big CR is also planned. The draft CRs: R4-2508074, R4-2508075, R4-2508076 have been endorsed in RAN4#115, and this draft Big CR is incrporating those endorsed draft CRs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For NB-Iot based MOP, the tolerance way was not accepted and capability signaling was agreed. Editor’s note: The detailed way was not finished and the draft CR for this part is not complete. The configurated transmitted power are updated. Mainly the Pcmax related equations and tolerances for Pumax. For eMTC based Iot-NTN, the tolerance for Pumax is the same to LTE, since the upper/lower tolerance are same to LTE (+2/-3); For NB-Iot based Iot-NTN, the tolerance of Pumax equation is re-write to increase the higher tolerance MAX{TH,c, T(PCMAX_H,c)}, since the upper tolearnce can be increased by 1dB. Editor’s note: Since the MOP tolerance part was updated in last minute, this Pumax part may also need to be updated. E.g. Higher tolerance may not need to be updated, the new tolerance table may be applied to PC1/2 only (PC3 can also consider, PC5 is different.). MPR table for PC2, PC1, PC3 were seperated. The tentative A-MPR requirements were included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NTN HPUE requirements for 36.102 is still absent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A, 6.2B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.521-4 CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

6.2A	Transmit power for category M1

6.2A.1	UE maximum output power for category M1

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 6.2A.1-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 | 31 | +2/-3 | 26 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 255 | 31 | +2/-3 | 26 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 254 | 31 | +2/-3 | 26 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 253 |  |  |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Generally, PC1 UE is not targeted for smartphone form factor. |  |  |  |  |  |  |  |  |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

The UE shall meet the following additional requirements for maximum transmission power density specified in Table 6.2A.1-2 when NS is signaled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2A.1-2: Additional requirements for transmit EIRP density

| E-UTRA Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum power density |
| --- | --- | --- | --- | --- |
| 254 | NS_04N | 1.4 | 1610 - 1618.25 | 27dBm/4kHz (mean EIRP limit) |
|  | NS_05N | 1.4 | 1618.25 - 1626.5 |  |
|  | NS_11N | 1.4 | 1610 - 1618.25 | 15dBm/4kHz (peak EIRP limit) |
|  | NS_12N | 1.4 | 1618.25 - 1626.5 |  |

6.2A.2	UE maximum output power reduction for category M1

For category M1 UE, the allowed Maximum Power Reduction (MPR) for the maximum output power specified in Table 6.2A.1-1 due to higher order modulation and transmit bandwidth configuration (resource blocks) is specified in Table 6.2A.2-1.

For subPRB allocation of category M1 UE[ of PC3 and PC2][], no MPR applies.

Table 6.2A.2-1: Maximum Power Reduction (MPR) for category M1 UE for Power Class [1, ]2, 3 and 5

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) | MPR (dB) |
| --- | --- | --- |
|  | 1.4 MHz |  |
| QPSK | > 2 | ≤ 1 |
| QPSK | > 5 | ≤ 2 |
| 16 QAM | ≤ 2 | ≤ 1 |
| 16 QAM | >2 | ≤ 2 |
| NOTE: MPR only applicable for NRB ≥ 1 |  |  |

For PRACH, PUCCH and SRS transmissions, the allowed MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For each subframe, the MPR is evaluated per slot and given by the maximum value taken over the transmission(s) within the slot; the maximum MPR over the two slots is then applied for the entire subframe.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2A.4 apply.

6.2A.3	UE additional maximum output power reduction for category M1 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2A.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

The specific requirements and identified subclauses are specified in Table 6.2A.3-1 and Table 6.2A.3-1a along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2A.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE for PC3 and PC5

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |  |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | Table 5.3A-1 | N/A |  |
| NS_24 | 6.5A.4.4.3 | 256 | Table 5.3A-1 | PC3 | PC5 |
|  |  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5A.4.4.2 | 255 | Table 5.3A-1 | N/A |  |
| NS_03N | 6.5A.4.4.4 | 254 | Table 5.3A-1 | N/A |  |
| NS_04N | 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_05N | 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_11N | 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_12N | 6.5A.4.4.6 | 254 | Table 5.3A-1 | N/A |  |

Table 6.2A.3-1a: Additional Maximum Power Reduction (A-MPR) for category M1 UE for PC1 and PC2

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |  |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | Table 5.3A-1 | N/A |  |
| NS_24 | 6.5A.4.4.3 | 256 | Table 5.3A-1 | [PC1] | PC2 |
|  |  |  |  | ≤ TBD | ≤ [6.5] |
| NS_02N | 6.5A.4.4.2 | 255 | Table 5.3A-1 | [N/A] |  |
| [NS_03N] | 6.5A.4.4.4 | 254 | Table 5.3A-1 | [N/A] |  |

For subPRB allocation, the allowed A-MPR values specified below in Table 6.2A.3-2 and Table 6.2A.3-3 for category M1 UE are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-2: Additional Maximum Power Reduction (A-MPR) for category M1 UE for subPRB allocation for PC3 and PC5

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5A.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5A.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5A.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5A.4.4.5 | 254 | N/A |  |
| NS_05N | 6.5A.4.4.5 | 254 | N/A |  |
| NS_11N | 6.5A.4.4.5 | 254 | PC3 | PC5 |
|  |  |  | ≤ 3 | 0 |
| NS_12N | 6.5A.4.4.6 | 254 | PC3 | PC5 |
|  |  |  | ≤ 3 | 0 |

Table 6.2A.3-3: Additional Maximum Power Reduction (A-MPR) for category M1 UE for subPRB allocation for PC1 and PC2

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5A.4.4.3 | 256 | [PC1] | PC2 |
|  |  |  | ≤ TBD | ≤ [6.5] |
| NS_02N | 6.5A.4.4.2 | 255 | [N/A] |  |
| [NS_03N] | 6.5A.4.4.4 | 254 | [N/A] |  |
| [NS_04N] | 6.5A.4.4.5 | 254 | [N/A] |  |
| [NS_05N] | 6.5A.4.4.6 | 254 | [N/A] |  |

6.2A.4	Configured transmitted Power for category M1

The configured transmitted power requirements in clause 6.2.5 of TS 36.101 [7] shall apply except for:

	PCMAX_L,c = MIN { PEMAX,c ,  PPowerClass – MAX(MPRc + A-MPRc, P-MPRc)}

	PCMAX_H,c = MIN { PEMAX,c ,  PPowerClass}

, wherein

-	PPowerClass is the maximum UE power specified in Table 6.2A.1-1 without taking into account the tolerance specified in the Table 6.2A.1-1

-	The Maximum output power requirements are specified in subclause 6.2A.1

-	The MPR requirements are specified in subclause 6.2A.2

-	The A-MPR requirements are specified in subclause 6.2A.3.

-	PEMAX,c and P-MPRc are specified in subclause 6.2.5 of TS 36.101 [7]

6.2B	Transmit power for category NB1 and NB2

6.2B.1	UE maximum output power for category NB1 and NB2

Category NB1 and NB2 UE Power Classes are specified in Table 6.2B.1-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75 kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2B.1-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 | 31 | +2/-3 | 26 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 255 | 31 | +2/-3 | 26 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 254 | 31 | +2/-3 | 26 | +/-2 | 23 | +/-2 | 20 | +/-2 |
| 253 |  |  |  |  | 23 | +/-2 | 20 | +/-2 |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

The UE shall meet the following additional requirements for maximum transmission power density specified in Table 6.2B.1-2 when NS is signaled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2B.1-2: Additional requirements for transmit EIRP density

| E-UTRA Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum power density |
| --- | --- | --- | --- | --- |
| 254 | NS_04N | 0.2 | 1610 - 1618.25 | 27dBm/4kHz (mean EIRP limit) |
|  | NS_05N | 0.2 | 1618.25 - 1626.5 |  |
|  | NS_11N | 0.2 | 1610 - 1618.25 | 15dBm/4kHz (peak EIRP limit) |
|  | NS_12N | 0.2 | 1618.25 - 1626.5 |  |

6.2B.2	UE maximum output power reduction for category NB1 and NB2

For UE category NB1 and NB2, the allowed Maximum Power Reduction (MPR) for the maximum output power are specified in following tables.

Table 6.2B.2-1: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 3 and 5

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 1 Tone allocation | 0-11 |  |  |  |
| MPR | 0 dB |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0.5 dB | 0 dB |  | ≤ 0.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 1 dB |  | ≤ 1 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |

Table 6.2B.2-2: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz | 3.75kHz |  |  |
| Tone positions for 1 Tone allocation | [0-11] | [0-7] | [8-39] | [40-47] |
| MPR | [0] dB | [0.5] dB | [0] dB | [0.5] dB |
| Note 1: For single-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Table 6.2B.2-3: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [1] dB | [0] dB |  | ≤ [1] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [1.5] dB |  | ≤ [1.5] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [3] dB |  |  |  |
| [Note 1: For multi-tone allocation, the listed modulation and SCS are supported.] |  |  |  |  |

Table 6.2B.2-4: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  | 3.75kHz |  |  |
| Tone positions for 1 Tone allocation | [0-2] | [3-8] | [8-11] | [0-7] | [8-39] | [40-47] |
| MPR | ≤ [1.5~2] dB | [0.5] dB | ≤[1.5~2] dB | ≤[3.5-4]dB | [0.5] dB | ≤[3.5-4]dB |
| [Note 1: For single-tone allocation, the listed modulation and SCS are supported. ] |  |  |  |  |  |  |

Table 6.2B.2-5: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [2] dB | [1.5] dB |  | ≤ [2] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤[2.5]dB |  | ≤[2.5]dB |  |
| [Tone positions for 12 Tones allocation] | 0-11 |  |  |  |
| [MPR] | ≤ [5-5.5] dB |  |  |  |
| [Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

For the UE maximum output power modified by MPR, the power limits specified in sub-clause 6.2B.4 apply.

6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

For UE PC3/PC5 and PC1/PC2, the specific requirements and identified subclauses are specified in Table 6.2B.3-1 and 6.2B.3-2 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2-1.

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5B.4.4.5 | 254 |  |  |
| NS_05N | 6.5B.4.4.5 | 254 |  |  |
| NS_11N | 6.5B.4.4.5 | 254 |  |  |
| NS_12N | 6.5B.4.4.6 | 254 |  |  |

Table 6.2B.3-2: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC1 and PC2

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC1 | PC2 |
|  |  |  | ≤ [8.5~10.5] | ≤ [5.5] |
| NS_02N | 6.5B.4.4.2 | 255 | [N/A] |  |
| NS_03N | 6.5B.4.4.4 | 254 | [N/A] |  |

6.2B.4	Configured transmitted Power for category NB1 and NB2

For category NB1 and NB2 UE, the configured transmitted power requirements in clause 6.2.5F of TS 36.101 [7] shall apply except for:

The definition of PCMAX_L,c which would be substituted by:

	PCMAX_L,c = MIN { PEMAX,c ,  PPowerClass – MAX(MPRc + A-MPRc, P-MPRc) [+ ΔPNBBoostPC2]}

	PCMAX_H,c = MIN { PEMAX,c,  PPowerClass + ΔPNBBoostPC2 }

, wherein

-	PPowerClass is the maximum UE power specified in Table 6.2B.1-1 without taking into account the tolerance specified in the Table 6.2B.1-1

-	ΔPNBBoostPC2 is defined as 0.8 dB for power class 2, if the UE diciate support for UE capability [powerBoosting-NB-PC2-r19], and if IE [powerBoostNBPC2-r19] is set to 1.

-	The Maximum output power requirements are specified in subclause 6.2B.1

-	The MPR requirements are specified in subclause 6.2B.2

-	The A-MPR requirements are specified in subclause 6.2B.3.

-	PEMAX,c and P-MPRc are specified in subclause 6.2.5 of TS 36.101 [7]

The measured maximum output power PUMAX,c shall be within the following bounds:

PCMAX_L,c –  MAX{TL,c, T(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  MAX{T(PCMAX_H,c)}

where T(PCMAX) is defined by the tolerance table in Table 6.2B.4-1 and applies to PCMAX_L,c and PCMAX_H,c separately. The tolerance TL,c is the absolute value of the lower tolerance for the applicable operating band as specified in Table 6.2B.1-1.

Table 6.2B.4-1: PCMAX tolerance

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

< End of changes >