3GPP TSG-RAN4 Meeting #116	R4-2510274

# Bengaluru, India, 25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0814 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of index number of TCI states for STxMP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to RAN1 LS R4-2509008, the index number definition for STxMP currently is different from RAN1 and RAN2. In order to keep alignment, RAN1 specificaion is suggested to be changed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the index number for TCI state from {0,1} to {1,2}. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TCI state indexing remain not align with RAN1 and RAN2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2K.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

6.2K.4	Configured transmitted power for simultaneous transmission to multiple directions

A UE configured for simultaneous transmission to multiple directions can configure two maximum output powers. The configured UE maximum output power PCMAX,f,c,k for each of joint/UL TCI states k (k=1,2) indicated for simultaneous transmission to multiple directions of carrier f and serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11].

The configured UE maximum output power PCMAX,f,c,k shall be set such that the corresponding measured peak EIRP PUMAX,f,c,k is within the following bounds

PPowerclass + ΔPIBE – MAX(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP + ΔMBP,n, P-MPRf,c,k) – MAX{T(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP), T(P-MPRf,c,k)} – ΔTSTxMP  ≤ PUMAX,f,c,k ≤ EIRPmax

and PUMAX,f,c, the corresponding measured peak EIRP for carrier f of a serving cell c, aggregated over all indicated joint/UL TCI states in a given direction, satisfies over all directions

PUMAX,f,c ≤ EIRPmax

while the corresponding measured total radiated power PTMAX,f,c is bounded by

PTMAX,f,c ≤ TRPmax

< End of changes >