3GPP TSG-RAN WG4 Meeting # 116	R4-2510276

Bengaluru, India, August 25th – 29th, 2025

Source: 	vivo

Title: 	(NR_newRAT-Core, LTE-RF) Discussion on addition of higher harmonic exceptions for UE co-existence requirements

Agenda Item:	4.2.2

Document for:	Approval

Introduction

There is a note for harmonic exception for UE co-existence for LTE/NR. However, it has not been updated with introduction even higher frequency band for a long time.

In this paper, the history was traced, and some observations and proposals were made for this problem.

Discussion

Current status

Currently in UE co-existence requirements, there is the following note which is aligned for LTE and NR which is for the exception of harmonics:

Observation 1: There is an exception for harmonics in UE co-existence requirements.

Background & Analysis

Traditionally, all the possible harmonic cases were treated as exception.

In Rel-8/9 36.101, only 2nd and 3rd harmonics were considered, which is as following:

When band 22(3.5GHz) were introduced in [1], there begins to have overlapping of 4th harmonics for band 8, and the note2 was updated as following and captured into 36.101 V10.4.0:

Observation 2: In early days, the exception for harmonics were updated when new higher frequency band was introduced.

After that, the first 1 MHz frequency range immediately outside the harmonic emission was also studied and incorporated into the note2 as exception in [2]. In the meantime, the harmonics were extended to [5th] harmonics. The revision was agreed in CR [3] for 36.101 v10.17.0.

In later stage, more bands in higher range were introduced, such as band n79. The possible 6th harmonics for some lower bands (e.g. band 8/28 etc) could be overlapping with n79, and may also apply an exception. However, this note was never updated.

When NR started, this note was directly reused from LTE for NR without even remove the square brackets of [5th]. E.g. for certain band e.g. band 28/n28, the 6th harmonics might fall into n79, and Note 2 is also needed for the UE protection.

Observation 3: The update of the harmonics note did not continual in LTE and NR even new higher order of harmonics emerge with the introduction of higher frequency band.

Although this may not seems a serious problem, since the considered harmonics is already larger than 5 and spec exist for a long time, it actually may bring more difficulty and unnecessary complexity for UE implementation. In addition, with the introduction of more bands, the possibility of such overlapping might increase.

Observation 4: Neglecting the problem would bring unnecessary complexity for UE implementation.

Proposal

With this, it is suggested to further update the note, at least cover 6th order of harmonics which would have some impact on n79. A set of draft CRs were submitted such as [4][5].

In addition, certain band also need to be updated for the 6th harmonic exception such as band 28/n28, which is also included in the CRs.

Proposal: Adding 6th harmonics in the Note 2 of UE co-existence requirements, and change the applicability of certain band to cover this case.

Conclusion

In this paper, the note for harmonic exception for UE co-existence for LTE/NR were discussed. The following observations and proposal are listed.

Observation 1: There is an exception for harmonics in UE co-existence requirements.

Observation 2: In early days, the exception for harmonics were updated when new higher frequency band was introduced.

Observation 3: The update of the harmonics note did not continual in LTE and NR even new higher order of harmonics emerge with the introduction of higher frequency band.

Observation 4: Neglecting the problem would bring unnecessary complexity for UE implementation.

Proposal: Adding 6th harmonics in the Note 2 of UE co-existence requirements, and change the applicability of certain band to cover this case.

- References

[1] R4-114762, Introduction of Band 22, TeliaSonera, RAN4#60

[2] R4-147329, Clarification to measurement procedure for harmonic exception, Nokia, RAN4#73

[3] R4-148034, Correction to Note 2 of Harmonic Signal Exceptions in Spurious Emissions, Nokia, RAN4#73

[4] R4-2510277, (LTE-RF) Addition of higher harmonic exceptions for UE co-existence requirements, vivo, RAN4#116

[5] R4-2510279, (NR_newRAT-Core) Addition of higher harmonic exceptions for UE co-existence requirements, vivo, RAN4#116