3GPP TSG-RAN4 Meeting #116	R4-2510288

# Bangalore, India, 25 Aug - 29 Aug, 2025

Agenda item:	 7.10.3.2

Source:	 vivo

Title:	Proposals on concluding NTN OTA test method and performance metric

Document for:	Approval

- Introduction

RAN4 made good progress on NTN positioning guideline and potential performance metric for scenario 2 last meeting [1]. On top of that from test setup perspective, traditional AC methodology for smartphone can be reused as much as possible, only difference would be some UE configurations, e.g., dwell time, Doppler setting.

However, there are still few open issues to be resolved this meeting, e.g., the final performance metric for scenario 2, positioning guideline.

In this contribution, we share some proposals to conclude NTN OTA core part.

- Discussion

Issue 2-1-2: alignment of understanding on UE positioning guideline for Scenario 2

Agreements:

- For Scenario 2 UEs:

  - The DUT shall be mounted in a suitable hand phantom

  - The elevation tilt of the default position of the DUT can be:

    - 0 degree (similar to Figure 6.1-1 of TR 38.870, but with hand phantom) or as shown below

    - [±15, ±30, ±45] degrees; Other tilt angles are not precluded

    - NOTE 1: the granularity of the elevation tilt to be further discussed next meeting

  - The elevation tilt of the hand phantom & DUT is determined based on OEM declaration

For scenario 2 positioning guideline, it was agreed that 0-degree elevation tilt is the default position [1]:

During the discussions last meeting, few companies commented that the scenario 2 usage may cover different angle, the tilt angle selection should be more flexible. In the agreed WF [1], 15, 30 and 45 degrees are candidate options. In our understanding, for scenario 2 usage, most likely, a positioning assistance application is needed to

Issue 2-1-4: Whether the partial sphere is using the =0° (top pole) as a center

Agreements:

the partial sphere metric is using the  =0° (top pole) as a center

Issue 2-1-5: What is the angle range of the partial sphere-after concluding center of the sphere

Agreements:

- RAN4 further discuss the theta angle range of partial sphere metric, from ±30°, ±60°, ±90°.

  - Option 1:  single range is selected and captured in TR

  - Option 2:  multi-range in TR but based on UE declaration, details are FFS

- Note1: the phi range should be 360 degrees

- Note2: if multiple ranges are captured in TR, the metric applicability rule should also be defined. The target is selecting one for NTN scenario 2.

help users for connecting the satellite via a small range of scanning.

The positioning guideline decision is also related to how to specify the potential performance metric: 1) if the performance metric is based on a small range of sphere, then it would be good to select more tilt angles to reflect the usage cases. 2) if the performance metric is more generic, e.g., upper hemisphere, then the tilt angle of UE would not have big impact on final performance, as FR1 antenna is not a directional pattern.

Regarding the option 1 and option 2 in the WF, we prefer to keep multi-range metric in TR to provide a comprehensive partial sphere metric to the industry, because the TR may be adopted by different organizations for OTA testing of many kinds of devices. Meanwhile, for scenario 2 performance metric, to ensure the comparable of different declared positioning, we think a single range as final metric would be reasonable.

Therefore, based on above considerations, half sphere could be a safer metric to quantify the performance. For NTN scenario 2, just select upper hemisphere as the single performance metric. The performance metric should be specifically stated in the test procedure but not based on UE declaration. Meanwhile, the elevation tilt angle of ±15, ±30, ±45 degrees as positioning guideline is sufficient.

Issue 2-2-3: NTN MU

Agreements:

- For NTN OTA, there is no MU update identified. Partial sphere metric related MU may be further discussed, if needed.

Proposal 1: Capture multi-range metric in the TR and use upper Hemisphere as the performance metric for scenario 2 NTN. The positioning guideline includes elevation tilt angle of ±15, ±30, ±45 degrees is based on UE declaration.

Regarding the MU assessment of NTN OTA, considering partial sphere is data processing based on traditional TRP/TRS measurements, the overall MU assessment can cover this case, so no MU update is identified.

Proposal 2: No MU update is identified for NTN OTA scenario 2. The defined MU assessment for AC TRP/TRS can be applied.

A CR for NTN OTA based on endorsed CR [2] and above proposals, in prepared in [3].

- Conclusion

In this contribution, we discuss remaining open issues for NTN OTA:

Proposal 1: Capture multi-range metric in the TR and use upper Hemisphere as the performance metric for scenario 2 NTN. The positioning guideline includes elevation tilt angle of ±15, ±30, ±45 degrees is based on UE declaration.

Proposal 2: No MU update is identified for NTN OTA scenario 2. The defined MU assessment for AC TRP/TRS can be applied.

With this, the NTN OTA test method can be concluded.

# References

- R4-2508782, Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3, vivo, RAN4#115

- R4-2506880, draft CR to 38.870 on NTN OTA performance metric and test method, vivo, RAN4#115

- R4-2510290, draft CR to 38.870 on NTN OTA performance metric and test method, vivo, RAN4#116