3GPP TSG-RAN4 Meeting #116	R4-2510289

# Bangalore, India, 25 Aug - 29 Aug, 2025

Agenda item:	 7.10.4.1

Source:	 vivo

Title:	Discussions on Rel-19 OTA performance part

Document for:	Approval

- Introduction

Last meeting, RAN4 made good progress on 2Tx lab alignment activity, size 2 (width ≤ 72mm) UE requirements, working procedure for 1Tx performance measurement campaign [1].

To move forward, this contribution share the updated status of 2Tx lab alignment activity, updated working procedure, and latest status of Rel-19 OTA measurement campaign.

Some actions for next steps are also discussed.

- Discussion

2Tx lab alignment activity

The phase-1 lab alignment outcome is summarized in [2], all three test labs are well aligned. In this meeting, two more test labs finalize the measurements [3-4], all the results are considered and summarized in [5], also copied here:

Same as Rel-17 and Rel-18 lab alignment, the Rel-19 2Tx lab alignment outcome will be captured in TR 38.870 for information.

Proposal 1: Conclude 2Tx lab alignment activity if no further results received, the outcome will be captured in TR 38.870.

Status of Rel-19 OTA measurement campaign

Rel-19 OTA measurement campaign is well organized based on offline coordination with all the volunteer test labs during the summer time, current test plan from each test lab and DUT providing information from operators are summarized in the table below:

Based on the agreed timeline, it is expected to collect all the measurement results before the end of RAN4#117 Nov meeting.

Some of the feedback from measurement campaign activity:

- To ensure collection of sufficient number of devices, the Year-of-production should be changed back to original timeline, i.e., 2022 half to 2025.

- How to improve the connection issue of some devices provided by operators, support from device provider is needed. The gNB emulator model information is provided by some operators, it is encouraged that test labs use the same call box to minimize the connection issue.

Working procedure update

The working procedure for measurement campaign is updated based on latest status and feedback from test labs, as following:

Working procedure for TRP TRS Performance Test Campaign to define requirements

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: n3, n5, n7, and n8 [2Tx at bands n41;]

  - Use scenarios:

    - For 1Tx, consider both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR;

    - For 2Tx, only consider Browsing mode (Hand phantom only) requirements in Rel-19;

  - Hand Phantom: Wide Grip Hand and PDA hand

  - Operation mode: SA with 1Tx as phase 1. SA with 2Tx (non-coherent UL-MIMO) as phase 2 after 2Tx lab alignment activity.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: UE (width >56mm and ≤92mm)

  - DUT capability: support for Bands those listed in the latest WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - The following selection criteria can also be considered:

- Year of production: [2022 half-2025]

- Brand variety

- Price range (to capture different price segment, including High/Mid/Low-end products)

- Popularity

- Number of bands supported

  - Power Class: PC2 and PC3. PC3 for all 1Tx bands. PC2 for 2Tx band n41.

- Devices provisioning:

  - Any 3GPP member can work with test labs to provide devices to the aligned test labs (each test lab shall provide a single measurement result set for repeated UE model into RAN4 measurement campaign data pool)

    - FFS for the same UE model with supporting different set of bands

  - Logistical aspects for devices provisioning to the labs are TBD

- Test results submitting:

  - UE information disclosure: the following rules should be considered

    - UE model information based on template [Rel-18 R4-2408104 as starting point, update is needed] can be shared to MCC. RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure.

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

    - laboratories use the same template to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

      - Information of the devices that are going to be measured shall be shared with the RAN4 Secretary as soon as available (i.e., before the measurement activity on such devices starts)

      - The RAN4 Secretary updates the summary of statistical information  and publishes it to 3GPP RAN4 (i.e., living document) in order to monitor the achievement of the thresholds in a timely manner and take the proper actions if these are not met

      - Separate information sheets are used for 1Tx and 2Tx measurement campaigns

    - Statistics of UE information can be shared to RAN4 reflector after TBD meeting by RAN4 secretary after post-processing

    - Consider following aspects as thresholds related to the devices pool for Rel-19

      - Minimum number of devices for each band, each device size, each power class: [45~50] (based on the assumption that all test labs can submit results on time according to current test plan)

      - Minimum number of device models: same as above (based on experience last release, there is no repeated model in data pool)

      - Minimum number of devices' vendors: TBD

      - Percentage of devices from second-half 2022 to 2025: all

      - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: all

  - Laboratories use the same worksheet template in [Rel-18 R4-2315844 as starting point, update is needed] to submit the measurement results for Rel-19 3GPP TRP TRS performance data pool.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - The allowed maximum number of submitted devices from each lab is 15 (depends on how many test labs will join the activity) with a minimum of [3] devices.

  - Only the results from aligned labs will be considered for specifying requirements

  - The progress in each lab are encouraged to share on the RAN4 reflector (for example - how many devices have been measured and on which bands)

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

- Specify TRP TRS requirements:

  - Minimum number of devices for defining requirements for each band, each device size, each power class (requirement will not be specified if measurement results is less than): [30]

  - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-19.

  - Method of limits derivation: per-band Data driven approach, working procedure in Rel-18 on how to specify the requirement is baseline.

  - The value at [80%] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion

  - only PC3 requirements of selected bands for 1Tx.

  - no differentiation of Size 1 and Size 2 TRP/TRS requirements

- Test lab procedures for 1Tx

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - only PC3 testing is needed.

  - For new labs of 1Tx requirements work, those not declared aligned in Rel-17 and Rel-18 also need to complete 1Tx lab alignment on at least n78 (high band) and n28 (low band).

  - Only legacy grid, i.e., 15degree TRP and 30 degree TRS, is used for this measurements campaign

- Test lab procedures for 2Tx

  - Only non-coherent device. FFS details

Proposal 2: Approve above updated working procedure for Rel-19 performance measurement campaign.

- Conclusion

In this contribution, we propose several actions for Rel-19 TRP TRS work:

Proposal 1: Conclude 2Tx lab alignment activity if no further results received, the outcome will be captured in TR 38.870.

Proposal 2: Approve above updated working procedure for Rel-19 performance measurement campaign.

# References

- R4-2508782 Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3, vivo, RAN4#115

- R4-2508761, Analysis of 3GPP TRP TRS AC lab alignment for 2Tx, vivo, RAN4#115

- R4-2509041, Sporton measurement results for Rel-19 TRP TRS 2Tx AC lab alignment activity, Sporton International Inc, RAN4#116

- R4-2509171, Measurement results for Rel-19 TRP TRS 2Tx AC lab alignment activity, Element Materials Technology, RAN4#116

- R4-2510287, Analysis of 3GPP TRP TRS AC lab alignment for 2Tx-PHASE 2, vivo, RAN4#116

- TR 38.870 v19.1.0